| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0179-v1 |
| --- | --- |

| Title: | CE3: Results on Multiple 4-tap filter in JVET-J0024 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Narae Choi Min Woo Park Chanyul Kim Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | narae1.choi@samsung.com m.w.park@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

- Abstract

This contribution presents an intra filtering method proposed in JVET-J0024. The proposed method uses multiple four-tap linear interpolation filters which have different degrees of smoothness to generate a prediction block. Multi-type filters are alternatively applied depending on the position of the prediction pixels. Simulation results reportedly show average -0.41% luma BD-rate gain with 112% and 103% encoding and decoding time for all intra (AI), -0.21% luma BD-rate gain with 103% and 100% encoding and decoding time for random access (RA) based on VTM configuration.

Proposed method

In this contribution, multiple 4-tap interpolation filter for intra prediction proposed in JVET-J0024 [1] is presented. Four-tap linear prediction is composed with interpolation filter and smoothing filter. Smoothing filter makes a prediction block natural and transform friendly but removes detail information especially near the reference pixels. In this proposal, multiple filters which have different degrees of smoothness are used in the same block to improve both accuracy and naturalness of intra prediction block.

The proposed filters are alternatively applied depending on the position of the prediction pixels. The degree of smoothness is increased with the distance from the reference pixel. Following equation and figure 1-1 describes the proposed algorithm where px,y represents the predicted pixel, x and y represent the pixel position, fk denotes filter type, and ai represents the reference pixel. Five types of filter are denoted with (f0 (the weakest), f1, f2, f3, f4 (the strongest)).

px, y  = fk, 0 * a-1 + fk, 1 * a0 +fk, 2 * a1 + fk, 3 * a2 , where fk, i: filter coefficient (0 ≤ k ≤ 5, 0 ≤ i ≤ 3).

Figure 1-1. Multiple four-tap filter description

In the small CU (width or height ≤ 8), one filter is adapted according to intra mode. For example, the filter f1 (second weak) is applied to a block with odd mode index and the filter f2 is applied with even mode index. Otherwise, each filter is adapted according to absolute distance from the reference pixels. The absolute distance is changed with block size. In this proposal, two absolute distance set is used.

- Both width and height ≥ 32: (f0: [0 2), f1: [2 4), f2: [4 8), f3: [8 16), f4: [16 size)).

- Otherwise: (f0: [0 1), f1: [1 2), f2: [2 3), f3: [3 4), f4: [4 size)).

Multiple 4-tap filter is applied to luma block for angular prediction. The interpolation filter accuracy is equal to 1/32 fractional pixels.

Experimental Results

The proposed scheme is implemented into the BMS version 1.0 software. The first anchor is the VTM with ‘vtm’ configuration files and the second anchor is the BMS with ‘bms’ configuration files. Both the ‘All Intra’ and ‘Random Access’ test configurations were used. The simulation was carried out under the JVET common test condition [2]. The software was compiled with GCC version 4.8.3 and the AVX SIMD instructions.

Table 2-1. Results for multiple four-tap filter (Sub-CE 2.10.1)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0(VTM_CFG) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.19% | -0.10% | 113% | 103% | -0.11% | -0.16% | -0.13% | 103% | 101% |
| Class A2 | -0.19% | -0.02% | -0.03% | 112% | 104% | -0.09% | -0.04% | -0.10% | 103% | 103% |
| Class B | -0.38% | -0.10% | -0.09% | 111% | 104% | -0.15% | -0.11% | -0.18% | 103% | 103% |
| Class C | -0.57% | -0.47% | -0.46% | 109% | 100% | -0.17% | -0.19% | -0.09% | 102% | 101% |
| Class E | -0.67% | -0.11% | -0.08% | 114% | 103% | -0.10% | -0.05% | -0.29% | 103% | 103% |
| Overall | -0.41% | -0.19% | -0.16% | 112% | 103% | -0.13% | -0.11% | -0.16% | 103% | 102% |
| Class D | -0.51% | -0.19% | -0.32% | 110% | 103% | -0.23% | -0.19% | -0.27% | 102% | 101% |
| Class F (optional) | -0.08% | -0.07% | -0.14% | 111% | 103% | 0.21% | 0.14% | 0.04% | 103% | 101% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0(VTM_CFG) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.15% | -0.06% | 104% | 100% | -0.03% | 0.08% | -0.01% | 100% | 99% |
| Class A2 | -0.13% | -0.10% | 0.00% | 103% | 99% | -0.02% | -0.05% | 0.00% | 100% | 99% |
| Class B | -0.21% | -0.09% | -0.08% | 103% | 101% | -0.09% | -0.02% | -0.10% | 99% | 99% |
| Class C | -0.32% | -0.23% | -0.29% | 104% | 101% | -0.11% | -0.11% | -0.39% | 99% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.21% | -0.14% | -0.12% | 103% | 100% | -0.07% | -0.03% | -0.14% | 99% | 98% |
| Class D | -0.22% | -0.33% | -0.41% | 103% | 101% | -0.09% | -0.14% | -0.41% | 102% | 102% |
| Class F (optional) | 0.02% | 0.10% | 0.16% | 104% | 100% | 0.16% | 0.19% | 0.43% | 103% | 104% |

Conclusion

In this contribution, the interpolation filters with multiple degrees of smoothness are used to reconstruct a prediction block. Also, some restrictions are proposed to improve the coding performance. Multiple 4-tap interpolation filter method can be an alternative of the current interpolation filter of VTM.

References

[1] “Description of SDR, HDR and 360° video coding technology proposal considering mobile application scenario by Samsung, Huawei, GoPro, and HiSilicon,” JVET-J0024, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, San Diego, US, Apr. 2018.

[2] “JVET common test conditions and software reference configuration,” JVET-J1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, San Diego, US, Apr. 2018.

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).