| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0180-v1 |
| --- | --- |

| Title: | CE3: Results on Multi-combined intra prediction in JVET-J0024 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Narae Choi Min Woo Park Chanyul Kim Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | narae1.choi@samsung.com m.w.park@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

- Abstract

This contribution presents an intra prediction method to produce natural pattern of prediction block by combining original and filtered reference pixels. Filtered reference is calculated with weighted summation of original reference pixels. The direction of reference filtering is determined by intra direction mode. Simulation results reportedly show average -0.12% luma BD-rate gain with 196% and 100% encoding and decoding time for all intra (AI), -0.02% luma BD-rate gain with 113% and 100% encoding and decoding time for random access (RA) based on VTM configuration.

Proposed method

Directional intra prediction often results in non-smooth and unnatural patterns which are not transform-friendly. It is possible to reduce this problem by using a smoothing filter, but conventional smoothing filters tend to also remove meaningful details from the predicted results.

The purpose of multi-combined intra prediction (MIP) proposed in JVET-J0024 [1] is to generate more natural patterns by combining original and filtered reference pixels. The filtered reference pixel is generated with weighted summation of original reference pixels. The filtering direction follows the direction of intra mode. Following equations and Figure 1-1 describe how to generate the filtered references where a represents original reference and a’ represents filtered reference.

Reference filtering = Left → Right, Top → Bottom : a'j=14*(aj-1+3aj)Right → Left, Bottom → Top : a'j=14*(3aj+aj+1)DC, PLANAR, and HOR and VER adjacent mode : a'j=18*(aj-1+6aj+aj+1)

For example, when intra direction is determined from left to right or from top to bottom, the reference is filtered to forward direction (blue arrows). The HOR and VER adjacent modes are HOR_IDX ± 3, VER_IDX ± 3 modes.

Figure 1-1. Reference filtering algorithm

For each prediction unit, MIP processing is applied after the intra predictions.

- If MIP_index = 0, no MIP processing is applied.

- If MIP_index = 1, then a 3 points based filter is applied.

Detail of algorithm is described in the following equations and the figure (Figure 1-2). Ca[i,j] represents a coefficient for reference pixel and Cp[i,j] represents a coefficient for predicted pixel. The i and j are represent the position of the prediction pixel which are matched with low and column, respectively.

p'ij= Cai,jCpi,jCaj,i * a'ipija'j

,  where Cai,j= 121511531, Cpi,j= 29109612101214.

When i or j is bigger than 3, coefficient for i=3 or j=3 is used.

Figure 1-2. MIP description

Both block width and height are smaller than 8, MIP is not applied. Also, the intrinsic reference filtering in VTM is not performed when the MIP_index is 1 and the block width or height is bigger than 16. This method is applied to luma intra prediction.

Experimental Results

The proposed scheme is implemented into the BMS version 1.0 software. The first anchor is the VTM with ‘vtm’ configuration files and the second anchor is the BMS with ‘bms’ configuration files. Both the ‘All Intra’ and ‘Random Access’ test configurations were used. The simulation was carried out under the JVET common test condition [2]. The software was compiled with GCC version 4.8.3 and the AVX SIMD instructions.

Table 2-1. Results for multi-combined intra prediction (Sub-CE 2.11.1)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0(VTM_CFG) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.06% | -0.05% | 197% | 100% | 0.05% | -0.03% | -0.16% | 210% | 100% |
| Class A2 | -0.19% | -0.08% | 0.02% | 196% | 101% | -0.03% | -0.06% | 0.01% | 212% | 101% |
| Class B | -0.12% | -0.02% | -0.03% | 197% | 101% | 0.01% | 0.10% | 0.05% | 212% | 102% |
| Class C | -0.14% | -0.01% | -0.18% | 195% | 99% | -0.04% | 0.06% | 0.05% | 209% | 100% |
| Class E | -0.03% | -0.05% | -0.07% | 196% | 97% | 0.07% | 0.10% | 0.03% | 218% | 100% |
| Overall | -0.12% | -0.04% | -0.06% | 196% | 100% | 0.01% | 0.04% | 0.00% | 212% | 101% |
| Class D | -0.13% | -0.01% | 0.02% | 195% | 101% | 0.03% | 0.19% | 0.06% | 209% | 100% |
| Class F (optional) | -0.07% | 0.04% | 0.08% | 196% | 101% | 0.03% | 0.16% | -0.17% | 221% | 101% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0(VTM_CFG) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | -0.06% | 116% | 98% | 0.05% | 0.12% | -0.08% | 144% | 98% |
| Class A2 | -0.05% | -0.05% | 0.02% | 110% | 99% | 0.02% | -0.05% | 0.01% | 137% | 100% |
| Class B | -0.01% | -0.16% | -0.38% | 113% | 101% | 0.04% | -0.11% | -0.15% | 140% | 100% |
| Class C | -0.03% | -0.06% | -0.12% | 114% | 100% | 0.07% | 0.00% | -0.24% | 143% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.02% | -0.09% | -0.17% | 113% | 100% | 0.05% | -0.02% | -0.13% | 141% | 100% |
| Class D | 0.02% | -0.04% | -0.39% | 113% | 100% | 0.13% | 0.12% | 0.18% | 144% | 101% |
| Class F (optional) | -0.03% | 0.08% | 0.08% | 122% | 100% | 0.02% | 0.03% | 0.12% | 160% | 101% |

Conclusion

In this contribution, the original and filtered references were combined to generate a prediction block with natural pattern. Also, some restrictions are proposed to improve the coding performance.

References

[1] “Description of SDR, HDR and 360° video coding technology proposal considering mobile application scenario by Samsung, Huawei, GoPro, and HiSilicon,” JVET-J0024, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, San Diego, US, Apr. 2018.

[2] “JVET common test conditions and software reference configuration,” JVET-J1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, San Diego, US, Apr. 2018.

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).