| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SL, 10–18 July 2018 | Document: JVET-K0182 |
| --- | --- |

| Title: | CE13: Parallel-to-Axis Uniform cubemap projection (PAU) in JVET-J0033 (Test 7) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yule Sun, Xuchang Huangfu, Bin Wang, Lu Yu Zhejiang University Hangzhou, Zhejiang 310027,China | Tel: Email: | +86 57187953107 sunyule@zju.edu.cn yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

- Abstract

This contribution reports the results of CE13 [1] for Test 7 (Parallel-to-axis uniform cubemap projection (PAU) without padding and PAU with padding). Test results show that PAU without padding yields -11.65% (VTM) and -11.50% (BMS) in BD-rate (using end-to-end Y-WS-PSNR) reduction on average in comparison to PERP in VTM and BMS respectively. For PAU padding case, the coding performance are -11.58% (VTM) and -11.41% (BMS) compared with PERP in VTM and BMS respectively.

Introduction

The proposed PAU format is similar to ACP and EAC. The difference is that the sampling rate adjusting function of PAU in horizontal and vertical directions are different, which can make the sampling distribution on cube face more uniform. For PAU, one direction of cube face is first resampled to achieve approximately equal-angle sampling. Then another orthometric direction is equal-angle sampling based on the resampling position in first direction. In J0033, vertical direction is first determined.

The PAU has the same face definition as CMP and frame packing is shown in 1. Assuming (u’, v’) is defined in a CMP face, and (u, v) is the corresponding point in a PAU face. Each face is defined in the range of [-1, 1]. The equations (1)(2) and (3) are the forward adjustment function from CMP to PAU. Vertical direction is first adjusted by (1). For the horizontal direction, sampling adjustments of u are related to vertical sampling v, which is expressed in (2) and (3).

| v=4.0π*tan-1v' | (1) |
| --- | --- |
| α=tan-1v' | (2) |
| u=tan-1u'*cosαtan-1cosα | (3) |

For the backward adjustment function, the equations (4)(5) and (6) are used to map PAU to CMP.

| v'=tanvπ4.0 | (4) |
| --- | --- |
| β=tan-1v' | (5) |
| u'=1cos⁡(β)*tanu*tan-1cosβ | (6) |

Figure 1. Frame packing of PAU without padding (left) and with padding (right).

Padding: To avoid obvious seam artifact near the discontinuous face boundary, padding is performed on the vertical face boundary, which is shown in the right figure of Figure 6. The gray areas are the padding pixels. In this CE, padding width is 3 per face and the value of pixel are copied from the nearest pixel in vertical direction inside each face. The content of each face is scaled on an area of (face width) * (face height-3). So white area in each face is scaled to [-1, 1] in vertical direction.

Blending: No blending is used.

Simulation results

The tests are conducted according to the JVET common test conditions for 360 video [2] and on both the Test Model (VTM) and the Benchmark Set (BMS) with RA configuration. BD rate results using end-to-end WS-PSNR are listed below. Other metrics’ results can be seen in attached excel. Subjective comparison of PAU Padding and PAU without padding are also presented.

Objective results

Table 1 BD-Rate using end-to-end WS-PSNR (VTM)

| PAU without padding | Anchor: PERP |  |  | Anchor: CMP |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -18.90% | -19.95% | -19.45% | -10.78% | -8.87% | -8.89% |
| ChairLift | -26.14% | -19.54% | -20.06% | -9.01% | -5.86% | -5.81% |
| KiteFlite | -10.42% | -2.18% | -4.57% | -6.55% | -2.49% | -2.50% |
| Harbor | -10.50% | -3.57% | -2.80% | -11.54% | -1.92% | -1.97% |
| Trolley | -8.99% | 2.09% | -3.17% | -7.29% | -1.83% | -2.37% |
| GasLamp | -5.59% | 3.38% | 9.95% | -7.55% | -3.40% | -3.68% |
| Balboa | -12.09% | -8.89% | -10.27% | -7.35% | -8.53% | -8.82% |
| Broadway | -9.57% | -8.15% | -7.36% | -9.52% | -7.44% | -7.69% |
| Landing2 | -3.10% | -3.29% | -6.21% | -7.57% | -8.27% | -9.88% |
| BranCastle2 | -11.23% | -9.10% | -8.64% | -9.66% | -9.45% | -9.50% |
| Overall | -11.65% | -6.92% | -7.26% | -8.68% | -5.80% | -6.11% |

Table 2 BD-Rate using end-to-end WS-PSNR (VTM)

| PAU with padding | Anchor: PERP |  |  | Anchor: CMP |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -19.01% | -20.61% | -20.44% | -10.88% | -9.60% | -9.97% |
| ChairLift | -25.96% | -19.19% | -19.64% | -8.76% | -5.44% | -5.30% |
| KiteFlite | -10.41% | -1.94% | -4.61% | -6.53% | -2.26% | -2.55% |
| Harbor | -10.48% | -3.96% | -4.19% | -11.52% | -2.33% | -3.41% |
| Trolley | -8.79% | 1.85% | -3.29% | -7.07% | -2.07% | -2.50% |
| GasLamp | -5.60% | 2.88% | 9.84% | -7.56% | -3.86% | -3.79% |
| Balboa | -12.02% | -8.74% | -10.01% | -7.26% | -8.38% | -8.56% |
| Broadway | -9.62% | -8.35% | -7.58% | -9.56% | -7.63% | -7.91% |
| Landing2 | -3.01% | -4.53% | -7.20% | -7.46% | -9.39% | -10.77% |
| BranCastle2 | -10.92% | -8.90% | -8.47% | -9.34% | -9.25% | -9.34% |
| Overall | -11.58% | -7.15% | -7.56% | -8.59% | -6.02% | -6.41% |

Table 3 BD-Rate using end-to-end WS-PSNR (BMS)

| PAU without padding | Anchor: PERP |  |  | Anchor: CMP |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -19.86% | -20.59% | -20.95% | -10.94% | -9.41% | -10.16% |
| ChairLift | -22.12% | -12.68% | -16.28% | -8.65% | -6.61% | -4.99% |
| KiteFlite | -11.19% | -3.09% | -7.00% | -7.22% | -3.47% | -2.62% |
| Harbor | -11.65% | -3.61% | -3.44% | -12.41% | -2.71% | -2.10% |
| Trolley | -10.26% | 4.06% | -2.35% | -8.02% | -1.56% | -2.22% |
| GasLamp | -6.89% | 2.36% | 9.04% | -8.19% | -3.83% | -3.90% |
| Balboa | -9.86% | -7.70% | -9.19% | -6.91% | -9.33% | -9.42% |
| Broadway | -8.12% | -7.18% | -5.94% | -9.48% | -8.36% | -7.95% |
| Landing2 | -3.01% | -3.02% | -6.40% | -6.62% | -8.21% | -9.92% |
| BranCastle2 | -12.04% | -9.91% | -8.73% | -8.41% | -8.93% | -8.26% |
| Overall | -11.50% | -6.14% | -7.12% | -8.69% | -6.24% | -6.15% |

Table 4 BD-Rate using end-to-end WS-PSNR (BMS)

| PAU with padding | Anchor: PERP |  |  | Anchor: CMP |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -19.95% | -21.42% | -21.79% | -11.02% | -10.33% | -11.06% |
| ChairLift | -21.87% | -12.76% | -15.86% | -8.33% | -6.70% | -4.51% |
| KiteFlite | -11.19% | -3.64% | -7.59% | -7.20% | -4.02% | -3.26% |
| Harbor | -11.63% | -4.91% | -5.10% | -12.38% | -4.06% | -3.79% |
| Trolley | -10.12% | 2.91% | -2.57% | -7.86% | -2.69% | -2.47% |
| GasLamp | -6.85% | 1.42% | 8.19% | -8.14% | -4.71% | -4.64% |
| Balboa | -9.74% | -8.41% | -9.11% | -6.77% | -10.02% | -9.36% |
| Broadway | -8.14% | -7.79% | -5.95% | -9.50% | -8.98% | -7.96% |
| Landing2 | -2.88% | -4.06% | -7.19% | -6.48% | -9.15% | -10.65% |
| BranCastle2 | -11.71% | -9.77% | -8.51% | -8.06% | -8.79% | -8.06% |
| Overall | -11.41% | -6.84% | -7.55% | -8.58% | -6.94% | -6.58% |

Subjective results

Evil viewports are generated to assess the visibility of potential seam artifacts. The evil viewports for padding and no padding are presented. Evil viewports are generated from sequences coding with VTM at the lowest rate point (QP=37) according to CTC [2].

Figure 2 and Figure 3 are the evil viewport of static viewport 1 centered at vertex 1 for no padding and padding case. The seam artifacts are similar because this vertex is not near padding area in the vertical face boundary. Figure 4 and Figure 5 are the evil viewport of Static viewport 1 centered at Vertex 2 for no padding and padding case. This viewport includes padding area and shows padding partly solve the seam artifacts near face boundaries. However, the artifacts near the boundary are still not completely solved by padding.

Table 5. Vertex positions for “evil viewports”

| Format | Vertex 1 (yaw, pitch) (in degrees) | Vertex 2 (yaw, pitch) (in degrees) |
| --- | --- | --- |
| ERP | −180, 0 | 0, 0 |
| CMP | −135, 35 | 45, −35 |
| ACP | −135, 35 | 45, −35 |
| EAC | −135, 35 | 45, −35 |
| RSP | −135, 0 | 0, −45 |
| PAU | −135, 35 | 45, −35 |

Figure 2. Evil viewport of Static viewport 1 centered at Vertex 1 (PAU without padding, Trolley, Frame=143)

Figure 3. Evil viewport of Static viewport 1 centered at Vertex 1 (PAU with padding, Trolley, Frame=143)

Figure 4. Evil viewport of Static viewport 1 centered at Vertex 2 (PAU without padding, Trolley, Frame=132)

Figure 5. Evil viewport of Static viewport 1 centered at Vertex 2 (PAU with padding, Trolley, Frame=132)

Conclusion

The test results of CE13 for Test 7 (Parallel-to-axis uniform cubemap projection (PAU) without padding and PAU with padding) are presented in this contribution. PAU without padding yields -11.65% (VTM) and -11.50% (BMS) in BD-rate (using end-to-end Y-WS-PSNR) reduction on average in comparison to PERP in VTM and BMS respectively. For PAU padding case, the coding performance are -11.58% (VTM) and -11.41% (BMS) compared with PERP in VTM and BMS respectively. Compared with no padding PAU, padding PAU can partly solve the seam artifacts near face boundaries with some coding loss. However, the seam artifacts near face boundaries still need to be further processed to improve subjective results.

References

[1] P. Hanhart, J.-L. Lin, “Description of Core Experiment 13 (CE13): Projection formats,” JVET-J1033, April 2018.

[2] P. Hanhart, J. Boyce, K. Choi “JVET common test conditions and evaluation procedures for 360° video,” JVET-J1012, Apr 2018

Patent rights declaration(s)

Zhejiang university may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).