| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0184-v1 |
| --- | --- |

| Title: | CE4: Affine motion compensation with fixed sub-block size (Test 1.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang Jianle Chen | Tel: Email: | +8615017554651 chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of affine motion compensation with fixed the sub-block size 4x4 (test a) and 8x8 (test b), compared with the adaptive sub-block size for affine in BMS1.0. For test a, negligible coding gain is achieved with around 1% decoding time increase. For test b, simulation results reportedly show on average 0.60%/0.48% luma BD-rate drop with 5%/7% encoding time and 6%/4% decoding time reduction in RA/LB configurations when compared to VTM with affine on, and 0.54%/0.36% luma BD-rate drop with 0%/2% encoding time and 1%/3% decoding time reduction in RA/LB configurations when compared to BMS1.0.

Introduction

In BMS1.0, in order to simplify the motion compensation prediction, sub-block based affine transform prediction is applied. The sub-block size M×N is derived as in Equation 1, where MvPre is the motion vector fraction accuracy (1/16), (v2x, v2y) is motion vector of the bottom-left control point, calculated according to 4-parameter motion model. After derived by Equation 1, M and N should be adjusted downward if necessary to make it a divisor of w and h, respectively [1]. The minimum sub-block size of affine block is 4x4 in BMS1.0.

M=clip3(4,w,w×MvPremaxabsv1x-v0x,absv1y-v0y)N=clip3(4,h,h×MvPremaxabsv2x-v0x,absv2y-v0y)	            (1)

In this contribution, the sub-block size is fixed to 4x4 in test 1 and 8x8 in test 2. While in test 2, the condition check for affine merge mode is modified from “W*H >= 64” to “W >= 8 && H >=8”, to make sure all affine block can be split into 8x8 sub-blocks during MC process.

Simulation results

The contribution has been implemented on top of BMS1.0 [2]. The test condition descripted in JVET-J1024 [3] is used for verifying the contribution. The results are summarized in Table 1 and Table 2. More detailed results are provided from Table 3 to Table 6.

For test a, negligible coding gain is achieved with around 1% decoding time increase.

For test b, the results reportedly show on average 0.60%/0.48% luma BD-rate drop with 5%/7% encoding time and 6%/4% decoding time reduction in RA/LB configurations when compared to VTM with affine on, and 0.54%/0.36% luma BD-rate drop with 0%/2% encoding time and 1%/3% decoding time reduction in RA/LB configurations when compared to BMS1.0.

Table 1 Summary of overall performance in RA/LB configuration (over VTM and BMS – AFFINE)

| Test a | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -3.00% | -2.22% | -2.21% | 138% | 113% | -1.92% | -1.36% | -1.40% | 109% | 103% |
| LB | -2.09% | -1.22% | -1.49% | 168% | 109% | -2.06% | -1.59% | -1.49% | 125% | 105% |
| Test b | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -2.43% | -1.73% | -1.71% | 130% | 106% | -1.40% | -1.01% | -0.99% | 109% | 100% |
| LB | -1.60% | -0.99% | -1.03% | 156% | 103% | -1.64% | -1.10% | -1.33% | 122% | 101% |

Table 2 Summary of overall performance in RA/LB configuration (over VTM + AFFINE and BMS)

| Test a | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -0.01% | -0.03% | 0.01% | 101% | 101% | 0.00% | -0.02% | -0.05% | 101% | 101% |
| LB | -0.02% | 0.11% | 0.04% | 100% | 101% | -0.08% | -0.06% | 0.20% | 100% | 101% |
| Test b | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | 0.60% | 0.48% | 0.53% | 95% | 94% | 0.54% | 0.34% | 0.37% | 100% | 99% |
| LB | 0.48% | 0.35% | 0.51% | 93% | 96% | 0.36% | 0.45% | 0.38% | 98% | 97% |

Test a

Table 3 Coding performance of test a (over VTM and BMS – AFFINE)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.97% | -1.63% | -1.62% | 135% | 107% | -1.14% | -0.82% | -0.93% | 109% | 102% |
| Class A2 | -5.88% | -4.76% | -4.43% | 139% | 120% | -3.43% | -2.51% | -2.44% | 110% | 105% |
| Class B | -3.27% | -2.20% | -2.34% | 140% | 113% | -2.17% | -1.53% | -1.54% | 110% | 102% |
| Class C | -1.27% | -0.77% | -0.82% | 137% | 111% | -1.06% | -0.70% | -0.81% | 109% | 102% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.00% | -2.22% | -2.21% | 138% | 113% | -1.92% | -1.36% | -1.40% | 109% | 103% |
| Class D | -1.81% | -0.97% | -1.15% | 137% | 108% | -1.59% | -1.26% | -1.05% | 108% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.50% | -1.73% | -1.83% | 165% | 110% | -2.55% | -1.61% | -1.76% | 121% | 105% |
| Class C | -1.48% | -0.55% | -0.56% | 148% | 107% | -1.42% | -1.31% | -1.06% | 117% | 105% |
| Class E | -2.22% | -1.26% | -2.16% | 205% | 109% | -2.08% | -1.92% | -1.63% | 143% | 105% |
| Overall | -2.09% | -1.22% | -1.49% | 168% | 109% | -2.06% | -1.59% | -1.49% | 125% | 105% |
| Class D | -2.95% | -2.02% | -1.97% | 141% | 109% | -2.96% | -2.58% | -2.55% | 117% | 104% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 4 Coding performance of test a (over VTM + AFFINE and BMS)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | 0.03% | 101% | 101% | 0.00% | -0.02% | -0.04% | 101% | 101% |
| Class A2 | -0.01% | -0.02% | 0.03% | 101% | 101% | -0.01% | -0.02% | -0.01% | 101% | 101% |
| Class B | -0.01% | -0.04% | 0.00% | 101% | 101% | -0.01% | -0.05% | -0.03% | 101% | 101% |
| Class C | -0.01% | -0.01% | -0.01% | 100% | 101% | 0.01% | 0.01% | -0.10% | 100% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.03% | 0.01% | 101% | 101% | 0.00% | -0.02% | -0.05% | 101% | 101% |
| Class D | -0.02% | 0.04% | 0.06% | 100% | 98% | -0.01% | 0.02% | 0.03% | 100% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | -0.25% | -0.05% | 100% | 102% | -0.02% | -0.14% | 0.30% | 100% | 101% |
| Class C | -0.04% | 0.21% | 0.04% | 100% | 100% | -0.02% | -0.13% | -0.04% | 100% | 101% |
| Class E | -0.04% | 0.59% | 0.18% | 101% | 102% | -0.23% | 0.14% | 0.37% | 101% | 101% |
| Overall | -0.02% | 0.11% | 0.04% | 100% | 101% | -0.08% | -0.06% | 0.20% | 100% | 101% |
| Class D | -0.01% | -0.03% | 0.37% | 100% | 99% | -0.01% | 0.22% | 0.13% | 100% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Test b

Table 5 Coding performance of test b (over VTM and BMS – AFFINE)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.71% | -1.47% | -1.48% | 128% | 103% | -0.95% | -0.86% | -0.80% | 109% | 101% |
| Class A2 | -4.93% | -3.97% | -3.75% | 131% | 106% | -2.56% | -1.87% | -1.86% | 109% | 101% |
| Class B | -2.59% | -1.65% | -1.71% | 131% | 106% | -1.55% | -1.03% | -1.08% | 109% | 100% |
| Class C | -0.88% | -0.36% | -0.35% | 130% | 107% | -0.69% | -0.44% | -0.39% | 108% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.43% | -1.73% | -1.71% | 130% | 106% | -1.40% | -1.01% | -0.99% | 109% | 100% |
| Class D | -1.08% | -0.33% | -0.44% | 130% | 107% | -0.83% | -0.64% | -0.63% | 107% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.96% | -1.25% | -1.24% | 154% | 103% | -2.05% | -1.18% | -1.31% | 119% | 101% |
| Class C | -1.15% | -0.34% | -0.33% | 140% | 103% | -1.10% | -0.84% | -0.87% | 115% | 101% |
| Class E | -1.61% | -1.44% | -1.62% | 187% | 104% | -1.68% | -1.30% | -1.96% | 137% | 102% |
| Overall | -1.60% | -0.99% | -1.03% | 156% | 103% | -1.64% | -1.10% | -1.33% | 122% | 101% |
| Class D | -2.29% | -1.84% | -1.20% | 133% | 104% | -2.25% | -2.07% | -2.55% | 114% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 6 Coding performance of test b (over VTM + AFFINE and BMS)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.27% | 0.12% | 0.18% | 95% | 96% | 0.19% | -0.05% | 0.09% | 101% | 99% |
| Class A2 | 1.00% | 0.82% | 0.75% | 95% | 89% | 0.90% | 0.63% | 0.59% | 100% | 98% |
| Class B | 0.72% | 0.54% | 0.66% | 94% | 94% | 0.64% | 0.46% | 0.45% | 101% | 98% |
| Class C | 0.39% | 0.41% | 0.46% | 95% | 97% | 0.39% | 0.27% | 0.33% | 99% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.60% | 0.48% | 0.53% | 95% | 94% | 0.54% | 0.34% | 0.37% | 100% | 99% |
| Class D | 0.74% | 0.71% | 0.79% | 95% | 97% | 0.78% | 0.66% | 0.46% | 99% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.56% | 0.25% | 0.56% | 94% | 95% | 0.51% | 0.32% | 0.77% | 98% | 98% |
| Class C | 0.30% | 0.43% | 0.28% | 94% | 97% | 0.31% | 0.35% | 0.15% | 99% | 97% |
| Class E | 0.58% | 0.41% | 0.73% | 92% | 96% | 0.18% | 0.79% | 0.05% | 96% | 98% |
| Overall | 0.48% | 0.35% | 0.51% | 93% | 96% | 0.36% | 0.45% | 0.38% | 98% | 97% |
| Class D | 0.69% | 0.14% | 1.15% | 94% | 95% | 0.76% | 0.74% | 0.14% | 98% | 96% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusions

This contribution reports the results of affine motion compensation fixed the sub-block size 4x4 (test a) and 8x8 (test b). For test a, negligible coding gain is achieved with around 1% decoding time increase. For test b, the results reportedly show on average 0.60%/0.48% luma BD-rate drop with 5%/7% encoding time and 6%/4% decoding time reduction in RA/LB configurations, compared to VTM with affine on, and 0.54%/0.36% luma BD-rate drop with 0%/2% encoding time and 1%/3% decoding time reduction in RA/LB configurations, compared to BMS1.0.

References

- J. Chen, E. Alshina, G. J. Sullivan, J-R. Ohm, “Algorithm Description of Joint Exploration Test Model 7 (JEM 7)”, JVET-G1001, July 2017.

- BMS reference software, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- H. Yang, S. Liu, K. Zhang, “Description of CE4: Inter prediction and motion vector coding”, JVET-J1024, April 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).