| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0186-v2 |
| --- | --- |

| Title: | CE4: Affine merge enhancement (Test 2.10) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang Jianle Chen | Tel: Email: | +8615017554651 chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of integrating affine merge enhancement in JVET-J0024 to BMS1.0. The original affine merge mode in BMS is enhanced by constructing an affine merge candidate list with 5 candidates. The affine merge candidate list is constructed by inserting 2 kinds of candidates: model based candidates and control point based candidates.

Simulation results reportedly show on average 1.13%/0.62% luma BD-rate gain with 13%/16% encoding time and 4%/5% decoding time increase in RA/LB configurations, compared to VTM with affine on, and 0.86%/0.54% luma BD-rate drop with 11%/17% encoding time and 1%/1% decoding time increase in RA/LB configurations, compared to BMS1.0.

Introduction

The affine merge candidate list is constructed as following steps:

- Insert model based affine candidates

Figure 1 model based candidates for affine merge mode

Model based affine candidate means that the candidate is derived from the valid neighbor reconstructed block coded with affine mode. As shown in Figure 1, the scan order for the candidate block is from left, above, above right, left bottom to above left. The same derived method in BMS is used [2].

- Insert control point based affine candidates

Control points based candidate means the candidate is constructed by combining the neighbor motion information of each control point.

The motion information for the control points is derived firstly from the specified spatial neighbors and temporal neighbor shown in Figure 2. CPk (k=1, 2, 3, 4) represents the k-th control point. A, B, C, D, E, F and G are spatial positions for predicting CPk (k=1, 2, 3); H is temporal position for predicting CP4.

The coordinates of CP1, CP2, CP3 and CP4 is (0, 0), (W, 0), (H, 0) and (W, H), respectively, where W and H are the width and height of current block.

The motion information of each control point is obtained according to the following priority order:

- For CP1, the checking priority is ABC, A is used if it is available. Otherwise, if B is available, B is used. If both A and B are unavailable, C is used. If all the three candidates are unavailable, the motion information of CP1 cannot be obtained.

- For CP2, the checking priority is ED;

- For CP3, the checking priority is GF;

- For CP4, H is used.

Secondly, the combinations of controls points are used to construct the motion model.

Motion vectors of two control points are needed to compute the transform parameters in 4-parameter affine model. The two control points can be selected from one of the following six combinations ({CP1, CP4}, {CP2, CP3}, {CP1, CP2}, {CP2, CP4}, {CP1, CP3}, {CP3, CP4}). For example, use the CP1 and CP2 control points to construct 4-parameter affine motion model, denoted as Affine (CP1, CP2).

Motion vectors of three control points are needed to compute the transform parameters in 6-parameter affine model. The three control points can be selected from one of the following four combinations ({CP1, CP2, CP4}, {CP1, CP2, CP3}, {CP2, CP3, CP4}, {CP1, CP3, CP4}). For example, use CP1, CP2 and CP3 control points to construct 6-parameter affine motion model, denoted as Affine (CP1, CP2, CP3).

Figure 2 control point based candidates for affine merge mode

All of these models will be converted to 6-parameter affine model represented by top-left, top-right, and bottom-left control point. During MC and motion vector derivation for sub-block, unified 6-parameter affine model is used.

Test specification

The tests for this contribution are descripted in Table 1.

Table 1 test specification for this contribution

| Test # | Test description |
| --- | --- |
| Test a | Model based affine candidates (2 candidates) |
| Test b | Additional control point based affine candidates (5 candidates) |

Simulation results

The contribution has been implemented on top of BMS1.0 [3]. The test condition descripted in JVET-J1024 [4] is used for verifying the contribution. The results are summarized in Table 2 and Table 3. More detailed results are provided from Table 4 to Table 7.

For test a, 0.11%/0.05 luma BD-rate gain is achieved in RA/LB configurations when compared to VTM with affine on, and 0.06%/0.07% luma BD-rate gain is achieved when compared to BMS1.0.

For test b, results reportedly show on average 1.13%/0.62% luma BD-rate gain with 13%/16% encoding time and 4%/5% decoding time increase in RA/LB configurations when compared to VTM with affine on, and 0.86%/0.54% luma BD-rate drop with 11%/17% encoding time and 1%/1% decoding time increase in RA/LB configurations when compared to BMS1.0.

Table 2 Summary of overall performance in RA/LB configuration (over VTM and BMS – AFFINE)

| Test a | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -3.09% | -2.27% | -2.28% | 137% | 110% | -1.97% | -1.39% | -1.42% | 110% | 101% |
| LB | -2.11% | -1.37% | -1.39% | 168% | 107% | -2.05% | -1.73% | -1.73% | 125% | 102% |
| Test b | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -1.13% | -1.07% | -1.12% | 113% | 104% | -0.86% | -0.69% | -0.79% | 111% | 101% |
| LB | -0.62% | -0.67% | -0.45% | 116% | 105% | -0.54% | -0.34% | -0.57% | 117% | 101% |

Table 3 Summary of overall performance in RA/LB configuration (over VTM + AFFINE and BMS)

| Test a | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -0.11% | -0.09% | -0.06% | 100% | 98% | -0.06% | -0.05% | -0.07% | 101% | 99% |
| LB | -0.05% | -0.04% | 0.13% | 100% | 99% | -0.07% | -0.21% | -0.03% | 101% | 99% |
| Test b | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -1.13% | -1.07% | -1.12% | 113% | 104% | -0.86% | -0.69% | -0.79% | 111% | 101% |
| LB | -0.62% | -0.67% | -0.45% | 116% | 105% | -0.54% | -0.34% | -0.57% | 117% | 101% |

Test a

Table 4 Coding performance of test a (over VTM and BMS – AFFINE)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.05% | -1.64% | -1.73% | 134% | 106% | -1.16% | -0.90% | -0.90% | 109% | 100% |
| Class A2 | -6.01% | -4.92% | -4.64% | 138% | 118% | -3.52% | -2.56% | -2.52% | 110% | 103% |
| Class B | -3.38% | -2.32% | -2.41% | 138% | 110% | -2.25% | -1.50% | -1.57% | 110% | 100% |
| Class C | -1.33% | -0.69% | -0.75% | 136% | 107% | -1.09% | -0.75% | -0.80% | 109% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.09% | -2.27% | -2.28% | 137% | 110% | -1.97% | -1.39% | -1.42% | 110% | 101% |
| Class D | -1.82% | -1.14% | -1.30% | 137% | 106% | -1.59% | -1.25% | -1.07% | 108% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM |  |  |  |  | Over BMS - AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.56% | -1.79% | -1.61% | 165% | 109% | -2.59% | -1.83% | -1.83% | 122% | 103% |
| Class C | -1.44% | -0.76% | -0.70% | 148% | 105% | -1.41% | -1.35% | -1.28% | 118% | 103% |
| Class E | -2.25% | -1.48% | -1.97% | 204% | 105% | -2.02% | -2.08% | -2.16% | 143% | 100% |
| Overall | -2.11% | -1.37% | -1.39% | 168% | 107% | -2.05% | -1.73% | -1.73% | 125% | 102% |
| Class D | -3.01% | -2.24% | -2.24% | 141% | 108% | -2.99% | -3.09% | -2.71% | 117% | 104% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 5 Coding performance of test a (over VTM + AFFINE and BMS)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.06% | -0.08% | 100% | 99% | -0.02% | -0.10% | -0.02% | 101% | 99% |
| Class A2 | -0.15% | -0.19% | -0.19% | 100% | 99% | -0.10% | -0.07% | -0.09% | 101% | 100% |
| Class B | -0.13% | -0.16% | -0.08% | 99% | 98% | -0.10% | -0.02% | -0.06% | 101% | 99% |
| Class C | -0.06% | 0.07% | 0.05% | 100% | 96% | -0.01% | -0.04% | -0.09% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.11% | -0.09% | -0.06% | 100% | 98% | -0.06% | -0.05% | -0.07% | 101% | 99% |
| Class D | -0.03% | -0.12% | -0.09% | 100% | 95% | -0.01% | 0.03% | 0.01% | 100% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.07% | -0.32% | 0.17% | 101% | 101% | -0.06% | -0.36% | 0.22% | 101% | 99% |
| Class C | 0.00% | 0.00% | -0.10% | 100% | 99% | -0.02% | -0.17% | -0.26% | 101% | 99% |
| Class E | -0.07% | 0.36% | 0.37% | 100% | 97% | -0.17% | -0.01% | -0.16% | 100% | 96% |
| Overall | -0.05% | -0.04% | 0.13% | 100% | 99% | -0.07% | -0.21% | -0.03% | 101% | 99% |
| Class D | -0.08% | -0.26% | 0.06% | 100% | 99% | -0.04% | -0.35% | -0.01% | 100% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Test b

Table 6 Coding performance of test b (over VTM and BMS – AFFINE)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.26% | -2.75% | -3.05% | 147% | 115% | -1.99% | -1.41% | -1.59% | 117% | 103% |
| Class A2 | -7.54% | -6.41% | -6.23% | 155% | 128% | -4.82% | -3.73% | -3.85% | 121% | 105% |
| Class B | -4.24% | -3.05% | -3.14% | 157% | 116% | -2.97% | -2.07% | -2.17% | 120% | 102% |
| Class C | -1.90% | -1.39% | -1.46% | 157% | 111% | -1.49% | -1.14% | -1.19% | 121% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -4.08% | -3.22% | -3.29% | 155% | 117% | -2.75% | -2.02% | -2.13% | 120% | 103% |
| Class D | -2.46% | -1.62% | -1.89% | 160% | 113% | -2.01% | -1.66% | -1.45% | 120% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM |  |  |  |  | Over BMS - AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -3.09% | -2.28% | -1.93% | 191% | 114% | -3.04% | -2.20% | -2.38% | 142% | 106% |
| Class C | -1.89% | -1.11% | -1.09% | 174% | 111% | -1.83% | -1.46% | -1.61% | 137% | 106% |
| Class E | -3.02% | -2.67% | -3.17% | 229% | 112% | -2.52% | -1.84% | -2.89% | 166% | 104% |
| Overall | -2.67% | -1.99% | -1.96% | 194% | 113% | -2.51% | -1.86% | -2.25% | 146% | 105% |
| Class D | -3.42% | -2.49% | -2.70% | 164% | 115% | -3.42% | -3.04% | -3.56% | 136% | 108% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 7 Coding performance of test b (over VTM + AFFINE and BMS)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.32% | -1.20% | -1.43% | 109% | 108% | -0.87% | -0.61% | -0.71% | 108% | 102% |
| Class A2 | -1.79% | -1.77% | -1.88% | 112% | 107% | -1.45% | -1.28% | -1.46% | 111% | 102% |
| Class B | -1.03% | -0.92% | -0.84% | 113% | 104% | -0.84% | -0.60% | -0.67% | 111% | 101% |
| Class C | -0.64% | -0.63% | -0.66% | 115% | 100% | -0.42% | -0.44% | -0.49% | 112% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.13% | -1.07% | -1.12% | 113% | 104% | -0.86% | -0.69% | -0.79% | 111% | 101% |
| Class D | -0.69% | -0.60% | -0.68% | 116% | 102% | -0.45% | -0.38% | -0.37% | 111% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.62% | -0.82% | -0.16% | 116% | 105% | -0.53% | -0.74% | -0.34% | 117% | 102% |
| Class C | -0.45% | -0.35% | -0.50% | 117% | 104% | -0.44% | -0.28% | -0.60% | 118% | 102% |
| Class E | -0.86% | -0.85% | -0.85% | 112% | 104% | -0.67% | 0.24% | -0.90% | 116% | 100% |
| Overall | -0.62% | -0.67% | -0.45% | 116% | 105% | -0.54% | -0.34% | -0.57% | 117% | 101% |
| Class D | -0.51% | -0.51% | -0.38% | 116% | 105% | -0.49% | -0.27% | -0.93% | 116% | 102% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Additional test results

Table 8 Summary of overall performance for different maximum candidate number

| # 1 | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -0.26% | -0.30% | -0.28% | 104% | 100% | -0.19% | -0.16% | -0.22% | 106% | 100% |
| LB | -0.04% | -0.03% | 0.03% | 105% | 101% | -0.07% | -0.18% | -0.13% | 108% | 99% |
| # 2 | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -0.70% | -0.73% | -0.75% | 107% | 102% | -0.51% | -0.41% | -0.49% | 107% | 100% |
| LB | -0.31% | -0.27% | -0.27% | 108% | 102% | -0.28% | -0.27% | -0.18% | 111% | 101% |
| # 3 | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -0.98% | -0.93% | -0.98% | 109% | 104% | -0.74% | -0.60% | -0.68% | 109% | 101% |
| LB | -0.53% | -0.36% | -0.46% | 112% | 104% | -0.45% | -0.43% | -0.33% | 114% | 101% |
| # 4 | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -1.10% | -1.05% | -1.10% | 111% | 104% | -0.84% | -0.69% | -0.77% | 110% | 101% |
| LB | -0.61% | -0.72% | -0.56% | 114% | 104% | -0.56% | -0.54% | -0.60% | 116% | 101% |
| # 5 | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -1.13% | -1.07% | -1.12% | 113% | 104% | -0.86% | -0.69% | -0.79% | 111% | 101% |
| LB | -0.62% | -0.67% | -0.45% | 116% | 105% | -0.54% | -0.34% | -0.57% | 117% | 101% |

In Table 8, additional results for different maximum candidate number of affine merge candidate list are provided. It is reportedly shown that most gain of affine merge mode can be achieved by increasing the maximum candidate number to 3.

Conclusions

This contribution reports the results of integrating affine merge enhancement in JVET-J0024 to BMS1.0. Simulation results reportedly show on average 1.13%/0.62% luma BD-rate gain with 13%/16% encoding time and 4%/5% decoding time increase in RA/LB configurations when compared to VTM with affine on, and 0.86%/0.54% luma BD-rate drop with 11%/17% encoding time and 1%/1% decoding time increase in RA/LB configurations when compared to BMS1.0.

We propose to adopt this affine merge enhancement into the test model.

References

- “Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario”, JVET-J0024, April 2018

- J. Chen, E. Alshina, G. J. Sullivan, J-R. Ohm, “Algorithm Description of Joint Exploration Test Model 7 (JEM 7)”, JVET-G1001, July 2017.

- BMS reference software, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- H. Yang, S. Liu, K. Zhang, “Description of CE4: Inter prediction and motion vector coding”, JVET-J1024, April 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).