| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0188-v2 |
| --- | --- |

| Title: | CE4: Symmetrical mode for bi-prediction (Test 3.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang Jianle Chen | Tel: Email: | +8615017554651 chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of integrating symmetrical mode for bi-prediction in JVET-J0063 to BMS1.0. The proposed mode derives the reference index and MVD for the list 0 based on the assumption of linear motion. Simulation results reportedly shows on average 0.84% BD-rate saving with 5% encoding time for RA configuration over VTM-1.0, and 0.15% BD-rate saving with 2% encoding time for RA configuration over BMS-1.0.

Introduction

In this contribution, a new mode is proposed for motion information coding in bi-prediction. A symmetrical mode flag indicating whether symmetrical mode is used or not is explicitly signaled if the prediction direction is bi-prediction. When the flag is true, the reference index and MVD for list 0 are derived from motion information of list 1. Moreover, the reference index of list 1 is inferred as 0 for this mode.

As shown in Figure 1, the ref_idx_l0 is decided by searching a reference picture in list 0, which is mirrored with the reference picture for list 1. Since the distance between current picture and the backward reference picture is same as the distance between current picture and the forward reference picture, MVD0 is just set to –MVD1. The final motion vectors are shown in formula (1).

mvx0, mvy0=(mvpx0-mvdx1,mvpy0-mvdy1) mvx1, mvy1=(mvpx1+mvdx1,mvpy1+mvdy1)	            (1)

Figure 1 Illustration for symmetrical mode

Simulation results

The proposal has been implemented on top of BMS-1.0 [2]. The test condition descripted in JVET-J0024 [3] is used for verifying the contribution. Since this tool is only applied for bi-prediction with symmetrical reference pictures, it was tested under RA configuration only.

It can be seen that the proposed tool achieves averagely 0.84% BD-rate gain with 5% encoding time increase over VTM-1.0, and 0.15% BD-rate gain with 2% encoding time increase over BMS-1.0.

Table 1 Test results for symmetrical mode

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.85% | -0.73% | -0.68% | 105% | 99% | -0.19% | -0.18% | -0.20% | 101% | 101% |
| Class A2 | -1.36% | -1.00% | -0.93% | 106% | 100% | -0.19% | -0.20% | -0.24% | 102% | 100% |
| Class B | -0.88% | -0.58% | -0.53% | 105% | 100% | -0.15% | -0.14% | -0.17% | 102% | 100% |
| Class C | -0.41% | -0.29% | -0.23% | 105% | 100% | -0.09% | -0.07% | -0.12% | 101% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.84% | -0.62% | -0.56% | 105% | 100% | -0.15% | -0.14% | -0.18% | 102% | 100% |
| Class D | -0.41% | -0.17% | -0.22% | 106% | 96% | -0.07% | -0.04% | 0.06% | 102% | 97% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusions

A symmetrical mode for bi-prediction is proposed to reduce the bit-rate of motion information for bi-prediction. Simulation results report 0.84% BD-rate saving in luma for RA configuration with 5% encoding time increase over VTM-1.0, and 0.15% BD-rate gain with 2% encoding time increase over BMS-1.0.

We propose to adopt this new mode into the test model.

References

- H. Chen, H. Yang, J. Chen, “Symmetrical mode for bi-prediction”, JVET-J0063, April 2018.

- BMS reference software, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- H. Yang, S. Liu, K. Zhang, “Description of CE4: Inter prediction and motion vector coding”, JVET-J1024, April 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).