| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0189-v1 |
| --- | --- |

| Title: | Non-CE4: ATMVP simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang Jianle Chen | Tel: Email: | +8615017554651 chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

In CE4.2.5 and CE4.2.6, simplified ATMVP with one fixed collocated picture were proposed to reduce the memory access of temporal motion information of the advanced temporal motion vector prediction (ATMVP) in BMS. In this proposal, ATMVP is further improved by restricting the temporal vector to constrain the corresponding block inside the collocated CTU. Simulation results reportedly show on average 0.04% luma BD-rate drop for RA and 0.04% luma BD-rate gain for LB configuration over VTM1.0 with ATMVP and high precision mv on, 0.0x%, 0.0x% luma BD-rate drop for RA and LB configuration over BMS1.0.

Introduction

In current ATMVP in BMS, a reference picture and the corresponding block is determined by the motion information of the spatial neighbouring blocks of the current CU. To avoid the repetitive scanning process of neighbouring blocks, the first merge candidate in the merge candidate list of the current CU is used. The first available motion vector as well as its associated reference index are set to be the temporal vector and the index to the motion source picture. This way, in ATMVP, the corresponding block may be more accurately identified, compared with TMVP, wherein the corresponding block (sometimes called collocated block) is always in a bottom-right or center position relative to the current CU.

In the current ATMVP design of the BMS, all the reference pictures in L0 and L1 can be used as the collocated picture to derive the motion vectors of the CUs that are coded by the ATMVP mode. In CE4.2.5 and CE4.2.6, ATMVP is simplified by fetching the temporal motion information from the same collocated picture as in HEVC (signaled in slice header).

While in these proposals, the temporal vector is not limited, so that current CU might fetch motion information from any position of the collocated picture. In this contribution, the corresponding block is constrained inside the collocated CTU by restricting the temporal vector.

Test description

The proposal has been implemented on top of CE4.2.5 [1] and CE4.2.6 [2]. Following tests are designed for verifying the proposal.

Table 1 test specification for this contribution

| Test # | Test description |
| --- | --- |
| Test a | Fixed collocated picture and disable temporal vector for ATMVP |
| Test b | Simplified ATMVP with one fixed collocated picture in CE4.2.5 + temporal vector constraint |
| Test c | Test2 + picture/slice-level adaptation of ATMVP sub-block size in CE4.2.5 |
| Test d | Simplified ATMVP with one fixed collocated picture in CE4.2.6 + temporal vector constraint |
| Test e | Test3 + picture/slice-level adaptation of ATMVP sub-block size in CE4.2.5 |

Simulation results

The test condition descripted in JVET-J1024 [3] is used for verifying the contribution, and the results are summarized in Table 2 and Table 3. More detailed results are provided from Table 4 to Table 8.

Results in test a reportedly show that 0.19% luma BD-rate drop in RA configuration when fetches the motion information from collocated CU directly, compared to fetch the motion information with the guidance of temporal vector.

When apply this proposal with CE4.2.5.a, CE4.2.5.c, CE4.2.6.a, CE4.2.6.b, the luma BD-rate drop in RA configuration is 0.04%, 0.10%, 0.06%, 0.10%, respectively.

Table 2 Summary of overall performance in RA/LB configuration over VTM and BMS-ATMVP

| Test a | Over VTM1.0 |  |  |  |  | Over BMS1.0-ATMVP |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -0.92% | -0.65% | -0.67% | 106% | 121% | -0.58% | -0.51% | -0.49% | 100% | 106% |
| LB | -1.24% | -1.02% | -1.05% | 105% | 114% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Test b | Over VTM1.0 |  |  |  |  | Over BMS1.0-ATMVP |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -1.06% | -0.83% | -0.83% | 106% | 122% | -0.71% | -0.62% | -0.59% | 101% | 106% |
| LB | -1.35% | -1.27% | -1.24% | 105% | 115% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Test c | Over VTM1.0 |  |  |  |  | Over BMS1.0-ATMVP |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -1.01% | -0.78% | -0.78% | 103% | 106% | -0.66% | -0.56% | -0.53% | 100% | 100% |
| LB | -1.28% | -1.19% | -1.23% | 102% | 107% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Test d | Over VTM1.0 |  |  |  |  | Over BMS1.0-ATMVP |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -1.05% | -0.79% | -0.81% | 106% | 122% | -0.71% | -0.64% | -0.62% | 100% | 106% |
| LB | -1.29% | -1.36% | -1.05% | 105% | 115% | -1.10% | -1.22% | -1.21% | 99% | 109% |
| Test e | Over VTM1.0 |  |  |  |  | Over BMS1.0-ATMVP |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -1.00% | -0.73% | -0.75% | 103% | 105% | -0.66% | -0.59% | -0.55% | 100% | 100% |
| LB | -1.26% | -1.28% | -1.13% | 102% | 107% | -1.08% | -1.24% | -1.20% | 100% | 104% |

Table 3 Summary of overall performance in RA/LB configuration over VTM+ATMVP+HPMV and BMS

| Test a | Over VTM1.0+ATMVP+HPMV |  |  |  |  | Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | 0.19% | 0.23% | 0.19% | 100% | 100% | 0.16% | 0.11% | 0.12% | 101% | 100% |
| LB | 0.08% | 0.28% | 0.20% | 100% | 101% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Test b | Over VTM1.0+ATMVP+HPMV |  |  |  |  | Over BMS1.0 |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | 0.04% | 0.05% | 0.02% | 100% | 100% | 0.03% | 0.00% | 0.02% | 101% | 101% |
| LB | -0.04% | 0.02% | 0.01% | 100% | 101% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Test c | Over VTM1.0+ATMVP+HPMV |  |  |  |  | Over BMS1.0 |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | 0.10% | 0.10% | 0.08% | 98% | 87% | 0.07% | 0.06% | 0.08% | 101% | 95% |
| LB | 0.03% | 0.10% | 0.02% | 97% | 95% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Test d | Over VTM1.0+ATMVP+HPMV |  |  |  |  | Over BMS1.0 |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | 0.06% | 0.10% | 0.05% | 100% | 100% | 0.03% | -0.02% | -0.01% | 101% | 100% |
| LB | 0.02% | -0.08% | 0.20% | 100% | 101% | -0.08% | -0.13% | 0.05% | 100% | 101% |
| Test e | Over VTM1.0+ATMVP+HPMV |  |  |  |  | Over BMS1.0 |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | 0.10% | 0.15% | 0.11% | 97% | 87% | 0.08% | 0.03% | 0.06% | 101% | 94% |
| LB | 0.05% | 0.01% | 0.12% | 97% | 94% | -0.06% | -0.15% | 0.06% | 101% | 97% |

Test a

Table 4 Coding performance of test a (over VTM+ATMVP+HPMV and BMS)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM + ATMVP + HPMV |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | 0.24% | 0.15% | 100% | 100% | 0.12% | 0.08% | 0.14% | 101% | 100% |
| Class A2 | 0.15% | 0.22% | 0.15% | 100% | 100% | 0.09% | 0.08% | 0.16% | 102% | 100% |
| Class B | 0.12% | 0.15% | 0.11% | 100% | 100% | 0.09% | 0.02% | 0.03% | 102% | 100% |
| Class C | 0.33% | 0.33% | 0.33% | 100% | 100% | 0.32% | 0.25% | 0.19% | 101% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.19% | 0.23% | 0.19% | 100% | 100% | 0.16% | 0.11% | 0.12% | 101% | 100% |
| Class D | 0.13% | 0.08% | 0.26% | 99% | 100% | 0.16% | 0.14% | 0.10% | 100% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM + ATMVP + HPMV |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.03% | 0.33% | 0.28% | 100% | 101% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.13% | 0.15% | 0.05% | 100% | 100% | 0.09% | 0.03% | 0.08% | 101% | 102% |
| Class E | 0.07% | 0.34% | 0.29% | 100% | 101% | -0.06% | 0.04% | -0.27% | 101% | 98% |
| Overall | 0.08% | 0.28% | 0.20% | 100% | 101% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.11% | 0.07% | 0.29% | 100% | 102% | 0.10% | 0.50% | -0.88% | 100% | 97% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Test b

Table 5 Coding performance of test b (over VTM+ATMVP+HPMV and BMS)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM + ATMVP + HPMV |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.11% | 0.08% | 101% | 100% | 0.04% | -0.02% | 0.09% | 101% | 101% |
| Class A2 | 0.03% | 0.04% | 0.06% | 101% | 100% | 0.01% | 0.03% | 0.02% | 102% | 101% |
| Class B | 0.05% | 0.07% | 0.05% | 100% | 101% | 0.02% | -0.02% | 0.01% | 102% | 100% |
| Class C | 0.03% | -0.01% | -0.08% | 100% | 100% | 0.05% | 0.02% | -0.02% | 101% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.04% | 0.05% | 0.02% | 100% | 100% | 0.03% | 0.00% | 0.02% | 101% | 101% |
| Class D | 0.03% | 0.12% | 0.15% | 100% | 100% | 0.03% | -0.01% | 0.02% | 100% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM + ATMVP + HPMV |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.02% | 0.19% | -0.01% | 100% | 101% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.07% | -0.04% | -0.13% | 100% | 101% | -0.09% | -0.06% | -0.19% | 101% | 102% |
| Class E | -0.09% | -0.20% | 0.22% | 101% | 102% | -0.15% | -0.36% | 0.30% | 101% | 100% |
| Overall | -0.04% | 0.02% | 0.01% | 100% | 101% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | -0.02% | -0.09% | 100% | 102% | 0.00% | 0.08% | -0.87% | 100% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Test c

Table 6 Coding performance of test c (over VTM+ATMVP+HPMV and BMS)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM + ATMVP + HPMV |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.14% | 0.11% | 98% | 87% | 0.03% | 0.04% | 0.10% | 101% | 96% |
| Class A2 | 0.08% | 0.06% | 0.08% | 98% | 84% | 0.04% | 0.10% | 0.05% | 101% | 94% |
| Class B | 0.11% | 0.12% | 0.09% | 97% | 88% | 0.07% | 0.00% | 0.08% | 101% | 95% |
| Class C | 0.13% | 0.09% | 0.04% | 97% | 88% | 0.13% | 0.12% | 0.09% | 101% | 94% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.10% | 0.10% | 0.08% | 98% | 87% | 0.07% | 0.06% | 0.08% | 101% | 95% |
| Class D | 0.14% | 0.21% | 0.24% | 97% | 87% | 0.16% | 0.16% | 0.12% | 100% | 93% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM + ATMVP + HPMV |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.02% | 0.19% | 0.03% | 97% | 95% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.07% | 0.03% | -0.15% | 98% | 96% | -0.09% | -0.11% | -0.26% | 100% | 99% |
| Class E | 0.18% | 0.02% | 0.25% | 97% | 91% | -0.07% | 0.00% | 0.07% | 101% | 95% |
| Overall | 0.03% | 0.10% | 0.02% | 97% | 95% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | -0.08% | -0.02% | 98% | 97% | 0.02% | 0.22% | -0.89% | 99% | 97% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Test d

Table 7 Coding performance of test d (over VTM+ATMVP+HPMV and BMS)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM + ATMVP + HPMV |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.18% | 0.11% | 100% | 100% | 0.04% | 0.00% | 0.00% | 101% | 101% |
| Class A2 | 0.05% | 0.07% | 0.06% | 101% | 100% | 0.01% | -0.01% | 0.08% | 101% | 100% |
| Class B | 0.06% | 0.10% | 0.07% | 100% | 100% | 0.03% | -0.08% | -0.03% | 101% | 100% |
| Class C | 0.05% | 0.05% | -0.04% | 100% | 101% | 0.06% | 0.02% | -0.07% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.06% | 0.10% | 0.05% | 100% | 100% | 0.03% | -0.02% | -0.01% | 101% | 100% |
| Class D | 0.06% | 0.03% | 0.00% | 99% | 100% | 0.05% | 0.01% | 0.02% | 100% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM + ATMVP + HPMV |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.02% | 0.13% | 0.29% | 100% | 101% | -0.04% | -0.33% | 0.21% | 100% | 101% |
| Class C | -0.02% | -0.29% | -0.07% | 100% | 101% | -0.08% | 0.08% | -0.01% | 100% | 101% |
| Class E | 0.09% | -0.15% | 0.42% | 101% | 101% | -0.16% | -0.10% | -0.14% | 101% | 101% |
| Overall | 0.02% | -0.08% | 0.20% | 100% | 101% | -0.08% | -0.13% | 0.05% | 100% | 101% |
| Class D | 0.08% | -0.14% | -0.04% | 100% | 102% | 0.03% | 0.18% | 0.13% | 100% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Test e

Table 8 Coding performance of test e (over VTM+ATMVP+HPMV and BMS)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM + ATMVP + HPMV |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.15% | 0.11% | 97% | 87% | 0.04% | 0.06% | 0.04% | 102% | 96% |
| Class A2 | 0.09% | 0.11% | 0.11% | 97% | 84% | 0.03% | -0.01% | 0.08% | 101% | 95% |
| Class B | 0.11% | 0.17% | 0.14% | 97% | 88% | 0.08% | -0.02% | 0.06% | 102% | 94% |
| Class C | 0.14% | 0.15% | 0.07% | 97% | 87% | 0.14% | 0.11% | 0.05% | 101% | 93% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.10% | 0.15% | 0.11% | 97% | 87% | 0.08% | 0.03% | 0.06% | 101% | 94% |
| Class D | 0.18% | 0.16% | 0.14% | 97% | 85% | 0.17% | 0.12% | 0.13% | 100% | 91% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM + ATMVP + HPMV |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.01% | 0.17% | 0.37% | 97% | 95% | -0.03% | -0.32% | 0.13% | 101% | 98% |
| Class C | -0.01% | -0.29% | -0.07% | 98% | 95% | -0.09% | 0.09% | -0.05% | 101% | 98% |
| Class E | 0.21% | 0.15% | -0.04% | 97% | 91% | -0.08% | -0.21% | 0.07% | 101% | 94% |
| Overall | 0.05% | 0.01% | 0.12% | 97% | 94% | -0.06% | -0.15% | 0.06% | 101% | 97% |
| Class D | 0.09% | -0.02% | -0.04% | 98% | 95% | 0.03% | 0.11% | 0.18% | 100% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusions

In this proposal, ATMVP is further improved by restricting the temporal vector to constrain the corresponding block inside the collocated CTU. Simulation results reportedly show on average 0.04% luma BD-rate drop for RA and 0.04% luma BD-rate gain for LB configuration over VTM1.0 with ATMVP and high precision mv on, 0.0x%, 0.0x% luma BD-rate drop for RA and LB configuration over BMS1.0.

We proposal to adopt this ATMVP simplification into the test model.

References

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE4/CE4-2.5

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE4/CE4-2.6

- H. Yang, S. Liu, K. Zhang, “Description of CE4: Inter prediction and motion vector coding”, JVET-J1024, April 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).