| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0190 |
| --- | --- |

| Title: | CE3: Tests of cross-component linear model in BMS (Test 4.1.8, 4.1.9, 4.1.10, 4.1.11) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Ma, Haitao Yang, Jianle Chen | Email: | maxiang6@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

For cross component prediction, there are Cb-to-Cr prediction, Pure LM (LM only or LM without Cb-to-Cr prediction), multiple model LM (MMLM), and multiple down-sampling filter LM (MFLM) in BMS1.0. This contribution reports the tests results of those LM related modes, including

- PureLM (LM only or LM without Cb-to-Cr prediction)

- LM (PureLM+ Cb-to-Cr prediction)

- LM + MMLM

- LM + MMLM + MFLM

Simulation results reportedly show BD rate saving on Y, Cb and Cr components for AI configuration over VTM1.0:

- PureLM: 					1.19%, 9.01%, and 8.00%, with 110% EncTime and 102% DecTime;

- LM: 						1.53%, 9.80% and 10.48%, with 113% EncTime and 104% DecTime;

- LM + MMLM: 			1.66%, 10.36% and 11.38%, with 122% EncTime and 104% DecTime;

- LM + MMLM + MFLM: 	1.68%, 10.65% and 11.59%, with 142% EncTime and 106 DecTime.

Tests description

For cross component prediction, there are Cb-to-Cr prediction, Pure LM, MMLM, and MFLM in BMS1.0. For the detail description of those modes, please refer to [1] and [2].

There are 4 tests totally:

Test 4.1.8: PureLM (LM only or LM without Cb-to-Cr prediction)

Test 4.1.9: LM (PureLM+ Cb-to-Cr prediction)

Test 4.1.10: LM+MMLM

Test 4.1.11: LM+MMLM+MFLM

Experimental results

The proposed tests are tested under VVC CTC [3], and the results are summarized in Table 1, and Table2. More detailed results are provided in Tabel3, Table4, Tabel5, Table6, Table7, Table8, Table9, and Table10.

Summary of all the tests

Table 1 summary of all the tests results of all tests (AI configuration)

Table 2 summary of all the tests results of all tests (RA configuration)

Test 4.1.8: Pure LM (LM only)

Table 3 test results of Pure LM (AI configuration)

Table 4 test results of Pure LM (RA configuration)

Test 4.1.9: LM (PureLM+ Cb-to-Cr prediction)

Table 5 test results of LM (AI configuration)

Table 6 test results of LM (RA configuration)

Test 4.1.10: LM+MMLM

Table 7 test results of LM+MMLM (AI configuration)

Table 8 test results of LM+MMLM (RA configuration)

Test 4.1.11: LM+MMLM+MFLM

Table 9 test results of LM+MMLM+MFLM (AI configuration)

Table 10 test results of LM+MMLM+MFLM (RA configuration)

Conclusions

This proposal presents the test results of CCLM in BMS1.0. It can be seen that, at AI configuration over VTM1.0, MMLM provide 0.13%, 0.56% and 0.90% higher coding gain on Y, Cb and Cr components beyond LM mode. And MFLM can provide 0.02%, 0.29% and 0.21% higher coding gain on Y, Cb and Cr components beyond LM+MMLM mode.

References

- Jianle Chen, Elena Alshina, Gary J. Sullivan, Jens-Rainer Ohm, and Jill Boyce, “Algorithm Description of Joint Exploration Test Model 7 (JEM 7)”, JVET-G1001-v1, July 2017

- Geert Van der Auwera, Jin Heo, and Alexey Filippov, “Description of Core Experiment 3: Intra Prediction and Mode Coding”, JVET-J1023_r1, Apr. 2018

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, April 2018.

Patent rights declaration(s)

The authors of this contribution do not know whether Huawei Technologies Co., Ltd has any current or pending patent rights relating to the technology described in this contribution