| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0191 |
| --- | --- |

| Title: | CE3: Multi-directional LM (Test 4.2.1, 4.2.2, 4.2.3, 4.2.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Ma, Haitao Yang, Jianle Chen | Email: | maxiang6@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

For cross component prediction, there are LM, multiple model LM (MMLM), and multiple down-sampling filter LM (MFLM) in BMS1.0. Besides, in this contribution, multi-directional LM (MDLM) is presented. This contribution reports the results of MDLM related tests, including:

- MDLM

- LM+MDLM

- LM+MDLM+MMLM

- LM+MDLM+MMLM+MFLM

Simulation results reportedly show BD rate saving on Y, Cb and Cr components for AI configuration over VTM1.0:

- MDLM:                  1.20%, 9.50%, and 8.54%, with 121% EncTime and 103% DecTime;

- LM+MDLM:              1.61%, 11.11% and 12.07%, with 135% EncTime and 106% DecTime;

- LM+MDLM+MMLM: 			1.67%, 11.40% and 12.46%, with 145% EncTime and 106% DecTime;

- LM+MDLM+MMLM+MFLM:1.70%, 11.65% and 12.68%, with 165% EncTime and 108% DecTime;

Tests description

MDLM description

Besides the above template and left template can be used to calculate the linear model coefficients together, they also can be used alternatively in the other 2 LM modes, called MDLM_A, and MDLM_L modes.

In MDLM_A mode, only the above template are used to calculate the linear model coefficients. To get more samples, the above template are extended to (W+H).

Figure 1: MDLM_A mode

In MDLM_L mode, only left template are used to calculate the linear model coefficients. To get more samples, the left template are extended to (H+W).

Figure 2: MDLM_L mode

For a non-square coding block, another (N-W-H) samples will be sampled from (W+H) samples with equal steps to meet the condition of power of 2. Here N is the number of reference samples (to calculate the linear model coefficient). Here note that, N is the smallest integer value that is not smaller than (W+H), and N is a value of power of 2.

All tests

There are LM, MMLM, and MFLM in BMS1.0. For the detail description of those modes, please refer to [1] and [2].

There are 4 MDLM related tests totally:

Test 4.2.1: MDLM

Test 4.2.2: LM+MDLM

Test 4.2.3: LM+MDLM+MMLM

Test 4.2.4: LM+MDLM+MMLM+MFLM

Experimental results

The proposed tests are tested under VVC CTC [3], and the results are summarized in Table 1, and Table2. More detailed results are provided in Tabel3, Table4, Tabel5, Table6, Table7, Table8, Table9, and Table10.

Summary of all the tests

Table 1 summary of all the tests results of all tests (AI configuration)

Table 2 summary of all the tests results of all tests (RA configuration)

Test 4.2.1: MDLM

Table 3 test results of MDLM (AI configuration)

Table 4 test results of MDLM (RA configuration)

Test 4.2.1: LM+MDLM

Table 5 test results of LM+MDLM (AI configuration)

Table 6 test results of LM+MDLM (RA configuration)

Test 4.2.3: LM+MDLM+MMLM

Table 7 test results of LM+MDLM +MMLM (AI configuration)

Table 8 test results of LM+MDLM +MMLM (RA configuration)

Test 4.2.4: LM+MDLM+MMLM+MFLM

Table 9 test results of LM+MDLM +MMLM+MFLM (AI configuration)

Table 10 test results of LM+MDLM +MMLM+MFLM (RA configuration)

Conclusions

This proposal presents the results of MDLM related tests. It can be seen that, at AI configuration over VTM1.0, MDLM provide 1.20%, 9.50% and 8.54% coding gain on Y, Cb and Cr components. And LM+MDLM can provide 1.61%, 11.11% and 12.07% coding gain on Y, Cb and Cr components.

References

- Jianle Chen, Elena Alshina, Gary J. Sullivan, Jens-Rainer Ohm, and Jill Boyce, “Algorithm Description of Joint Exploration Test Model 7 (JEM 7)”, JVET-G1001-v1, July 2017

- Geert Van der Auwera, Jin Heo, and Alexey Filippov, “Description of Core Experiment 3: Intra Prediction and Mode Coding”, JVET-J1023_r1, Apr. 2018

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, April 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).