| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0192-v1 |
| --- | --- |

| Title: | CE2-3.3 SAO_Palette results and discussion |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Fabien Racapé | Tel: Email: | philippe.bordes@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This document presents the results of CE2-3.3 which proposes a modification to the Sample Adaptive Offset (SAO) and discusses its complexity. Two variants are studied: test-a with SAO block size equal to CTU size and test-b where the SAO block size can be adapted at slice/picture level.

It is reported the tool proposed in CE2-3.3 (test-b) provides average BD-rate savings of 0.17% in AI, 0.38% in RA and 0.52% in LDB for Luma component compared to VTM-1.0 anchors. The average encoding runtimes are unchanged, and the decoding runtimes increases by 10% in average compared to the VTM-1.0 anchors.

# Introduction

In the proposed tool, the inheritance of the SAO parameters is modified compared to HEVC so that the blocks may inherit parameters from all the other blocks, as described in JVET-J0022. This brings more flexibility to SAO by widening the range of possible SAO parameters for one block, as depicted in Figure 1. The SAO palette is composed of a set of different SAO parameters. For each block, the syntax element sao_palette_idx indicates which SAO parameter to use.

Figure 1: SAO palette

The size of the SAO palette is decoded first (sao_palette_num_cand), next the SAO parameters of the palette are decoded in their order of use, so they can be parsed progressively. The syntax of the SAO parameters is the same as HEVC, except the merge flags. For each block, the syntax element sao_palette_idx is parsed. At the post-filtering stage, the list of SAO parameters in the palette is re-ordered, so that the SAO parameters which were already used by the closest neighboring blocks appear on top of the list.

Several different SAO block sizes are evaluated. In the slice header, a flag slice_sao_size_id indicates whether the SAO block size is same as CTU (cuWidth, cuHeight) or if it is different. Several sets of SAO size are used (see Table 1-1).

Table 1-1: SAO block sizes used in test-b.

| slice_sao_size_id | SAO block width | SAO block height |  |
| --- | --- | --- | --- |
| 0 | cuWidth | cuHeight |  |
| 1 | cuWidth/2 | cuHeight/2 |  |
| 2 | cuWidth/2 | cuHeight |  |
| 3 | cuWidth | cuHeight/2 |  |
| 4 | 2 * cuWidth | cuHeight |  |
| 5 | cuWidth | 2 * cuHeight |  |

- Questions to be answered

  - Coding gains of SAO palette (SAO block size same as CTU size).

  - Coding gains of the whole proposal, i.e., SAO palette with adaptive SAO block size (see Table 1-1).

# Results and discussion

## BD-rate gains

The Table 2-1 and Table 2-2 summarize the BD-rate gains relative to the VTM-1.0 and BMS-1.0 anchor(s), for test-a and test-b respectively.

Table 2-1: BD-rate gains of SAO-palette of test-a.

| test a | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -1.26% | -1.36% | 100% | 90% | -0.16% | -0.93% | -0.57% | 100% | 95% |
| Class A2 | -0.13% | -0.58% | -0.32% | 100% | 88% | -0.10% | -0.42% | -0.24% | 103% | 94% |
| Class B | -0.15% | -0.29% | -0.31% | 100% | 89% | -0.15% | -0.04% | 0.14% | 99% | 95% |
| Class C | -0.18% | 0.22% | 0.32% | 100% | 90% | -0.17% | 0.47% | 0.61% | 96% | 93% |
| Class E | -0.16% | -0.53% | -0.81% | 100% | 103% | -0.16% | 0.00% | -0.24% | 101% | 95% |
| Overall | -0.17% | -0.43% | -0.43% | 100% | 92% | -0.15% | -0.13% | 0.00% | 99% | 94% |
| Class D | -0.08% | -0.05% | 0.00% | 100% | 90% | -0.09% | 0.11% | 0.31% | 102% | 93% |

| test a | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.45% | -2.31% | -2.25% | 100% | 99% | -0.25% | -1.94% | -1.92% | 105% | 94% |
| Class A2 | -0.29% | -1.01% | -0.41% | 101% | 95% | -0.19% | -1.55% | -0.93% | 105% | 92% |
| Class B | -0.31% | -0.76% | -0.56% | 100% | 82% | -0.23% | -0.93% | -0.64% | 104% | 92% |
| Class C | -0.14% | 0.06% | -0.01% | 100% | 81% | -0.07% | -0.35% | -0.31% | 104% | 89% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.29% | -0.90% | -0.72% | 100% | 87% | -0.18% | -1.10% | -0.87% | 105% | 92% |
| Class D | -0.12% | 0.47% | 0.30% | 100% | 84% | -0.07% | 0.19% | 0.25% | 103% | 90% |

| test a | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.50% | -2.02% | -2.68% | 99% | 96% | -0.50% | -2.82% | -2.81% | 100% | 97% |
| Class C | -0.18% | -0.67% | -0.42% | 100% | 84% | -0.24% | -0.74% | -1.11% | 102% | 95% |
| Class E | -0.42% | -2.14% | -2.45% | 85% | 95% | -0.72% | -2.29% | -3.56% | 103% | 99% |
| Overall | -0.37% | -1.60% | -1.87% | 96% | 92% | -0.47% | -1.99% | -2.43% | 101% | 97% |
| Class D | -0.10% | -0.24% | -0.54% | 100% | 93% | -0.08% | -0.06% | -0.77% | 100% | 94% |

Table 2-2: BD-rate gains of SAO-palette of test-b.

| test b | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -1.21% | -1.32% | 102% | 93% | -0.15% | -0.83% | -0.52% | 101% | 100% |
| Class A2 | -0.12% | -0.43% | -0.24% | 105% | 93% | -0.08% | -0.25% | -0.12% | 101% | 99% |
| Class B | -0.16% | -0.26% | -0.43% | 101% | 94% | -0.15% | 0.01% | 0.12% | 102% | 97% |
| Class C | -0.20% | 0.27% | 0.24% | 100% | 91% | -0.18% | 0.51% | 0.66% | 101% | 92% |
| Class E | -0.17% | -0.29% | -0.52% | 100% | 96% | -0.15% | 0.25% | 0.02% | 101% | 96% |
| Overall | -0.17% | -0.33% | -0.41% | 102% | 93% | -0.14% | -0.02% | 0.07% | 101% | 96% |
| Class D | -0.09% | -0.05% | -0.02% | 100% | 90% | -0.08% | 0.10% | 0.30% | 101% | 93% |

| test b | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.50% | -2.70% | -2.55% | 95% | 106% | -0.29% | -2.19% | -2.29% | 100% | 98% |
| Class A2 | -0.39% | -1.23% | -0.53% | 96% | 104% | -0.26% | -1.86% | -1.11% | 100% | 96% |
| Class B | -0.43% | -1.07% | -0.97% | 99% | 98% | -0.31% | -1.28% | -1.15% | 100% | 94% |
| Class C | -0.21% | -0.08% | -0.33% | 99% | 94% | -0.10% | -0.55% | -0.64% | 100% | 93% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.38% | -1.16% | -1.03% | 97% | 100% | -0.24% | -1.38% | -1.23% | 100% | 95% |
| Class D | -0.13% | 0.05% | -0.05% | 100% | 92% | -0.04% | -0.16% | 0.01% | 100% | 94% |

| test b | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.67% | -2.49% | -3.17% | 98% | 119% | -0.61% | -3.29% | -3.64% | 104% | 98% |
| Class C | -0.28% | -0.99% | -1.36% | 99% | 110% | -0.33% | -1.28% | -1.49% | 104% | 95% |
| Class E | -0.57% | -2.80% | -3.91% | 97% | 101% | -0.80% | -3.20% | -4.57% | 107% | 97% |
| Overall | -0.52% | -2.07% | -2.75% | 98% | 111% | -0.56% | -2.60% | -3.15% | 105% | 97% |
| Class D | -0.14% | -0.07% | -0.42% | 99% | 115% | -0.09% | -0.41% | -1.60% | 104% | 94% |

## Complexity analysis

The average encoder runtime is not increased (see Table 2-1 and Table 2-2). The decoder runtime is increased by 10% in average in LDB. The reason is the amount of SAO blocks with non-zero parameters (sao_type_idx_luma != 0) with SAO-palette is significantly greater than with anchors (VTM-1.0)  as depicted in Figure 2 and Figure 3 for RA class-B. That’s because the SAO parameters choices are globally optimized to the whole picture statistics and maximally re-used. However, note that the worst case (maximum theoretical number of CTUs using non-zero offsets) is unchanged compared to HEVC.

Figure 2: average percentage of non-zero SAO blocks (RA – class-B) in VTM-1.0 (blue) and test-b

(orange) in luma.

Figure 3: average percentage of non-zero SAO blocks (RA – class-B) in VTM-1.0 (blue) and test-b

(orange) in chroma.

## Parallelization discussion

The parsing of the index of the SAO parameter is made independently to the parsing of the other SAO blocks. However, at the SAO filtering stage, the list of SAO parameters is re-ordered so that the neighboring SAO parameters appear first, the index indicates which SAO parameter in the list to use. In test-b, the considered neighborhood is the same as the causal raster scan SAO blocks (see Figure 4-left).

Figure 4: causal neighborhood used for SAO parameters re-ordering.

One can restrain the neighborhood used for the SAO parameter list re-ordering to be included in the wavefront parallel causal area as depicted in Figure 4-right when SAO block size is same as CTU size (slice_sao_size_id=0), and as depicted in Figure 5 when slice_sao_size_id is equal to 1 (Figure 5-left) or 3 (Figure 5-right).

Figure 5: causal neighborhood used for SAO parameters re-ordering with wavefront parallelization.

The BD-rate gains obtained with test-b when the re-ordering of the SAO parameters is restrained to the causal wavefront area is depicted in Table 2-3 (detailed results in template_J1005_WPP_test_b.xlsm).

Table 2-3: BD-rate gains of SAO-palette of test-b with wavefront parallelization.

| test b + WPP | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -1.18% | -1.31% | 105% | 111% |  |  |  |  |  |
| Class A2 | -0.12% | -0.34% | -0.20% | 107% | 109% |  |  |  |  |  |
| Class B | -0.17% | -0.21% | -0.36% | 104% | 103% |  |  |  |  |  |
| Class C | -0.20% | 0.27% | 0.26% | 102% | 102% |  |  |  |  |  |
| Class E | -0.17% | -0.30% | -0.54% | 104% | 109% |  |  |  |  |  |
| Overall | -0.18% | -0.30% | -0.38% | 104% | 106% |  |  |  |  |  |
| Class D | -0.09% | -0.05% | -0.01% | 102% | 102% |  |  |  |  |  |

| test b + WPP | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.52% | -2.76% | -2.58% | 99% | 118% |  |  |  |  |  |
| Class A2 | -0.37% | -1.16% | -0.46% | 104% | 113% |  |  |  |  |  |
| Class B | -0.41% | -1.00% | -0.88% | 104% | 110% |  |  |  |  |  |
| Class C | -0.19% | -0.07% | -0.25% | 101% | 106% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.37% | -1.14% | -0.97% | 102% | 111% |  |  |  |  |  |
| Class D | -0.12% | 0.26% | 0.12% | 103% | 104% |  |  |  |  |  |

| test b + WPP | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.62% | -2.44% | -3.32% | 97% | 113% |  |  |  |  |  |
| Class C | -0.30% | -1.01% | -1.14% | 95% | 97% |  |  |  |  |  |
| Class E | -0.50% | -2.74% | -4.24% | 103% | 108% |  |  |  |  |  |
| Overall | -0.49% | -2.04% | -2.82% | 98% | 106% |  |  |  |  |  |
| Class D | -0.14% | 0.15% | -0.56% | 94% | 102% |  |  |  |  |  |

We also tested to restrain the neighborhood used for the SAO parameter list re-ordering to one line of CTUs above current CTU, included in the wavefront parallel causal area as depicted in Figure 6 when slice_sao_size_id is equal to 1 (Figure 6-left) or 3 (Figure 6-right).

Figure 6: causal neighborhood used for SAO parameters re-ordering limited to one line of CTUs inside wavefront causal area.

The BD-rate gains obtained with test-b when the re-ordering of the SAO parameters is restrained to one line of CTUs above current CTU include in the causal wavefront area is depicted in Table 2-4 (detailed results in template_J1005_WPP_V2_test_b.xlsm).

Table 2-4: BD-rate gains of SAO-palette of test-b with wavefront parallelization (restricted to one line of CTUs inside wavefront causal area).

| test b + WPP one line | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.49% | -2.73% | -2.50% | 97% | 119% |  |  |  |  |  |
| Class A2 | -0.37% | -1.01% | -0.41% | 100% | 115% |  |  |  |  |  |
| Class B | -0.42% | -1.01% | -0.89% | 100% | 108% |  |  |  |  |  |
| Class C | -0.18% | -0.07% | -0.27% | 98% | 103% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.36% | -1.11% | -0.95% | 99% | 110% |  |  |  |  |  |
| Class D | -0.12% | 0.26% | 0.12% | 98% | 100% |  |  |  |  |  |

# Conclusion

In CE2-3.3, a modification of SAO is proposed which provides average BD-rate savings (test-b) of 0.17% in AI, 0.38% in RA and 0.52% in LDB for Luma component compared to VTM-1.0 anchors. The average encoding runtimes are unchanged, and the decoding runtimes increases by less than 10% in average compared to the VTM-1.0 anchors. It is reported the basic design can be adapted to wavefront parallelization constrain with almost same BD-rate performance (0.18% in AI, 0.37% in RA and 0.49% in LDB for Luma component compared to VTM-1.0 anchors).

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).