| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0193-v2 |
| --- | --- |

| Title: | CE4-related: On performance improvements of Enhanced Interpolation Filter (EIF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Maxim Sychev Georgy Zhulikov Timophey Solovyev Huanbang Chen Jianle Chen | Tel: Email: | sychev.maxim@huawei.com |
| Source: | Huawei Technologies Co.,Ltd., Moscow, Russia |  |  |

_____________________________

# Abstract

In this proposal the analysis of Enhanced Interpolation Filter (EIF) in inter prediction is provided with the further improvement. EIF was initially proposed in JVET-J0024 and further studied in JVET-K020. Currently studied version of EIF use 3-tap high-pass filter (EIF3) and proposed as a solution for complexity reduction and worst case memory bandwidth reduction raised in JVET-J0081 for sub-block size 4x4. EIF3 allow to achieves -0.25%/-0.41% of luma BD-rate in RA/LDB with complexity reduction by 4% (Enc) and 9% (Dec) for the VTM configuration (96%(Enc) 91%(Dec) in RA and 94%(Enc) 94%(Dec) in LDB). Further improvements of EIF3 is proposed in this document as the same approach but in half-pixel domain with 5-tap high-pass filter (EIF5). Proposed filters design allows full PU size processing which allows to avoid increasing of memory bandwidth in worst case scenario. The design is friendly for parallel implementation. EIF5 allow to achieve -0.53%/-0.32 of luma BD-rate in RA/LDB with low influence on complexity for the VTM configuration with BMS Affine in anchor (98%(Enc) 100%(Dec) in RA and 103%(Enc) 97%(Dec) in LDB) and -0.22%/-0.17 of luma BD-rate in RA/LDB with low influence on complexity for the BMS configuration (99%(Enc) 105%(Dec) in RA and 103%(Enc) 102%(Dec) in LDB).

Together with improved Affine (CE4.1.5c) EIF5 demonstrates even higher compression performance -0.69%/-0.53 of luma BD-rate in RA/LDB with low influence on complexity for the VTM configuration with CE4.1.5c in anchor (99%(Enc) 102%(Dec) in RA and 104%(Enc) 98%(Dec) in LDB) and -0.25%/-0.xx of luma BD-rate in RA/LDB with low influence on complexity for the BMS configuration (99%(Enc) 102%(Dec) in RA and xx%(Enc) xx%(Dec) in LDB)

# Introduction

Enhanced Interpolation Filter was introduced in JVET-J-0024 in San Diego in April 2018. Two filters was proposed based on bilinear interpolation with high-pass 3-tap filter in pixel precision and 5-tap filter in half-pixel precision (-6, 9, 26, 9 ,6)/32. Section 2 contain the analysis of influence different interpolation techniques on several affine interpolation methods, section 3 provides details of the proposed technique and section 4 reports detailed results.

# Interpolation techniques for non-translational inter prediction

In this section the wide list of CE4.1 affine model proposals provided with EIF on top of them. The reason is to demonstrate a trend of increasing performance of EIF on top of increased performance of affine models and, moreover, in the same time when gain is growing the complexity reduction is even higher.

Table 1

|  |  | CE4.1.X+EIF over VTM1.0 |  |  |  |  | EIF over CE4.1.X Affine |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Enc | Dec | Y | U | V | Enc | Dec |
| 1 | 4.1.1*1x1.4-tap.DCTIF | -3.27% | -2.50% | -2.57% |  |  | -0.29% | -0.33% | -0.37% |  |  |
| 2 | 4.1.1*1x1.4-tap.CE4.7.1 | -3.37% | -2.59% | -2.69% |  |  | -0.40% | -0.42% | -0.49% |  |  |
| 3 | 4.1.1*1x1.4-tap.J0024 | -3.45% | -2.71% | -2.86% |  |  | -0.48% | -0.54% | -0.68% |  |  |
| 4 | 4.1.2a (J0012, Nokia) | -3.30% | -2.38% | -2.37% | 140% | 110% | -0.33% | -0.20% | -0.16% | 102% | 100% |
| 5 | 4.1.2b (J0012, Nokia) | -2.90% | -1.97% | -1.93% | 134% | 105% | 0.10% | 0.23% | 0.30% | 98% | 95% |
| 6 | 4.1.3a (J0021, Qualcomm) | -3.52% | -2.68% | -2.79% | 135% | 115% | -0.32% | -0.28% | -0.38% | 95% | 93% |
| 7 | 4.1.3b (J0021, Qualcomm) | -3.74% | -2.77% | -2.95% | 135% | 116% | -0.35% | -0.23% | -0.37% | 96% | 93% |
| 8 | 4.1.3c (J0021, Qualcomm) | -4.28% | -3.21% | -3.42% | 156% | 117% | -0.42% | -0.29% | -0.53% | 93% | 93% |
| 9 | 4.1.3d (J0021, Qualcomm) | -4.26% | -3.22% | -3.41% | 144% | 117% | -0.42% | -0.28% | -0.51% | 94% | 93% |
| 10 | 4.1.4a (J0022, Technicolor) | -4.29% | -3.22% | -3.47% | 147% | 116% | -0.43% | -0.28% | -0.52% | 95% | 93% |
| 11 | 4.1.4b (J0022, Technicolor) | -4.31% | -3.26% | -3.40% | 145% | 115% | -0.45% | -0.33% | -0.47% | 94% | 93% |
| 12 | 4.1.4d*(J0022, Technicolor) | -4.32% | -3.23% | -3.40% | 147% | 115% | -0.45% | -0.28% | -0.43% | 95% | 93% |
| 13 | 4.1.5d (J0024, Huawei) | -3.23% | -2.41% | -2.52% | 132% | 102% | -0.25% | -0.23% | -0.32% | 96% | 91% |
| 14 | 4.1.5d*4x4 (J0024, Huawei) | -3.33% | -2.41% | -2.56% | 131% | 104% | -0.36% | -0.23% | -0.36% | 95% | 93% |
| 15 | 4.1.5e (J0024, Huawei) | -4.20% | -3.08% | -3.30% | 137% | 103% | -0.41% | -0.23% | -0.43% | 95% | 93% |
| 16 | 4.1.5e+4.2.10 (J0024, Huawei) | -5.01% | -3.92% | -4.20% | 157% | 118% | -0.48% | -0.28% | -0.49% | 93% | 90% |
| 17 | 4.1.6 (J0018, MediaTek) | -3.49% | -2.63% | -2.77% | 138% | 115% | -0.31% | -0.27% | -0.39% | 95% | 93% |
| 18 | 4.1.7a (J0017, LGE) | -3.52% | -2.61% | -2.73% | 138% | 115% | -0.31% | -0.25% | -0.31% | 97% | 93% |
| 19 | 4.1.7b (J0017, LGE) | -3.86% | -2.77% | -2.98% | 186% | 116% | -0.35% | -0.22% | -0.38% | 94% | 93% |
| 20 | 4.1.7c (J0017, LGE) | -3.76% | -2.75% | -2.94% | 152% | 115% | -0.31% | -0.22% | -0.39% | 95% | 93% |
| 21 | 4.1.8a (J0028, Sony) | -3.36% | -2.42% | -2.60% | 172% | 115% | -0.27% | -0.16% | -0.33% | 94% | 93% |
| 22 | 4.1.8b (J0028, Sony) | -3.31% | -2.43% | -2.58% | 172% | 115% | -0.26% | -0.18% | -0.32% | 95% | 93% |
| 23 | 4.1.8c (J0028, Sony) | -3.45% | -2.49% | -2.67% | 206% | 116% | -0.28% | -0.18% | -0.37% | 93% | 93% |

First 3 items of this table corresponds to pixel level interpolation with classic approach with 4-tap interpolation filter (for Chroma the DCTIF 4-tap is used for all 3 tests). There are 3 filters was tested:

- DCTIF Chroma interpolation filter from BMS

- Enhanced Chroma Interpolation filter proposed in CE4.7.1

- Enhanced Chroma Interpolation filter proposed in JVET-J0024 [2]

It can be seen that even in the same design the 4-tap filter based on EIF provides additional coding gain in inter prediction (almost twice better than DCTIF).

As can be noted that EIF3 allow to achieve up to 0.5% of BD-rate luma gain on top of one of the best method and demonstrate good complexity reduction 7% for encoder and 10% for decoder.

The amount of tests with different affine models also demonstrate the simplicity of implementation of EIF. It requires maximum half of an hour to implement it and check.

# Enhanced Interpolation Filter in half-pixel domain (EIF5)

In HEVC and in JEM, an 8-tap linear interpolation filter based on DCT-transform is used to generate the inter prediction block in motion compensation for Luma component. In the J0024 [2] and in CE4.1.5d(e), bilinear interpolation coupled with 3-tap high-pass inter interpolation filters (EIF3) is used for inter prediction of both Luma and Chroma component {-1,10, -1} with normalization factor 1/8. In J0025 [3] high-pass filter is 5-tap {-6, 9, 26, 9, -6} with normalization factor 1/32 and half-pel resolution.

The bilinear interpolation applied to compensate all fractional offset in half-pixel domain. High-pass filter applied immediately after bilinear interpolation. Memory bandwidth of 5-tap filter is exactly the same as for EIF3 and correspond to 4-tap interpolation filter for pixel-wise inter prediction. The parameters of the filters are fixed, the same filter is used for all predicted samples involved in non-translational (affine) motion compensation model when the width or height of affine sub-block is less than 8.

The summary table of comparative performance EIF3 and EIF5 is the following:

Table 3 Summary results for RA configuration on top of VTM1.0 and BMS with corresponded affine model

|  | EIF over VTM+Affine |  |  |  |  | EIF over BMS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc | Dec | Y | U | V | Enc | Dec |
| EIF 3tap | -0.25% | -0.23% | -0.32% | 96% | 91% | 0.07% | -0.08% | -0.09% | 100% | 98% |
| EIF 3tap (Affine=CE4.1.5c) | -0.41% | -0.23% | -0.43% | 95% | 93% | -0.03% | -0.09% | -0.14% | 98% | 97% |
| EIF 5tap | -0.53% | -0.53% | -0.69% | 98% | 100% | -0.22% | -0.17% | -0.23% | 99% | 105% |
| EIF 5tap (Affine=CE4.1.5c) | -0.69% | -0.63% | -0.78% | 99% | 102% | -0.25% | -0.28% | -0.36% | 99% | 102% |

As can be seen, EIF5 outperforms EIF3 almost twice, without much increase of complexity even by simple implementation. Moreover, EIF5 keep all advantages of EIF3 such as low memory bandwidth (exactly the same), low complexity (almost the same), simple implementation in SW and HW (a little bit higher due to half-pel resolution, but not so much).

Please also refer to JVET-K0194 to complexity and design related aspects…

# Results

According to the common test conditions in JVET-J1010 [6], simulations were performed for random-access (RA) and low-delay B (LDB) configuration with BMS-1.0 software in VTM and BMS configuration.

Table 2 Results for RA configuration on top of VTM1.0 and BMS w/o Affine (Test of Affine+EIF)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 noAffine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.32% | -1.69% | -1.82% | 144% | 108% | -1.18% | -0.85% | -0.89% | 107% | 104% |
| Class A2 | -6.98% | -5.54% | -5.58% | 148% | 120% | -3.89% | -2.68% | -2.75% | 108% | 105% |
| Class B | -3.95% | -3.00% | -3.40% | 147% | 109% | -2.52% | -2.05% | -2.20% | 108% | 102% |
| Class C | -1.17% | -0.93% | -0.97% | 144% | 105% | -0.78% | -0.71% | -0.75% | 108% | 105% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.49% | -2.69% | -2.87% | 146% | 110% | -2.06% | -1.58% | -1.66% | 108% | 104% |
| Class D | -1.16% | -0.89% | -1.14% | 143% | 105% | -0.92% | -0.97% | -0.76% | 108% | 105% |
| Class F (optional) | -2.18% | -2.17% | -2.19% | 140% | 103% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 3 Results for LDB configuration on top of VTM1.0 and BMS w/o Affine (Test of Affine+EIF)

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 noAffine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.83% | -2.31% | -2.62% | 181% | 109% | -2.70% | -2.16% | -2.29% | 123% | 103% |
| Class C | -1.56% | -1.25% | -1.21% | 160% | 104% | -1.23% | -1.16% | -1.31% | 118% | 103% |
| Class E | -2.71% | -1.40% | -1.59% | 230% | 103% | -2.08% | -1.92% | -1.74% | 143% | 100% |
| Overall | -2.38% | -1.73% | -1.90% | 184% | 106% | -2.05% | -1.77% | -1.83% | 126% | 102% |
| Class D | -2.34% | -1.99% | -2.01% | 151% | 105% | -1.92% | -1.84% | -2.40% | 118% | 103% |
| Class F (optional) | -2.73% | -2.66% | -2.65% | 167% | 101% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 3 Results for RA configuration relatively to VTM+Affine and BMS (Test of EIF only)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.36% | -0.11% | -0.17% | 97% | 102% | -0.06% | -0.02% | -0.09% | 99% | 105% |
| Class A2 | -1.19% | -0.84% | -1.19% | 98% | 102% | -0.27% | -0.14% | -0.20% | 99% | 105% |
| Class B | -0.74% | -0.87% | -1.11% | 97% | 99% | -0.31% | -0.22% | -0.31% | 99% | 104% |
| Class C | 0.11% | -0.16% | -0.16% | 99% | 99% | -0.19% | -0.23% | -0.25% | 100% | 105% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.53% | -0.53% | -0.69% | 98% | 100% | -0.22% | -0.17% | -0.23% | 99% | 105% |
| Class D | 0.67% | 0.15% | 0.09% | 99% | 100% | -0.30% | -0.46% | -0.47% | 100% | 107% |
| Class F (optional) | -0.73% | -0.79% | -0.78% | 105% | 101% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 3 Results for LDB configuration relatively to VTM+Affine and BMS (Test of EIF only)

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 + Affine |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.35% | -0.84% | -0.87% | 103% | 98% | -0.22% | -0.30% | -0.10% | 103% | 103% |
| Class C | -0.11% | -0.48% | -0.61% | 102% | 96% | -0.09% | 0.04% | -0.18% | 103% | 102% |
| Class E | -0.54% | 0.44% | 0.75% | 103% | 95% | -0.19% | -0.34% | -0.38% | 104% | 101% |
| Overall | -0.32% | -0.40% | -0.38% | 103% | 97% | -0.17% | -0.20% | -0.20% | 103% | 102% |
| Class D | 0.67% | 0.03% | 0.35% | 102% | 94% | -0.16% | -0.30% | 0.07% | 103% | 102% |
| Class F (optional) | -1.05% | -1.01% | -1.01% | 108% | 100% | -0.77% | -0.82% | -0.74% | 103% | 99% |

As was shown above EIF5 provide more coding gain on top of any Affine improvements. Here we providing results of EIF5 on top of CE4.1.5c.

Table 6 Results of EIF5+CE4.1.5c  for RA configuration relatively to VTM and BMS w/o Affine (Test of Affine+EIF)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 noAffine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.92% | -2.22% | -2.32% | 149% | 108% | -1.62% | -1.19% | -1.30% | 109% | 104% |
| Class A2 | -9.55% | -7.29% | -7.40% | 158% | 122% | -6.15% | -4.23% | -4.40% | 111% | 106% |
| Class B | -4.81% | -3.66% | -4.04% | 157% | 111% | -3.21% | -2.62% | -2.80% | 111% | 103% |
| Class C | -1.40% | -1.23% | -1.26% | 153% | 106% | -0.97% | -0.83% | -0.79% | 111% | 104% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -4.47% | -3.45% | -3.63% | 155% | 111% | -2.88% | -2.18% | -2.28% | 111% | 104% |
| Class D | -1.34% | -1.15% | -1.50% | 153% | 104% | -1.10% | -1.05% | -0.95% | 110% | 105% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 7 Results for LDB configuration relatively to VTM and BMS w/o Affine (Test of Affine+EIF)

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 noAffine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -3.39% | -2.78% | -2.96% | 190% | 110% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -1.84% | -1.92% | -1.91% | 166% | 105% | -1.30% | -1.39% | -1.21% | 121% | 102% |
| Class E | -3.40% | -0.61% | -1.27% | 247% | 104% | -2.46% | -1.94% | -1.87% | 150% | 100% |
| Overall | -2.88% | -1.95% | -2.19% | 194% | 107% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -2.43% | -2.53% | -2.72% | 156% | 104% | -1.96% | -2.29% | -2.12% | 120% | 103% |
| Class F (optional) | -4.70% | -4.57% | -4.18% | 175% | 102% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 8 Results for RA configuration relatively to VTM+CE4.1.5c and BMS+CE4.1.5c (Test of EIF only)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 noAffine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.38% | -0.05% | -0.12% | 97% | 103% | -0.05% | 0.01% | -0.04% | 99% | 103% |
| Class A2 | -1.75% | -0.93% | -1.42% | 99% | 102% | -0.79% | -0.16% | -0.50% | 99% | 102% |
| Class B | -0.85% | -0.98% | -1.16% | 98% | 102% | -0.44% | -0.75% | -0.76% | 99% | 102% |
| Class C | 0.07% | -0.38% | -0.33% | 100% | 103% | 0.25% | 0.01% | 0.01% | 100% | 103% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.69% | -0.63% | -0.78% | 99% | 102% | -0.25% | -0.28% | -0.36% | 99% | 102% |
| Class D | 0.63% | 0.09% | -0.20% | 100% | 104% | 0.62% | 0.34% | 0.23% | 100% | 105% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 9 Results for RA configuration relatively to VTM+CE4.1.5c and BMS+CE4.1.5c (Test of EIF only)

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 noAffine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.54% | -0.76% | -1.15% | 104% | 98% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.31% | -1.18% | -1.37% | 104% | 97% | 0.22% | -0.17% | 0.17% | 101% | 99% |
| Class E | -0.80% | 1.19% | 0.80% | 104% | 97% | -0.10% | 0.01% | 0.78% | 101% | 98% |
| Overall | -0.53% | -0.41% | -0.73% | 104% | 98% | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.61% | -0.20% | -0.43% | 103% | 96% | 1.22% | 0.69% | 1.25% | 101% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

# Conclusions

This contribution proposes Enhancement Interpolation Filter in half-pixel domain firstly described in JVET-J0025 [3] and integrated to BMS1.0. Proposed filters design allows full PU size processing which allows to avoid increasing of memory bandwidth in worst case scenario. The design is friendly for parallel implementation. Experimental results reportedly show -0.53%/-0.32 of luma BD-rate in RA/LDB with low influence on complexity for the VTM configuration with BMS Affine in anchor (98%(Enc) 100%(Dec) in RA and 103%(Enc) 97%(Dec) in LDB) and -0.22%/-0.17 of luma BD-rate in RA/LDB with low influence on complexity for the BMS configuration (99%(Enc) 105%(Dec) in RA and 103%(Enc) 102%(Dec) in LDB).

Together with improved Affine (CE4.1.5c) EIF5 demonstrates even higher compression performance -0.69%/-0.53 of luma BD-rate in RA/LDB with low influence on complexity for the VTM configuration with CE4.1.5c in anchor (99%(Enc) 102%(Dec) in RA and 104%(Enc) 98%(Dec) in LDB) and -0.25%/-0.xx of luma BD-rate in RA/LDB with low influence on complexity for the BMS configuration (99%(Enc) 102%(Dec) in RA and xx%(Enc) xx%(Dec) in LDB)

We are proposing to adopt Enhanced Interpolation Filter in half-pixel domain (EIF5) with new design or in 4-tap design into the test model.

# References

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVET-J1024, San Diego, USA, April 2018.

- A. Alshin, E. Alshina, K. Choi, N. Choi, W. Choi, S. Jeong, C. Kim, J. Min, J. H. Park, M. Park, M. W. Park, Y. Piao, A. Tamse, A. Dsouza, C. Pujara, H. Chen, J. Chen, R. Chernyak, S. Esenlik, A. Filippov, S. Gao, S. Ikonin, A. Karabutov, A. M. Kotra, X. Lu, X. Ma, V. Rufitskiy, T. Solovyev, V. Stepin, M. Sychev, T. Wang, Y.-K. Wang, W. Xu, H. Yang, V. Zakharchenko, H. Zhang, Y. Zhao, Z. Zhao, J. Zhou, C. Auyeung, H. Gao, I. Krasnov, R. Mullakhmetov, B. Wang, Y. F. Wong, G. Zhulikov, A. Abbas, D. Newman, J. An, X. Chen, Y. Lin, Q. Yu, J. Zheng, “Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario” JVET-J0024, San Diego, USA, April 2018.

- H. Chen, J. Chen, R. Chernyak, S. Esenlik, A. Filippov, S. Gao, S. Ikonin, A. Karabutov, A. M. Kotra, X. Lu, X. Ma, V. Rufitskiy, T. Solovyev, V. Stepin, M. Sychev, T. Wang, Y.-K. Wang, W. Xu, H. Yang, V. Zakharchenko, H. Zhang, Y. Zhao, Z. Zhao, J. Zhou, C. Auyeung, H. Gao, I. Krasnov, R. Mullakhmetov, B. Wang, Y. F. Wong, G. Zhulikov (Huawei), A. Abbas, D. Newman (GoPro), J. An, X. Chen, Y. Lin, Q. Yu, J. Zheng (HiSilicon), A. Alshin, E. Alshina, K. Choi, N. Choi, W. Choi, S. Jeong, C. Kim, J. Min, J. Park, M. Park, M. W. Park, Y. Piao, A. Tamse, A. Dsouza, C. Pujara (Samsung), “Description of SDR, HDR and 360° video coding technology proposal by Huawei, GoPro, HiSilicon, and Samsung – general application scenario”, JVET-J0025, San Diego, USA, April 2018.

- M. Zhou, W. Wan, T. Hellman, P. Chen, O. Hugosson, D. Symes, A. Duenas, E. Chai, “Comments on Test Model Development”, JVET-J0081, San Diego, USA, April 2018.

- A.Henkel, “Crosscheck of JVET-K0193: CE4-related: On performance improvements of Enhanced Interpolation Filter (EIF)”, JVET-K0448, Ljubljana,Slovenia, July 2018

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, April 2018.

- Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).