| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0194-v1 |
| --- | --- |

| Title: | CE4-related: On further complexity reduction of Enhanced Interpolation Filter (EIF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Maxim Sychev Georgy Zhulikov Timophey Solovyev Jianle Chen | Tel: Email: | sychev.maxim@huawei.com |
| Source: | Huawei Technologies Co.,Ltd., Moscow, Russia |  |  |

_____________________________

# Abstract

Proposed two aspects of design improvement and complexity reduction of Enhanced Interpolation Filter (EIF) for inter prediction. EIF was initially proposed in JVET-J0024 and further studied in JVET-K020. Currently studied version of EIF use 3-tap high-pass filter (EIF3) and proposed as a solution for complexity reduction and worst case memory bandwidth reduction raised in JVET-J0081 for sub-block size 4x4. There are two aspects included in the document:

- Directional Bilinear Interpolation Filter (DBIF) with reduced number of operation by 1/3 in respect to bilinear in EIF

- 16-bit friendly design of EIF

EIF3 allow to achieve -0.25%/-0.41% of luma BD-rate in RA/LDB with complexity reduction by 4% (Enc) and 9% (Dec) for the VTM configuration (96%(Enc) 91%(Dec) in RA and 94%(Enc) 94%(Dec) in LDB). The proposed DBIF demonstrate the same performance with low influence on complexity for the VTM configuration even in C implementation. Proposed filters design allows full PU size processing which allows to avoid increasing of memory bandwidth in worst case scenario. The design is friendly for parallel implementation. For each aspects the following influence on coding performance is demonstrated:

- -0.XX%/-0.XX of luma BD-rate in RA/LDB with low influence on complexity for the VTM configuration with BMS Affine in anchor (XX%(Enc) XX%(Dec) in RA and XX%(Enc) XX%(Dec) in LDB)

- -XX%/-0.XX of luma BD-rate in RA/LDB with low influence on complexity for the VTM configuration with BMS Affine in anchor (XX%(Enc) XX%(Dec) in RA and XX%(Enc) XX%(Dec) in LDB)

# Introduction

Enhanced Interpolation Filter was introduced in JVET-J-0024 in San Diego in April 2018. Two filters was proposed based on bilinear interpolation with high-pass 3-tap filter in pixel precision and 5-tap filter in half-pixel precision. Section 2 describes the bilinear interpolation filter adapted for non-translational interpolations, section 3 provides details of 16-bit design of EIF and in section 4 the influence of memory bandwidth constraints on EIF is provided.

# Directional Bilinear Interpolation Filter (DBIF)

As described in JVET-J0024, JVET-J0025 and in JVET-K020 the bilinear interpolation is the part of EIF. For the initial simplification of the interpolation method the classical bilinear interpolation was chosen. The classic bilinear interpolation has the following formula:

l=rix,iy1-fx1-fy+rix,iy+1*fx1-fy+rix+1,iy1-fxfy+rix+1,iy+1*fx*fy) 	(1)

- where rix,iy is the luminance of the reference pixel in integer position (ix,iy), fx and fy is the fractional offsets of interpolated position

It could be simplified to achieve 3 multiplication per one interpolated pixel:

m=rix,iy- rix,iy-rix,iy+1*fy

n=rix,iy+1- rix,iy+1-rix+1,iy+1*fy

l=m- n-m*fx

The 3 multiplication for one pixel is already small enough, but if we have the same fractional offset for each pixel (as for regular translational motion model) then it is possible to reuse already calculated values. So the right intermediate value n could be used as left intermediate value m for the next interpolated point in the raw. So for translational motion model there are only two multiplication is required. It case of non-translation motion model (like affine) we could not reuse intermediate values and always 3 multiplications required per one point as depicted in Figure 1.

Figure 1 Classical bilinear interpolation in case of non-translational transformations

The proposed approach allows to reuse intermediate values in the same style as for translational interpolation even for non-translational motion models. For non-translational interpolation proposed to use directional projection of interpolated point on the vertical line connecting integer position of reference pixels as depicted in Figure 2.

Figure 2 Proposed directional bilinear interpolation

For horizontal weighting the same coefficients is used as for classic approach – dx and (1-dx).

# 16-bit friendly design of EIF

Depending on precision of motion vectors and the size of the prediction unit different numbed of bit required for calculation in fixed point arithmetic. Due to horizontal and vertical steps of bilinear interpolation the result of the first step could exceed 16-bit range. In absence of appropriate operators in HW which could perform 2D bilinear operation in one command, we should reduce precision of intermediate values so that the input for the next step has only 16 bit for storage value. Even addition of 0.5 is not necessary before right shift operation for this intermediate rounding. It was not done in CE4.1.5d(e), but evidently could be performed with negligible influence on coding performance as demonstrated on the Table 1 and Table 2. And even with some complexity reduction.

Table 1 Result for RA configuration on top of VTM+BMS Affine+EIF anchor (current result is short simulations – will be replaced by full simulation results soon)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | -0.11% | 92% | 99% |
| Class A2 | 0.01% | -0.03% | 0.08% | 92% | 93% |
| Class B | -0.04% | -0.23% | 0.27% | 93% | 97% |
| Class C | -0.04% | -0.09% | 0.14% | 95% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.10% | 0.12% | 93% | 97% |
| Class D | -0.05% | -0.20% | 0.00% | 94% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 2 Result for LDB configuration on top of VTM+BMS Affine+EIF anchor (current result is short simulations – will be replaced by full simulation results soon)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | 0.06% | -0.03% | 90% | 98% |
| Class C | 0.01% | 0.17% | 0.13% | 94% | 100% |
| Class E | 0.16% | 0.21% | 0.04% | 86% | 99% |
| Overall | 0.01% | 0.14% | 0.04% | 90% | 99% |
| Class D | -0.03% | -0.24% | 0.00% | 94% | 101% |
| Class F (optional) |  |  |  |  |  |

# Conclusions

There are two aspects included in this contribution:

- Directional Bilinear Interpolation Filter (DBIF) with reduced number of operation by 1/3 in respect to bilinear in EIF

- 16-bit friendly design of EIF

Both aspect corresponds to filter design with full PU size processing which allows to avoid increasing of memory bandwidth in worst case scenario. The design is friendly for parallel implementation. Experimental results reportedly show on average 0.XX% luma BD-rate gain in RA configurations over to VTM anchor and show on average 0.XX% luma BD-rate gain over BMS configuration with low influence on complexity even in C implementation in respect to AVX2 anchor.

We are proposing further study of aspect a) and adoption of aspect b) jointly with one of the Enhanced Interpolation Filter (EIF3 or EIF5) into the test model.

# References

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVET-J1024, San Diego, USA, April 2018.

- A. Alshin, E. Alshina, K. Choi, N. Choi, W. Choi, S. Jeong, C. Kim, J. Min, J. H. Park, M. Park, M. W. Park, Y. Piao, A. Tamse, A. Dsouza, C. Pujara, H. Chen, J. Chen, R. Chernyak, S. Esenlik, A. Filippov, S. Gao, S. Ikonin, A. Karabutov, A. M. Kotra, X. Lu, X. Ma, V. Rufitskiy, T. Solovyev, V. Stepin, M. Sychev, T. Wang, Y.-K. Wang, W. Xu, H. Yang, V. Zakharchenko, H. Zhang, Y. Zhao, Z. Zhao, J. Zhou, C. Auyeung, H. Gao, I. Krasnov, R. Mullakhmetov, B. Wang, Y. F. Wong, G. Zhulikov, A. Abbas, D. Newman, J. An, X. Chen, Y. Lin, Q. Yu, J. Zheng, “Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario” JVET-J0024, San Diego, USA, April 2018.

- H. Chen, J. Chen, R. Chernyak, S. Esenlik, A. Filippov, S. Gao, S. Ikonin, A. Karabutov, A. M. Kotra, X. Lu, X. Ma, V. Rufitskiy, T. Solovyev, V. Stepin, M. Sychev, T. Wang, Y.-K. Wang, W. Xu, H. Yang, V. Zakharchenko, H. Zhang, Y. Zhao, Z. Zhao, J. Zhou, C. Auyeung, H. Gao, I. Krasnov, R. Mullakhmetov, B. Wang, Y. F. Wong, G. Zhulikov (Huawei), A. Abbas, D. Newman (GoPro), J. An, X. Chen, Y. Lin, Q. Yu, J. Zheng (HiSilicon), A. Alshin, E. Alshina, K. Choi, N. Choi, W. Choi, S. Jeong, C. Kim, J. Min, J. Park, M. Park, M. W. Park, Y. Piao, A. Tamse, A. Dsouza, C. Pujara (Samsung), “Description of SDR, HDR and 360° video coding technology proposal by Huawei, GoPro, HiSilicon, and Samsung – general application scenario”, JVET-J0025, San Diego, USA, April 2018.

- M. Zhou, W. Wan, T. Hellman, P. Chen, O. Hugosson, D. Symes, A. Duenas, E. Chai, “Comments on Test Model Development”, JVET-J0081, San Diego, USA, April 2018.

- X-Check JVET-Kxxxx

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, April 2018.

- Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).