| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0194-v2 |
| --- | --- |

| Title: | CE4-related: On further complexity reduction of Enhanced Interpolation Filter (EIF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Maxim Sychev Georgy Zhulikov Timophey Solovyev Jianle Chen | Tel: Email: | sychev.maxim@huawei.com |
| Source: | Huawei Technologies Co.,Ltd., Moscow, Russia |  |  |

_____________________________

# Abstract

Proposed two aspects of design improvement and complexity reduction of Enhanced Interpolation Filter (EIF) for inter prediction. EIF was initially proposed in JVET-J0024 and further studied in JVET-K020. Currently studied version of EIF use 3-tap high-pass filter (EIF3) and proposed as a solution for complexity reduction and worst case memory bandwidth reduction raised in JVET-J0081 for sub-block size 4x4. There are two aspects included in the document:

- Directional Bilinear Interpolation Filter (DBIF) with reduced number of operation by 1/3 in respect to bilinear in EIF

- 16-bit friendly design of EIF

EIF3 allow to achieve -0.25%/-0.41% of luma BD-rate in RA/LDB with complexity reduction by 4% (Enc) and 9% (Dec) for the VTM configuration (96%(Enc) 91%(Dec) in RA and 94%(Enc) 94%(Dec) in LDB). The proposed DBIF demonstrate the same performance with low influence on complexity for the VTM configuration even in C implementation with redundant operations. Proposed filters design allows full PU size processing which allows to avoid increasing of memory bandwidth in worst case scenario. The design is friendly for parallel implementation. For each aspects the following influence on coding performance is demonstrated:

- 0.05%/-0.09 of luma BD-rate in RA/LDB with not optimized implementation for the VTM configuration with BMS Affine and EIF3 in anchor (104%(Enc) 110%(Dec) in RA and 113%(Enc) 104%(Dec) in LDB). And for BMS configuration: 0.10%/0.19 of luma BD-rate in RA/LDB with not optimized implementation with EIF3 in anchor (100%(Enc) 105%(Dec) in RA and 105%(Enc) 103%(Dec) in LDB)

- 0.04/-0.08% of luma BD-rate in RA/LDB with low influence on complexity for the VTM configuration with BMS Affine and EIF3 in anchor (92%(Enc) 103%(Dec) in RA and 94%(Enc) 100%(Dec) in LDB). And for BMS configuration: 0.09%/0.18 of luma BD-rate in RA/LDB with EIF3 in anchor (96%(Enc) 104%(Dec) in RA and 96%(Enc) 101%(Dec) in LDB)

# Introduction

Enhanced Interpolation Filter was introduced in JVET-J-0024 [3] in San Diego in April 2018. Two filters was proposed based on bilinear interpolation with high-pass 3-tap filter in pixel precision and 5-tap filter in half-pixel precision [4]. Section 2 describes the bilinear interpolation filter adapted for non-translational interpolations, section 3 provides details of 16-bit design of EIF and in section 4the hardware implementation specific aspects are provided.

# Directional Bilinear Interpolation Filter (DBIF)

As described in JVET-J0024, JVET-J0025 and in JVET-K020 the bilinear interpolation is the part of EIF. For the initial simplification of the interpolation method the classical bilinear interpolation was chosen. The classic bilinear interpolation has the following formula:

l=rix,iy1-fx1-fy+rix,iy+1*fx1-fy+rix+1,iy1-fxfy+rix+1,iy+1*fx*fy) 	(1)

- where rix,iy is the luminance of the reference pixel in integer position (ix,iy), fx and fy is the fractional offsets of interpolated position

It could be simplified to achieve 3 multiplication per one interpolated pixel:

m=rix,iy- rix,iy+1-rix,iy*fy	(2)

n=rix,iy+1- rix+1,iy+1-rix,iy+1*fy	(3)

l=m- n-m*fx	(4)

Bi-linear interpolation in non-translational case have own fractional offset for each pixel.  To support full CU size processing there is only two additional vector multiplication is required:

fxfx=(fx+1)*k-1,0,1,2, .. w,w+1	(5)

fyfy=fy*k-1,0,1,2, .. w,w+1	(6)

k=1,2,3, .. w,w+2	(7)

After that we just need to apply mask for the whole vector to get fractional positions and use fxfx and fyfy in formulas (2), ((3) and ((4) as vector operations. High-Pass filter could be applied to the whole vector immediately right after bilinear interpolation by vector operations with only 2 lines with CU width. In comparison with current implementation of translation motion model the vector k (of w+filter length-1 elements) is the following:

k=1,1, .. 1,1  	(8)

The 3 multiplication for one pixel is already small enough, but if we have the same fractional offset for each pixel (as for regular translational motion model) then it is possible to reuse already calculated values. So the right intermediate value n could be used as left intermediate value m for the next interpolated point in the raw. So for translational motion model there are only two multiplication is required. In case of non-translation motion model (like affine) we could not reuse intermediate values and always 3 multiplications required per one point as depicted in Figure 1.

Figure 1 Classical bilinear interpolation in case of non-translational transformations

The proposed approach allows to reuse intermediate values in the same style as for translational interpolation even for non-translational motion models. For non-translational interpolation proposed to use directional projection of interpolated point on the vertical line connecting integer position of reference pixels as depicted in Figure 2.

Figure 2 Proposed directional bilinear interpolation for horizontal case (left) and vertical case (right).

For horizontal weighting the same coefficients are used as for classic approach – dx and (1-dx) according to the rule of similarity of triangles.

Figure 3 Possible further implementation of directional bilinear interpolation with vertical pipeline.

Figure 3 demonstrate the vertical pipeline where horizontal intermediate values from previous line are used for vertical interpolation (0th - blue then 1st - green, 2nd - yellow and 3rd - red). Similarly to horizontal case, the weighting factor – dy and (1-dy) is used instead of real distance along direction, according to the rule of similarity of triangles in order to keep normalization factor as a power of two. Only one line of circular memory buffer is required (one memory unit in each block). It is possible to split CU on slices in order to further parallelization (bit-exactly). One of the possible implementation is provided in section 4.

The influence on coding performance is quite small as demonstrated on the Table 1 and Table 2. And even straightforward implementation with redundant calculations do not make big influence on complexity. There is still enough room for further improvements both in quality and complexity reduction.

Table 1 Results for RA configuration on top of VTM+BMS Affine+EIF anchor

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 + Affine+EIF3 |  |  |  |  | Over BMS-1.0+EIF3 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.00% | -0.08% | 102% | 108% | 0.02% | -0.07% | -0.05% | 99% | 105% |
| Class A2 | -0.01% | -0.02% | -0.17% | 104% | 116% | 0.07% | 0.15% | 0.08% | 100% | 106% |
| Class B | 0.03% | -0.03% | 0.00% | 104% | 109% | 0.10% | 0.05% | 0.03% | 100% | 105% |
| Class C | 0.18% | -0.02% | 0.00% | 106% | 107% | 0.19% | 0.05% | 0.07% | 101% | 105% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.05% | -0.02% | -0.05% | 104% | 110% | 0.10% | 0.05% | 0.04% | 100% | 105% |
| Class D | 0.29% | 0.20% | 0.17% | 106% | 108% | 0.22% | 0.11% | -0.20% | 101% | 106% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 2 Results for LDB configuration on top of VTM+BMS Affine+EIF anchor

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 + Affine+EIF3 |  |  |  |  | Over BMS-1.0+EIF3 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.08% | -0.12% | 0.09% | 112% | 106% | 0.14% | 0.14% | 0.39% | 104% | 105% |
| Class C | 0.02% | -0.63% | -0.32% | 110% | 103% | 0.33% | 0.26% | -0.08% | 104% | 102% |
| Class E | -0.27% | 1.72% | 2.25% | 116% | 104% | 0.10% | 0.97% | 1.20% | 107% | 102% |
| Overall | -0.09% | 0.17% | 0.49% | 113% | 104% | 0.19% | 0.39% | 0.43% | 105% | 103% |
| Class D | 0.69% | 0.57% | 0.05% | 110% | 102% | 0.93% | 0.71% | 1.24% | 104% | 103% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

# 16-bit friendly design of EIF

Depending on precision of motion vectors and the size of the prediction unit different numbed of bit required for calculation in fixed point arithmetic. Due to horizontal and vertical steps of bilinear interpolation the result of the first step could exceed 16-bit range. In absence of appropriate operators in HW which could perform 2D bilinear operation in one command, we should reduce precision of intermediate values so that the input for the next step has only 16 bit for storage value. Even addition of 0.5 is not necessary before right shift operation for this intermediate rounding. It was not done in CE4.1.5d(e), but evidently could be performed with negligible influence on coding performance as demonstrated on the Table 3 and Table 4. And even with some complexity reduction.

Table 3 Result for RA configuration on top of VTM+BMS Affine+EIF anchor

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 + Affine+EIF3 |  |  |  |  | Over BMS-1.0+EIF3 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.00% | -0.08% | 102% | 108% | 0.02% | -0.07% | -0.05% | 99% | 105% |
| Class A2 | -0.01% | -0.02% | -0.17% | 104% | 116% | 0.07% | 0.15% | 0.08% | 100% | 106% |
| Class B | 0.03% | -0.03% | 0.00% | 104% | 109% | 0.10% | 0.05% | 0.03% | 100% | 105% |
| Class C | 0.18% | -0.02% | 0.00% | 106% | 107% | 0.19% | 0.05% | 0.07% | 101% | 105% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.05% | -0.02% | -0.05% | 104% | 110% | 0.10% | 0.05% | 0.04% | 100% | 105% |
| Class D | 0.29% | 0.20% | 0.17% | 106% | 108% | 0.22% | 0.11% | -0.20% | 101% | 106% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 4 Result for LDB configuration on top of VTM+BMS Affine+EIF anchor

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 + Affine+EIF3 |  |  |  |  | Over BMS-1.0+EIF3 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.08% | -0.12% | 0.09% | 112% | 106% | 0.14% | 0.14% | 0.39% | 104% | 105% |
| Class C | 0.02% | -0.63% | -0.32% | 110% | 103% | 0.33% | 0.26% | -0.08% | 104% | 102% |
| Class E | -0.27% | 1.72% | 2.25% | 116% | 104% | 0.10% | 0.97% | 1.20% | 107% | 102% |
| Overall | -0.09% | 0.17% | 0.49% | 113% | 104% | 0.19% | 0.39% | 0.43% | 105% | 103% |
| Class D | 0.69% | 0.57% | 0.05% | 110% | 102% | 0.93% | 0.71% | 1.24% | 104% | 103% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

# Hardware implementation aspects

# Few words about worth case complexity of several interpolation filters which could be used for non-translational motion models, like 4x4 sub-blocks with 8-tap/4-tap filters, 1x1 pixel-wise HEVC 8-tap/4-tap filters, Affine flexing from CE4.1.2, bilinear interpolation and several proposed EIF filters with 3-tap/5-tap filters inside and based on directional interpolation approach proposed in this contribution.

|  | Mults |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Blk size, N | 1x1 8-tap Affine | 4x4 Blk Affine | WIF 5-tap | WIF 5-tap BLDir | WIF 3-tap | WIF 3-tap BLDir | HEVC Transl 8-tap | HEVC Transl 4-tap | 1x1 4-tap Affine | Affine Flexing | bilinear |
| 4 | 1152 | 480 | 576 | 432 | 108 | 86 | 480 | 176 | 320 | 960 | 48 |
| 8 | 4608 | 1920 | 1776 | 1376 | 300 | 222 | 1472 | 608 | 1280 | 2944 | 192 |
| 16 | 18432 | 7680 | 6192 | 4896 | 972 | 686 | 4992 | 2240 | 5120 | 9984 | 768 |
| 32 | 73728 | 30720 | 23088 | 18464 | 3468 | 2382 | 18176 | 8576 | 20480 | 36352 | 3072 |
| 64 | 294912 | 122880 | 89136 | 71712 | 13068 | 8846 | 69120 | 33536 | 81920 | 138240 | 12288 |

|  | 1x1 8-tap Affine | 4x4 Blk Affine | WIF 5-tap | WIF 5-tap BLDir | WIF 3-tap | WIF 3-tap BLDir | HEVC Transl 8-tap | HEVC Transl 4-tap | 1x1 4-tap Affine | Affine Flexing | bilinear |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 240% | 100% | 120% | 90% | 23% | 18% | 100% | 37% | 67% | 200% | 10% |
| 8 | 313% | 130% | 121% | 93% | 20% | 15% | 100% | 41% | 87% | 200% | 13% |
| 16 | 369% | 154% | 124% | 98% | 19% | 14% | 100% | 45% | 103% | 200% | 15% |
| 32 | 406% | 169% | 127% | 102% | 19% | 13% | 100% | 47% | 113% | 200% | 17% |
| 64 | 427% | 178% | 129% | 104% | 19% | 13% | 100% | 49% | 119% | 200% | 18% |

# It is worth nothing that bi-linear interpolation is widely used for many application and implemented in HW in many different ways [6][7][8].

# Figure 4 Block diagram for classic bilinear interpolation [8].

# Authors  of [6] and [8] asserts that for those environments the HW complexity is the following:

# a)b)

Figure 5 Resource utilization of classic bilinear interpolation a) – for XC2S200 from [6], b) – for XC4VSX55 from [8].

Our own implementation of classic bilinear interpolation demonstrates:

TSMC                                                    CLK Freq.             AREA(sq.um)                     POWER(mW)

28HPC(28nm)                                    500MHz                                640                                         0.58

16FFplus-GL(16nm)                          500MHz                                391                                         N/A

Evidently that for modern HW architecture and more specific/simplified design the complexity/resources will be much lower.

The following block diagram (Figure 6 and 7) corresponds to proposed algorithm. It is provided to demonstrate the difference between existing solutions and proposed one. The intermediate data in EIF3 is much lower range. There are no needs to switch between filters and the final latency is not more than for 4-tap filter.

Figure 6 Block-diagram of sub modules of proposed enhanced bilinear interpolation: a)-directional bilinear interpolation, b) high-pass filter.

Figure 7 Block-diagram of proposed enhanced bilinear interpolation.

# Both schemes on Figure 6 a) and b) could be reduced twice by reusing the same units in case of EIF worst case twice lower than 8-tap DCTIF filter. It could be useful in order to reduce AREA on chip.

# Conclusions

There are two aspects included in this contribution:

- Directional Bilinear Interpolation Filter (DBIF) with reduced number of operation by 1/3 in respect to bilinear in EIF

- 16-bit friendly design of EIF

Both aspect corresponds to filter design with full PU size processing which allows to avoid increasing of memory bandwidth in worst case scenario. The proposed DBIF demonstrate the same performance with low influence on complexity for the VTM configuration even in C implementation with redundant operations. The design is friendly for parallel implementation. For each aspects the following influence on coding performance is demonstrated:

- 0.05%/-0.09 of luma BD-rate in RA/LDB with not optimized implementation for the VTM configuration with BMS Affine and EIF3 in anchor (104%(Enc) 110%(Dec) in RA and 113%(Enc) 104%(Dec) in LDB). And for BMS configuration: 0.10%/0.19 of luma BD-rate in RA/LDB with not optimized implementation with EIF3 in anchor (100%(Enc) 105%(Dec) in RA and 105%(Enc) 103%(Dec) in LDB)

- 0.04/-0.08% of luma BD-rate in RA/LDB with low influence on complexity for the VTM configuration with BMS Affine and EIF3 in anchor (92%(Enc) 103%(Dec) in RA and 94%(Enc) 100%(Dec) in LDB). And for BMS configuration: 0.09%/0.18 of luma BD-rate in RA/LDB with EIF3 in anchor (96%(Enc) 104%(Dec) in RA and 96%(Enc) 101%(Dec) in LDB)

We are proposing further study of aspect a) and adoption of aspect b) jointly with one of the Enhanced Interpolation Filter (EIF3 or EIF5) into the test model.

# References

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVET-J1024, San Diego, USA, April 2018.

- J. Chen, E. Alshina, G. J. Sullivan, J-R. Ohm, “Algorithm Description of Joint Exploration Test Model 7 (JEM 7)”, JVET-G1001, July 2017.

- A. Alshin, E. Alshina, K. Choi, N. Choi, W. Choi, S. Jeong, C. Kim, J. Min, J. H. Park, M. Park, M. W. Park, Y. Piao, A. Tamse, A. Dsouza, C. Pujara, H. Chen, J. Chen, R. Chernyak, S. Esenlik, A. Filippov, S. Gao, S. Ikonin, A. Karabutov, A. M. Kotra, X. Lu, X. Ma, V. Rufitskiy, T. Solovyev, V. Stepin, M. Sychev, T. Wang, Y.-K. Wang, W. Xu, H. Yang, V. Zakharchenko, H. Zhang, Y. Zhao, Z. Zhao, J. Zhou, C. Auyeung, H. Gao, I. Krasnov, R. Mullakhmetov, B. Wang, Y. F. Wong, G. Zhulikov, A. Abbas, D. Newman, J. An, X. Chen, Y. Lin, Q. Yu, J. Zheng, “Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario” JVET-J0024, San Diego, USA, April 2018.

- H. Chen, J. Chen, R. Chernyak, S. Esenlik, A. Filippov, S. Gao, S. Ikonin, A. Karabutov, A. M. Kotra, X. Lu, X. Ma, V. Rufitskiy, T. Solovyev, V. Stepin, M. Sychev, T. Wang, Y.-K. Wang, W. Xu, H. Yang, V. Zakharchenko, H. Zhang, Y. Zhao, Z. Zhao, J. Zhou, C. Auyeung, H. Gao, I. Krasnov, R. Mullakhmetov, B. Wang, Y. F. Wong, G. Zhulikov (Huawei), A. Abbas, D. Newman (GoPro), J. An, X. Chen, Y. Lin, Q. Yu, J. Zheng (HiSilicon), A. Alshin, E. Alshina, K. Choi, N. Choi, W. Choi, S. Jeong, C. Kim, J. Min, J. Park, M. Park, M. W. Park, Y. Piao, A. Tamse, A. Dsouza, C. Pujara (Samsung), “Description of SDR, HDR and 360° video coding technology proposal by Huawei, GoPro, HiSilicon, and Samsung – general application scenario”, JVET-J0025, San Diego, USA, April 2018.

- M. Zhou, W. Wan, T. Hellman, P. Chen, O. Hugosson, D. Symes, A. Duenas, E. Chai, “Comments on Test Model Development”, JVET-J0081, San Diego, USA, April 2018.

- K. Gribbon, D. Bailey, “A novel approach to real-time bilinear interpolation,” in Proceedings of the 2nd IEEE International Workshop on Electronic Design, Test and Applications (DELTA ’04), pp. 126–131, January 2004.

- F. Kelly and A. Kokaram. Fast image interpolation for motion estimation using graphics hardware. In Proceedings of the SPIE, pages 184-194, 2004.

- Suhaib A. Fahmy, "Generalised parallel bilinear interpolation architecture for vision systems", International Conference on Reconfugurable Computing and FPGAs, pp. 331-336, 2008.

- B. Huang, “A Resampling Theory for Non-bandlimited Signals and Its Applications”, Thesis, Massey University, Wellington, New Zealand, 2008.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, April 2018.

- Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).