| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0195-v1 |
| --- | --- |

| Title: | CE4-5 related: Inter/Intra Boundary Padding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jens Brandenburg, Robert Skupin, Heiko Schwarz, Detlev Marpe, Thomas Schierl, Thomas Wiegand Einsteinufer 37 10587 Berlin, Germany | Email: | robert.skupin@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

In HEVC, reference pictures are extended by a perpendicular padding of the picture boundary samples. CE4-5 investigates three new methods for boundary padding which rely either on inter-prediction based techniques (CE4-5.2 and CE4-5.3) or intra-prediction based techniques (CE4-5.1).

This document proposes a harmonized boundary padding approach using inter-prediction based per-picture techniques in combination with an intra-prediction based per-reference technique. When the amount of motion compensated samples in the padded area of a reference block falls below a threshold, intra-prediction based boundary padding is applied.

Reported experimental results show BD-rate gains in RA and LD configurations. For VTM RA overall (Class A1 to C), -0,24% YUV-BD-rate gain at 109% combined Enc/Dec runtime are reported. For small video resolutions such as Class D, YUV-BD-rate gains of -0.61% at 125% combined Enc/Dec runtime for RA are reported.

# Introduction

In HEVC, reference pictures are extended by a perpendicular padding of the picture boundary samples. CE4-5 [1] investigates three new methods for boundary padding which use either inter-prediction based techniques (CE4-5.2 [2] and CE4-5.3 [3]) or intra-prediction based techniques (CE4-5.1 [4]).

Inter-prediction based boundary padding as proposed in CE4-5.2 and CE4-5.3 uses motion compensated prediction to extend the area of the reference picture [2][3]. The boundary extension area is divided into blocks of 4xM or Mx4 samples. Each block is filled by motion compensation using the motion information of the adjacent reference picture block. For boundary extensions blocks without associated motion information and for boundary extension areas for which motion information points outside of the reference picture, fallback perpendicular padding is carried out. The padding method in CE4-5.2 entails addition of an average residual offset to the boundary extension samples, while the padding method in CE4-5.3 supports bi-prediction of boundary extension samples.

Intra-prediction based boundary padding as proposed in CE4-5.1 uses angular intra-prediction to fill the area of a referenced block outside the reference picture [4]. The applied angular intra-prediction mode is chosen in encoder and decoder using a probing approach of decoded picture samples.

This document presents a harmonized boundary padding approach using inter-prediction and intra-prediction based boundary padding and reports experimental results.

# Proposal: Inter/Intra-based boundary padding

This document proposes an inter/intra-prediction based boundary padding, that combines per-picture inter-prediction based boundary padding with per-reference intra-prediction based boundary padding. After generation of the inter-prediction based boundary padding, for each reference block entailing boundary padding samples, the number of boundary padding samples originating from perpendicular boundary padding is evaluated. If this number exceeds a threshold (50% of boundary padding samples), intra-prediction based boundary padding is used for the reference block instead.

Modifications to the CE4-5.2 implementation r627 (CE4-5.2_mod):

- MC block size adjustment: Motion compensation is performed for the necessary amount of samples instead of max CTU size (128x128 samples). In comparison to the CE4-5.2 implementation, the decoder runtime overhead decreases from 129% to 105% measured for 49 pictures VTM random access configuration in average over classes A-E.

- Chroma cost calculation: Due to a missing index increment, the average difference of the luma residual is added to the motion compensated prediction of the chroma components, leading to a degradation of the chroma PSNR. In comparison to the CE4-5.2 implementation, the UV-PSNR performance increases from +0,34% to -0,12% measured for 49 pictures VTM random access configuration in average over class A-E.

- Sample value clipping: The final padded area samples are clipped against zero but not against max value, leading to possible encoder/decoder mismatches.

# Simulation Results

In this section, experimental results in terms of Bjøntegaard Delta (BD) bit rate savings [5] and Encoder Decoder runtimes are presented. Simulation runs are based on test sets and configurations specified in the JVET common test conditions [6]. Simulation results with VTM and BMS as reference have been generated for the following boundary padding approaches:

- CE4-5.1: intra-prediction based boundary padding

- CE4-5.2_mod: uni-directional inter-prediction based boundary padding with average residual offset based on CE4-5.2 with above modifications

- CE4-5.3: bi-directional inter-prediction based boundary padding

- CE4-5.1 + CE4-5.2_mod: proposed harmonization of CE4-5.1 and CE4-5.2 with modifications

- CE4-5.1 + CE4-5.3: proposed harmonization of CE4-5.1 and CE4-5.3

## VTM

CE4-5.1

|  | Random Access |  |  |  |  | Low Delay |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.09% | -0.06% | 102% | 100% |  |  |  |  |  |
| Class A2 | -0.07% | -0.02% | -0.03% | 101% | 99% |  |  |  |  |  |
| Class B | -0.05% | -0.07% | -0.02% | 102% | 103% | -0.07% | 0.12% | 0.11% | 105% | 104% |
| Class C | -0.09% | -0.03% | -0.16% | 104% | 102% | -0.08% | -0.20% | -0.14% | 106% | 102% |
| Class E |  |  |  |  |  | -0.05% | -0.23% | -0.45% | 101% | 101% |
| Overall | -0.07% | -0.05% | -0.07% | 102% | 101% | -0.07% | -0.07% | -0.11% | 104% | 103% |
| Class D | -0.14% | 0.03% | -0.29% | 109% | 108% | -0.44% | -1.08% | -0.85% | 117% | 111% |
| Class F (optional) | -0.02% | -0.05% | -0.06% | 101% | 100% | -0.13% | -0.22% | 0.18% | 104% | 102% |

CE4-5.2_mod

|  | Random Access |  |  |  |  | Low Delay |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.19% | -0.14% | 101% | 101% |  |  |  |  |  |
| Class A2 | -0.14% | -0.08% | -0.11% | 100% | 102% |  |  |  |  |  |
| Class B | -0.23% | -0.21% | -0.14% | 102% | 108% | -0.03% | 0.18% | 0.10% | 100% | 105% |
| Class C | -0.25% | -0.30% | -0.23% | 103% | 110% | 0.02% | -0.20% | 0.15% | 100% | 107% |
| Class E |  |  |  |  |  | -0.02% | 0.14% | -0.31% | 100% | 111% |
| Overall | -0.21% | -0.20% | -0.16% | 101% | 106% | -0.01% | 0.05% | 0.01% | 100% | 107% |
| Class D | -0.51% | -0.38% | -0.34% | 103% | 114% | -0.03% | -0.19% | 0.16% | 101% | 111% |
| Class F (optional) | -0.05% | -0.08% | -0.05% | 101% | 107% | -0.14% | 0.07% | 0.34% | 101% | 110% |

CE4-5.3

|  | Random Access |  |  |  |  | Low Delay |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.16% | -0.15% | 100% | 102% |  |  |  |  |  |
| Class A2 | -0.12% | -0.14% | -0.11% | 100% | 104% |  |  |  |  |  |
| Class B | -0.20% | -0.15% | -0.10% | 100% | 111% | 0.00% | 0.11% | 0.10% | 100% | 106% |
| Class C | -0.26% | -0.29% | -0.29% | 100% | 113% | 0.02% | 0.03% | 0.03% | 100% | 111% |
| Class E |  |  |  |  |  | -0.06% | 0.27% | -0.14% | 100% | 114% |
| Overall | -0.18% | -0.19% | -0.16% | 100% | 108% | -0.01% | 0.12% | 0.01% | 100% | 109% |
| Class D | -0.44% | -0.28% | -0.36% | 101% | 121% | -0.09% | -0.55% | -0.20% | 100% | 117% |
| Class F (optional) | -0.03% | -0.02% | -0.02% | 101% | 112% | -0.02% | -0.08% | 0.08% | 101% | 113% |

CE4-5.1 + CE4-5.2_mod

|  | Random Access |  |  |  |  | Low Delay |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.20% | -0.16% | 100% | 104% |  |  |  |  |  |
| Class A2 | -0.16% | -0.09% | -0.09% | 100% | 103% |  |  |  |  |  |
| Class B | -0.27% | -0.28% | -0.18% | 103% | 113% | -0.08% | -0.04% | 0.41% | 104% | 108% |
| Class C | -0.31% | -0.34% | -0.34% | 105% | 116% | -0.11% | -0.19% | -0.07% | 106% | 114% |
| Class E |  |  |  |  |  | -0.06% | -0.20% | -0.13% | 101% | 113% |
| Overall | -0.25% | -0.24% | -0.20% | 102% | 110% | -0.08% | -0.13% | 0.11% | 104% | 111% |
| Class D | -0.61% | -0.47% | -0.72% | 109% | 129% | -0.51% | -0.63% | -0.89% | 117% | 125% |
| Class F (optional) | -0.04% | -0.05% | -0.07% | 102% | 111% | -0.13% | -0.52% | 0.42% | 104% | 110% |

CE4-5.1 + CE4-5.3

|  | Random Access |  |  |  |  | Low Delay |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.23% | -0.18% | 101% | 104% |  |  |  |  |  |
| Class A2 | -0.16% | -0.13% | -0.15% | 101% | 103% |  |  |  |  |  |
| Class B | -0.23% | -0.20% | -0.14% | 101% | 110% | -0.09% | 0.04% | 0.18% | 105% | 109% |
| Class C | -0.31% | -0.27% | -0.33% | 102% | 113% | -0.05% | -0.25% | 0.12% | 106% | 112% |
| Class E |  |  |  |  |  | -0.04% | 0.03% | -0.22% | 101% | 114% |
| Overall | -0.22% | -0.21% | -0.20% | 101% | 108% | -0.07% | -0.06% | 0.06% | 104% | 112% |
| Class D | -0.54% | -0.36% | -0.59% | 106% | 121% | -0.49% | -1.06% | -0.89% | 118% | 128% |
| Class F (optional) | -0.04% | -0.07% | -0.09% | 101% | 110% | -0.09% | -0.28% | 0.37% | 104% | 114% |

## BMS

CE4-5.1

|  | Random Access |  |  |  |  | Low Delay |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.12% | -0.08% | 101% | 102% |  |  |  |  |  |
| Class A2 | -0.01% | -0.10% | -0.08% | 102% | 101% |  |  |  |  |  |
| Class B | -0.05% | -0.09% | -0.04% | 102% | 101% | -0.09% | -0.33% | 0.14% | 104% | 100% |
| Class C | -0.06% | -0.10% | -0.14% | 103% | 102% | -0.08% | 0.04% | 0.01% | 104% | 100% |
| Class E |  |  |  |  |  | -0.29% | 0.19% | -0.51% | 101% | 100% |
| Overall | -0.04% | -0.10% | -0.08% | 102% | 102% | -0.13% | -0.08% | -0.07% | 103% | 100% |
| Class D | -0.11% | -0.24% | -0.27% | 105% | 108% | -0.42% | -0.77% | -1.14% | 110% | 106% |
| Class F (optional) | 0.00% | 0.06% | 0.05% | 101% | 101% | 0.07% | -0.13% | 0.29% | 102% | 98% |

CE4-5.2_mod

|  | Random Access |  |  |  |  | Low Delay |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.25% | -0.15% | 100% | 101% |  |  |  |  |  |
| Class A2 | -0.15% | -0.11% | -0.11% | 101% | 102% |  |  |  |  |  |
| Class B | -0.25% | -0.21% | -0.17% | 101% | 102% | -0.03% | -0.26% | 0.25% | 101% | 101% |
| Class C | -0.18% | -0.17% | -0.13% | 101% | 105% | -0.03% | 0.04% | 0.13% | 101% | 102% |
| Class E |  |  |  |  |  | -0.27% | -0.27% | 0.02% | 99% | 103% |
| Overall | -0.20% | -0.19% | -0.14% | 101% | 103% | -0.09% | -0.16% | 0.15% | 101% | 102% |
| Class D | -0.44% | -0.47% | -0.44% | 102% | 108% | -0.06% | 0.19% | -0.54% | 99% | 102% |
| Class F (optional) | -0.01% | -0.01% | -0.01% | 100% | 104% | 0.05% | 0.20% | 0.52% | 101% | 103% |

CE4-5.3

|  | Random Access |  |  |  |  | Low Delay |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.22% | -0.11% | 101% | 102% |  |  |  |  |  |
| Class A2 | -0.13% | -0.17% | -0.09% | 101% | 102% |  |  |  |  |  |
| Class B | -0.23% | -0.15% | -0.18% | 101% | 102% | -0.02% | -0.19% | 0.17% | 101% | 101% |
| Class C | -0.20% | -0.16% | -0.27% | 101% | 105% | 0.00% | 0.16% | 0.19% | 101% | 103% |
| Class E |  |  |  |  |  | -0.24% | -0.50% | -0.41% | 101% | 105% |
| Overall | -0.18% | -0.17% | -0.17% | 101% | 103% | -0.07% | -0.15% | 0.03% | 101% | 103% |
| Class D | -0.39% | -0.30% | -0.23% | 101% | 111% | -0.05% | 0.03% | -0.35% | 100% | 108% |
| Class F (optional) | -0.01% | 0.00% | -0.02% | 100% | 106% | 0.05% | 0.01% | 0.68% | 101% | 105% |

CE4-5.1 + CE4-5.2_mod

|  | Random Access |  |  |  |  | Low Delay |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -0.33% | -0.20% | 101% | 102% |  |  |  |  |  |
| Class A2 | -0.16% | -0.16% | -0.20% | 101% | 103% |  |  |  |  |  |
| Class B | -0.31% | -0.23% | -0.24% | 102% | 104% | -0.09% | -0.30% | 0.14% | 103% | 101% |
| Class C | -0.27% | -0.24% | -0.25% | 102% | 107% | -0.12% | 0.14% | -0.22% | 106% | 108% |
| Class E |  |  |  |  |  | -0.27% | -0.50% | -0.67% | 101% | 107% |
| Overall | -0.25% | -0.24% | -0.23% | 102% | 104% | -0.15% | -0.20% | -0.18% | 104% | 105% |
| Class D | -0.58% | -0.56% | -0.57% | 104% | 116% | -0.51% | -0.79% | -1.44% | 110% | 113% |
| Class F (optional) | -0.02% | -0.01% | -0.06% | 101% | 106% | 0.09% | -0.22% | 0.14% | 103% | 105% |

CE4-5.1 + CE4-5.3

|  | Random Access |  |  |  |  | Low Delay |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.20% | -0.14% | 100% | 102% |  |  |  |  |  |
| Class A2 | -0.15% | -0.22% | -0.18% | 101% | 103% |  |  |  |  |  |
| Class B | -0.24% | -0.26% | -0.16% | 101% | 103% | -0.08% | -0.44% | 0.11% | 104% | 102% |
| Class C | -0.24% | -0.21% | -0.33% | 102% | 106% | -0.12% | -0.21% | -0.20% | 104% | 105% |
| Class E |  |  |  |  |  | -0.22% | -0.02% | 0.07% | 101% | 107% |
| Overall | -0.20% | -0.23% | -0.21% | 101% | 104% | -0.13% | -0.26% | 0.00% | 103% | 104% |
| Class D | -0.51% | -0.56% | -0.61% | 104% | 113% | -0.44% | -0.83% | -0.71% | 109% | 111% |
| Class F (optional) | -0.02% | 0.03% | -0.02% | 101% | 105% | -0.03% | -0.08% | 0.47% | 102% | 104% |

# References

- H. Yang, S. Liu and K. Zhang, “Description of CE4: Inter prediction and motion vector coding“ JVET, doc JVET-J1024, April 2018.

- Y.-W. Chen, W.-J. Chien, H.-C. Chuang, M. Coban, J. Dong, H. E. Egilmez, N. Hu, M. Karczewicz, A. Ramasubramonian, D. Rusanovskyy, A. Said, V. Seregin, G. Van Der Auwera, K. Zhang, L. Zhang, P. Bordes, Y. Chen, C. Chevance, E. François, F. Galpin, M. Kerdranvat, F. Hiron, P. de Lagrange, F. Le Léannec, K. Naser, T. Poirier, F. Racapé, G. Rath, A. Robert, F. Urban and T. Viellard, “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – low and high complexity versions,“ JVET, doc JVET-J0021, April 2018.

- H. Chen, J. Chen, R. Chernyak, S. Esenlik, A. Filippov, S. Gao, S. Ikonin, A. Karabutov, A. M. Kotra, X. Lu, X. Ma, V. Rufitskiy, T. Solovyev, V. Stepin, M. Sychev, T. Wang, Y.-K. Wang, W. Xu, H. Yang, V. Zakharchenko, H. Zhang, Y. Zhao, Z. Zhao, J. Zhou, C. Auyeung, H. Gao, I. Krasnov, R. Mullakhmetov, B. Wang, Y. F. Wong, G. Zhulikov, A. Abbas, D. Newman, J. An, X. Chen, Y. Lin, Q. Yu, J. Zheng, A. Alshin, E. Alshina, K. Choi, N. Choi, W. Choi, S. Jeong, C. Kim, J. Min, J. Park, M. Park, M. W. Park, Y. Piao, A. Tamse, A. Dsouza and C. Pujara, “Description of SDR, HDR and 360° video coding technology proposal by Huawei, GoPro, HiSilicon, and Samsung – general application scenario,” JVET, doc. JVET- J0025, April 2018.

- M. Albrecht, C. Bartnik, S. Bosse, J. Brandenburg, B. Bross, J. Erfurt, V. George, P. Haase, P. Helle, C. Helmrich, A. Henkel, T. Hinz, S. de Luxan Hernandez, S. Kaltenstadler, P. Keydel, H. Kirchhoffer, C. Lehmann, W.-Q. Lim, J. Ma, D. Maniry, D. Marpe, P. Merkle, T. Nguyen, J. Pfaff, J. Rasch, R. Rischke, C. Rudat, M. Schaefer, T. Schierl, H. Schwarz, M. Siekmann, R. Skupin, B. Stallenberger, J. Stegemann, K. Suehring, G. Tech, G. Venugopal, S. Walter, A. Wieckowski, T. Wiegand and M. Winken, “Description of SDR, HDR and 360° video coding technology proposal by Fraunhofer HHI,“ JVET, doc. JVET-J0014, April 2018.

- G. Bjøntegaard, “Calculation of average PSNR differences between RD-curves,” VCEG-M33, Austin, Mar. 2001.

- J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations,” JVET, doc. JVET-J1010, April 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).