| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0196-v2 |
| --- | --- |

| Title: | CE3-related: Non-linear weighted intra prediction (cross-check report in JVET-K0262) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philipp Helle, Tobias Hinz, Roman Rischke, Jonathan Pfaff, Philipp Merkle, Michael Schäfer, Björn Stallenberger, Valeri George, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-188, philipp.helle@hhi.fraunhofer.de, jonathan.pfaff@hhi.fraunhofer.de |
| Source: | Fraunhofer-HHI |  |  |

_____________________________

# Abstract

In this document, non-linear intra-picture-predictors are proposed which predict the samples in rectangular blocks by using neighbouring decoded samples. The prediction uses two steps both of which consist of a linear map and the addition of an offset vector. Between the two steps, a non-linear rectifier function is applied to each component of the intermediate result. The output of the second step predicts the picture-samples whereby the prediction is calculated on a subset of all components in the frequency domain.

The predictors use multiple modes, one of which is signalled to the decoder for each block to be predicted. All modes share the first step of the prediction process, but have individual second steps. For the purpose of entropy coding the prediction-mode, a similar two-step process is used to predict the most probable modes from neighbouring decoded samples.

The proposed technique is a further development of the technique proposed in JVET-J0037. It uses fewer parameters and fewer arithmetic operations than the technique proposed in JVET-J0037.

For the proposed technique, BD-Rates of -3.42%, -1.66%, -1.67% (Y-, U- and V-component) are reported for the VTM AI configuration and BD-Rates of -1.67%, 0.50%, 0.42% are reported for the VTM RA configuration. BD-Rates of -2.54%, -2.50%, -2.55% are reported for the BMS AI configuration and BD-Rates of -1.33%, -1.41%, -1.64% are reported for the BMS RA configuration.

A cross-check of this proposal was performed and the report is accessible in the document JVET-K0262.

# Description

## Picture-sample prediction

The picture sample prediction proposed in the current document modifies the prediction process proposed in JVET-J0037 [1], sections 1.1.1 and 1.1.6 in the following two ways. First, only one intermediate result instead of three intermediate results needs to be computed in order to generate a prediction signal in the frequency domain. Second, to obtain the output of the intermediate result, a rectified linear unit is used instead of the exponential linear unit.

To predict the samples in a rectangular block of size MxN, the proposed predictors take as their input the reconstructed samples (reference samples) r in a neighbourhood of two sample lines left and above of the current block, see Fig. 1. These reconstructed samples form a vector of dimension d0=2(M+N+2).

Figure 1 Prediction of MxN samples with the proposed technique

For a fixed matrix A1 that has d1 rows and d0 columns and a fixed offset vector b1 of dimension d1, one first computes

t1=Clip1(σ(A1∙r+b1 )).

Here, ∙ denotes the ordinary matrix-vector product. Moreover, the non-linear rectifier function σ is defined as

σvi=vi, if vi>00, else,

where σvi and vi denote the i-th component of the vector. Finally, Clip1 denotes a clipping operation to a fixed range.

The dimension d1 depends on M and N as follows: d1=d0 if maxM,N<32 and d1=M+N+1 otherwise. For a fixed matrix A2,k that has d2=MN rows and d1 columns and for a fixed offset vector b2,k of dimension d2, one computes the vector

pred=Clip2A2,k∙t1+b2,k

k=predmode

Here, predmode identifies the prediction mode and Clip2 again denotes a clipping operation to a fixed range. The vector pred is regarded as a prediction signal in the frequency domain. Thus, the overall predicted samples are obtained by applying the inverse DCT to the vector pred. The overall process of generating a prediction signal is depicted in Figure 1 above.

In the same way as described in [1], section 1.1.6, the vector pred is constrained to a fixed sparsity pattern that depends only on predmode but not on the input vector r. This means that for some frequency components, the vector pred is constrained to be zero in the corresponding component. For each such frequency component, the row of the matrix A2,k corresponding to that component consists only of zeros and no multiplications need to be carried out in the matrix vector product A2,k∙t1 for that row. The latter effect is illustrated by the fact that for example on square blocks of size 32x32, it is reported that at least 923 and up to 1005 rows of the 1024 rows of the matrices A2,k are identically zero for the 11 proposed intra prediction modes.

## Signalization of the specific proposed intra mode

In the present document, n different intra prediction modes are proposed, where n is set to 35 for maxM,N<32 and to 11 else. Thus, an index predmode with 0≤predmode<n is to be transmitted in the bit-stream. It is proposed to do this in a similar way as described in [1], section 1.1.2 but using the rectified linear unit function σ as a nonlinear function. 
Thus, it is proposed that first an index predIdx with 0≤predIdx<n is signaled as described in [1], Table 1. From the index predIdx, the actual prediction mode index predmode is derived as follows. One takes the reconstructed samples r from the previous section as input. There is a fixed square-matrix A1' which has d0 rows resp. columns and there is a fixed offset vector b1'of dimension d0 such that one computes

t1'=σ(A1'∙r+b1').

Next, there exists a matrix A2' which has n rows and d0 columns and there is a fixed offset vector b2' of dimension n such that one computes

lgt=A2'∙t1'+b2' .

The index predmode is now derived as being the position of the predIdx-th largest component of lgt.

## Extension to larger blocks

For blocks of size 64x64, the proposed intra prediction modes for blocks of size 32x32, described in section 1.1, are reused by applying the method of subsampling the input and upsampling the output that is described in [1], section 1.1.3. Also, for blocks of size 32x32, the proposed intra prediction modes for blocks of size 16x16 can be used additionally using the latter method.

## Symmetries between proposed prediction modes

Given a prediction mode on an MxN block as described in section 1.1, it is proposed to generate a transposed prediction signal on an NxM block as follows. First, the input vector r is rearranged to a vector rtr  by exchanging the x- and y-coordinates in the two-dimensional plane. Next, it is proposed to generate the vector predtr out of the vector rtr  by the given prediction mode for the MxN block. Then, it is proposed to rearrange predtr by exchanging again the x- and y-coordinates to obtain a vector pred.  Finally, it is proposed to apply the NxM-Inverse Discrete Cosine Transform to pred to obtain a prediction signal on the given NxM block.
It is proposed that for M>N, all prediction modes for a block of size NxM are the transposed prediction modes of the proposed prediction modes for a block of size MxN. Also, it is proposed that the generation of the vector lgt, described in section 1.2, is performed by using the matrices A1', A2' and the offset vectors  b1', b2' for the blocksize MxN as well as the vector rtr as input.  
Finally, for square blocks with n=2j+1 proposed prediction modes, it is proposed that for 0<k≤j, the (j+k)-th proposed prediction mode is the transposed of the k-th proposed prediction mode. For k=0, there is no transposed counterpart.

## Parameters needed to generate the prediction signals

According to the description in section 1.1 and 1.2, fixed parameters that represent the matrices and offset vectors are needed to generate the proposed prediction signals. These parameters, namely the matrices A1,  A1', A2' , the non-zero rows of the matrices A2,k and the offset vectors b1, b2,k, b1', b2' are represented by 16bit fixed point numbers and can therefore be stored in 16 bit per element. These parameters are needed for each block-size MxN with M and N being equal to 4, 8, 16 or 32 and with M≤N. The latter constraint is due to the transposed symmetries between blocks of size MxN and NxM described in the previous section. Thus, parameters are needed for 10 different block-sizes. For each such block-size MxN, the full set of needed parameters can be described as follows. First, one matrix A1 with d1 rows and d0 columns as well as one vector b1 of size d1  are needed. It is pointed out that for a fixed block-size, the matrix A1 and the offset vector b1  are equal for all different prediction modes. Second, m matrices  A2,k, each of which have d2  rows and d1 columns as well as m vectors b2,k , each of which is of size d2 are  needed. Here, according to section 1.2 and section 1.4, the number m depends on M and N as follows. If M<N and N<32, one has m=35. If M<N and N=32, one has m=11. If M=N and N<32, one has m=18. Finally, if M=N and N=32, one has m=6.

According to section 1.1, there are rows of the matrices A2,k equal to zero. Thus, for each 0≤k<m, it is proposed to provide a mask that indicates which rows of the matrix A2,k are non-zero. Finally, according to section 1.2, a square-matrix A1' which has d0 rows resp. columns, an offset vector b1' of size d0, a matrix A2' which has n rows and d0 columns and an offset vector b2' of size n are needed. Here one has n=35 for maxM,N<32 and n=11, else.

## Integration into the BMS-1.0 codec

The proposed intra prediction modes are integrated in the underlying reference software in a similar way as described in [1], sections 1.1.4, 1.1.5. 
Thus, at the parsing stage, the following two steps are proposed to be carried out by a decoder. First, for each luma Coding Unit (CU) in intra mode, a flag is parsed that determines whether a proposed intra prediction mode is to be applied for the present CU or not. If the present CU is of size 32x32 and if a proposed intra mode is to be applied, another flag is parsed that determines whether the proposed intra mode is to be generated using the method described in section 1.3. Second, if a proposed intra mode is to be applied, an index predIdx is parsed according to section 1.2.

At the reconstruction stage, it is proposed that the decoder carries out the following two steps. First, the logits lgt are computed as described in section 1.2 resp. 1.3. Using the index predIdx, the index predmode  and thus the specific proposed prediction mode is determined according to section 1.2. Second, the prediction signal corresponding to predmode is obtained as described in section 1.1. resp. 1.3.

The proposed intra prediction modes are applied to the luma component only. If the chroma components are to be predicted in DM-chroma mode on a given CU and if the luma prediction signal is generated by the proposed modes, the chroma prediction signals are given by the planar prediction signals. If on a CU or on a TU the conventional intra prediction (DC-, planar- or angular-prediction) is to be performed at the luma component and the list of most probable modes is to be generated, all neighboring, already reconstructed CUs in intra mode that have the proposed intra prediction modes are regarded as having the planar prediction mode

If, besides the DCT, additional mode-dependent primary transforms are present for the generation of the intra-prediction residuals corresponding to the conventional intra-prediction modes, for each proposed intra prediction mode the primary transforms of the planar prediction mode are proposed as additional primary transforms. No secondary transforms for the proposed intra prediction modes are proposed in the current document, regardless of whether they are used for the conventional intra prediction modes or not.

At the intra search for the encoder, a Hadamard-transform-based fast search among all proposed intra modes followed by a full RD search on a smaller set is proposed to select the best proposed intra mode for a given block. The latter mode is proposed to be compared to the best conventional intra mode found by the underlying intra search and it is proposed to select the overall best result as the best intra configuration for a current block.

# Results

According to the common test conditions in JVET-J1010 [2], the proposed method is evaluated for the intra-only (AI) and random-access (RA) configurations with the BMS-1.0 software in VTM and BMS configuration. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and GCC 7.2.1 compiler.

Table 1. Result for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -3.51% | -2.33% | -2.05% | 278% | 126% |
| Class A2 | -2.68% | -1.51% | -1.74% | 271% | 131% |
| Class B | -3.11% | -1.22% | -0.88% | 274% | 138% |
| Class C | -2.66% | -1.24% | -1.50% | 267% | 136% |
| Class E | -5.57% | -2.42% | -2.77% | 272% | 141% |
| Overall | -3.42% | -1.66% | -1.67% | 272% | 135% |
| Class D | -2.82% | -1.22% | -1.24% | 264% | 143% |
| Class F (optional) |  |  |  |  |  |

Table 2. Result for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1.75% | -0.34% | -0.35% | 124% | 104% |
| Class A2 | -1.43% | 1.02% | 0.77% | 119% | 103% |
| Class B | -1.83% | 1.06% | 1.12% | 123% | 109% |
| Class C | -1.61% | 0.04% | -0.15% | 125% | 110% |
| Class E |  |  |  |  |  |
| Overall | -1.67% | 0.50% | 0.42% | 123% | 107% |
| Class D | -1.39% | 0.60% | -0.11% | 125% | 106% |
| Class F (optional) |  |  |  |  |  |

Table 3. Result for BMS AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -3.40% | -3.96% | -4.02% | 149% | 116% |
| Class A2 | -2.22% | -2.43% | -1.93% | 146% | 117% |
| Class B | -2.29% | -2.16% | -2.65% | 147% | 117% |
| Class C | -1.60% | -1.78% | -1.83% | 140% | 117% |
| Class E | -3.68% | -2.63% | -2.47% | 151% | 118% |
| Overall | -2.54% | -2.50% | -2.55% | 146% | 117% |
| Class D | -1.46% | -1.60% | -1.54% | 136% | 120% |
| Class F (optional) |  |  |  |  |  |

Table 4. Result for BMS RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1,92% | -2,45% | -2,95% | 114% | 106% |
| Class A2 | -1,09% | -1,22% | -0,33% | 115% | 102% |
| Class B | -1,40% | -1,01% | -1,55% | 116% | 104% |
| Class C | -0,98% | -1,27% | -1,73% | 116% | 103% |
| Class E |  |  |  |  |  |
| Overall | -1,33% | -1,41% | -1,64% | 116% | 104% |
| Class D | -0,80% | -1,12% | -1,09% | 116% | 104% |
| Class F (optional) |  |  |  |  |  |

# Cross check

A cross-check of this proposal was performed by Huawei [3].

# Conclusion

The results show that the proposed intra prediction modes lead to coding gains for all configurations.

# References

- J. Pfaff et al., “Intra Prediction Modes based on Neural Networks,” JVET-J0037, San Diego, USA, April 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, April 2018.

- S. Ikonin, J. Chen, “Cross-check report on JVET-K0196 CE3-related: Non-linear weighted intra prediction”, Ljubljana, SI, July 2018

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).