| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K00XX |
| --- | --- |

| Title: | CE 4: Enhanced Merge Mode (Test 4.2.15) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xu Chen Na Zhang Jianhua Zheng | Tel: Email: | +86 13480990067 anderson.chen@hisilicon.com zhengjianhua@hisilicon.com |
| Source: | HiSilicon Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution presents an enhanced merge mode based on VTM-1.0 and BMS-1.0. Firstly, the extended spatial merge candidates are added into the merge candidates list, followed by more MV offsets added to the first merge candidate, then the combined average merge candidates are generated to replace the original combined bi-predictive candidates. The proposed technologies can provide 1.42%, 0.90%, 1.01% gain for RA, LB, LP respectively compared to VTM-1.0 anchor with around 34% encoding time and 2% decoding time increase, and can provide X.X%, X.X%, X% gain for RA,LB, LP respectively compared to BMS-1.0 anchor.

Introduction

In current VTM (BMS), the set of possible candidates in the merge mode consists of spatial neighbour candidates, ATMVP candidates, a temporal candidate, and generated candidates. Figure 1 shows the positions of five spatial candidates. For each candidate position, the availability is checked according to the numerical order {1, 2, 3, 4, 5}.

Figure 1. Positions of spatial candidates in HEVC.

Proposed improvements

Extended Spatial Merge Candidates (test A)

In this proposed method, more spatial positions are checked as shown in Figure 2. The extended spatial positions from 6 to 27 are checked according to their numerical order after the temporal candidate. In order to save the MV line buffer, all the spatial candidates are restricted within two CTU lines. That is, the spatial candidates beyond the CTU line above the current CTU line are excluded.

Figure 2. Positions of extended spatial candidates in the proposed method.

The number of candidates within the merge candidate list is controlled by NumMrgCands. In HEVC, NumMrgCands=5. In BMS, the NumMrgCands is added by 2 when ATMVP is turned on. In this proposed method, the NumMrgCands is added by 6, so the NumMrgCands equals to 11 when ATMVP is off and 13 when ATMVP is on.

The whole process of adding candidates will stop as soon as the number of candidates reaches NumMrgCands during the merge candidate list construction process. The redundancy check is done for all the merge candidates except the generated ones. That is, only unique motion candidate from spatial positions, ATMVP, and temporal candidate, can be included the merge candidate list. The merge candidate list is constructed as follows:

- Add the spatial positions from position 1 to 5, ATMVP candidates, and temporal candidate, which is the same as in current JEM; (include redundancy check)

- Add the extended spatial positions from position 6 to 27; (include redundancy check)

Merge offset extension (test B)

In current BMS, the set of possible candidates in the merge mode consists of spatial neighbour candidates, ATMVP candidates, a temporal candidate, and generated candidates.

In Merge offset extension, more candidates are checked based on existed candidate. As Figure 3 shows, new candidates has mv offset to the mv of existed candidate (except ATMVP candidate).

Figure 3.  New MV candidate

The number of candidates within the merge candidate list is controlled by NumMrgCands. In HEVC, NumMrgCands=5. In JEM, the NumMrgCands is added by 2 when ATMVP is turned on. For extended spatial merge candidates, the NumMrgCands is added by 6, so the NumMrgCands equals to 11 when ATMVP is off and 13 when ATMVP is on. For merge offset extension, 8 new MV candidates with one luma sample offset, so the NumMrgCands is added by 8. The NumMrgCands equals to 19 when ATMVP is off and 21 when ATMVP is on (set Extended spatial merge candidates on). The NumMrgCands equals to 13 when ATMVP is off and 15 when ATMVP is on (set Extended spatial merge candidates off).

Figure 4 New merge candidate list construction

As Figure 4 shows, new extended mv offset candidates are constructed only based on the first candidate of merge candidate list. New extended mv offset candidates only has mv offset to the current mv of first candidate and other prediction information is same to first candidate.

In the proposed method, 8 new MV candidates include MV' surrounding candidates with one luma sample offset to the first candidate MV in either the horizontal or vertical direction, or both. Finally, the best refined MV' are used for generating the final prediction results. For bi-prediction with one MV from a reference picture in the past and another from a reference picture in the future case, add the mv offset to first candidate MV0, and add the reverse mv offset to first candidate MV1. Other bi-prediction cases add same mv offset to first candidate MV0 and MV1 respectively.

Combined Average Merge Candidates (test C)

Combined bi-predictive merge candidates in current VTM-1.0 and BMS-1.0 are created by combining original merge candidates. They are added following original merge candidates to decrease coding loss by using fixed list size. However, it is invalid for the P slice. Combined bi-predictive merge candidates are replaced by the proposed combined average merge candidates in random access configuration and low delay P configuration for the higher coding efficiency.

Combined average merge candidates are generated as follows. The motion information of the original merge candidates for each list is scaled to the derived reference picture for a given list. The scaled motion information is stored in a new array. The derived reference picture for a given list is the reference picture with the highest utilization rate among the reference pictures of the original merge candidates in that list. After scaling the motion information, all available motion vectors (up to 2) of the combined merge candidates are averaged separately for each reference list. The averaged motion vector is assigned as the motion vector of the combined average merge candidate. The derived reference picture for a given list is the reference picture of all the combined average merge candidates in that list.

In Figure 5, three combined average merge candidates are added into the merge candidate list.

| Merge_idx | L0 | L1 |
| --- | --- | --- |
| 0 | mvL0_A, ref0 | mvL1_A, ref1 |
| 1 | mvL0_B, ref0 | mvL1_B, ref1 |
| 2 | - | mvL1_C, ref1 |

(a)Original merge candidate list

| Merge_idx | L0 | L1 |
| --- | --- | --- |
| 0 | mvL0_A, ref0 | mvL1_A, ref1 |
| 1 | mvL0_B, ref0 | mvL1_B, ref1 |
| 2 | - | mvL1_C, ref1 |
| 3 | ( mvL0_A + mvL0_B)/2, ref0 | ( mvL1_A + mvL1_B)/2, ref1 |
| 4 | mvL0_A, ref0 | ( mvL1_A + mvL1_C)/2, ref1 |
| 5 | mvL0_B, ref0 | ( mvL1_B + mvL1_C)/2, ref1 |

(b)Merge Candidate List after adding combined average candidates

Figure 5．An example for combined average merge candidates

The combined average merge candidates have six combinations at most. The six combinations are described as follows:

uiPriority0[6] = {0 , 0,  1,  0,  1,  2};

uiPriority1[6] = {1 , 2,  2,  3,  3,  3};

Experimental Results

The proposed method has been implemented on BMS-1.0[1], and all the tests were conducted under the common test conditions [2]. The experimental results of the combined average merge candidates compared to VTM-1.0 and BMS-1.0 are shown in Table1-4.

Table 1. Experimental results of Extended Spatial Merge Candidates (test A)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.95% | -0.98% | -1.16% | 106% | 100% |  |  |  |  |  |
| Class A2 | -1.14% | -1.43% | -1.51% | 108% | 102% |  |  |  |  |  |
| Class B | -1.44% | -1.12% | -1.06% | 109% | 101% |  |  |  |  |  |
| Class C | -0.92% | -1.00% | -1.08% | 111% | 100% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.14% | -1.12% | -1.17% | 109% | 101% |  |  |  |  |  |
| Class D | -0.70% | -0.69% | -0.87% | 114% | 100% |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.23% | -0.65% | -0.94% | 111% | 102% |  |  |  |  |  |
| Class C | -0.61% | -0.83% | -0.74% | 112% | 105% |  |  |  |  |  |
| Class E | -0.51% | -0.40% | -0.26% | 110% | 105% |  |  |  |  |  |
| Overall | -0.84% | -0.65% | -0.70% | 111% | 104% |  |  |  |  |  |
| Class D | -0.56% | -0.23% | -0.47% | 112% | 108% |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.74% | -1.17% | -1.41% | 116% | 103% |  |  |  |  |  |
| Class C | -1.22% | -1.42% | -1.27% | 116% | 104% |  |  |  |  |  |
| Class E | -1.15% | -1.21% | -0.36% | 116% | 105% |  |  |  |  |  |
| Overall | -1.42% | -1.26% | -1.10% | 116% | 104% |  |  |  |  |  |
| Class D | -1.10% | -1.37% | -0.57% | 116% | 105% |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 2. Experimental results of Merge offset extension (test B)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.13% | -0.85% | -1.02% | 127% | 102% | -0.49% | -0.30% | -0.56% | 117% | 101% |
| Class A2 | -1.48% | -1.85% | -1.94% | 126% | 101% | -0.70% | -1.00% | -1.12% | 117% | 102% |
| Class B | -1.11% | -1.05% | -0.98% | 127% | 102% | -0.65% | -0.72% | -0.75% | 116% | 101% |
| Class C | -0.54% | -0.76% | -0.87% | 126% | 101% | -0.46% | -0.67% | -0.82% | 116% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.04% | -1.09% | -1.15% | 126% | 101% | -0.58% | -0.68% | -0.80% | 116% | 101% |
| Class D | -0.48% | -0.75% | -0.92% | 126% | 98% | -0.46% | -0.58% | -0.73% | 115% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.53% | -0.52% | -0.60% | 129% | 103% | -0.39% | -0.76% | -0.39% | 113% | 102% |
| Class C | -0.42% | -0.74% | -0.92% | 124% | 101% | -0.40% | -0.48% | -0.81% | 110% | 100% |
| Class E | -0.25% | -0.09% | -0.56% | 120% | 102% | -0.33% | -0.47% | -0.17% | 107% | 100% |
| Overall | -0.42% | -0.48% | -0.70% | 125% | 102% | -0.38% | -0.60% | -0.48% | 110% | 101% |
| Class D | -0.34% | -0.35% | -0.76% | 121% | 103% | -0.30% | -0.16% | -1.18% | 109% | 102% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.38% | -0.37% | -0.76% | 137% | 101% | -0.21% | -0.33% | -0.57% | 115% | 101% |
| Class C | -0.35% | -0.54% | -0.96% | 131% | 100% | -0.13% | -0.33% | -0.52% | 110% | 101% |
| Class E | -0.43% | -0.47% | -0.39% | 128% | 101% | -0.45% | 0.09% | -1.39% | 110% | 101% |
| Overall | -0.38% | -0.45% | -0.73% | 133% | 100% | -0.24% | -0.22% | -0.76% | 112% | 101% |
| Class D | -0.27% | -0.55% | -0.15% | 127% | 102% | -0.21% | 0.04% | -1.02% | 110% | 102% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 3. Experimental results of the combined average merge candidates (test C)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.19% | -0.25% | 103% | 101% | -0.23% | -0.09% | -0.14% | 103% | 100% |
| Class A2 | -0.51% | -0.46% | -0.46% | 102% | 100% | -0.33% | -0.18% | -0.22% | 102% | 100% |
| Class B | -0.34% | -0.20% | -0.14% | 101% | 100% | -0.29% | -0.13% | -0.11% | 102% | 100% |
| Class C | -0.10% | -0.09% | -0.19% | 102% | 100% | -0.15% | -0.09% | -0.12% | 102% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.31% | -0.22% | -0.24% | 102% | 100% | -0.25% | -0.12% | -0.14% | 102% | 100% |
| Class D | -0.26% | -0.09% | -0.23% | 101% | 100% | -0.23% | -0.23% | -0.31% | 102% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 102% | 99% | 0.00% | 0.00% | 0.00% | 102% | 93% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 99% | 0.00% | 0.00% | 0.00% | 102% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 104% | 0.00% | 0.00% | 0.00% | 102% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 100% | 0.00% | 0.00% | 0.00% | 102% | 96% |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 101% | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.47% | 0.00% | 0.17% | 105% | 99% | -0.38% | 0.20% | 0.15% | 104% | 93% |
| Class C | -0.39% | -0.16% | -0.43% | 105% | 98% | -0.42% | 0.10% | -0.24% | 104% | 97% |
| Class E | -0.31% | -0.34% | 0.27% | 103% | 100% | -0.24% | 1.35% | 0.11% | 103% | 94% |
| Overall | -0.41% | -0.14% | -0.01% | 105% | 99% | -0.36% | 0.45% | 0.01% | 104% | 95% |
| Class D | -0.40% | -0.13% | -0.25% | 105% | 99% | -0.50% | -0.29% | 0.07% | 105% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 4. Experimental results of Enhanced merge mode (Performance of A+B+C)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.38% | -1.25% | -1.50% | 134% | 101% |  |  |  |  |  |
| Class A2 | -1.38% | -1.92% | -2.00% | 131% | 103% |  |  |  |  |  |
| Class B | -1.80% | -1.66% | -1.50% | 134% | 102% |  |  |  |  |  |
| Class C | -0.99% | -1.17% | -1.35% | 137% | 102% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.42% | -1.50% | -1.56% | 134% | 102% |  |  |  |  |  |
| Class D | -0.70% | -0.75% | -1.01% | 139% | 102% |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.37% | -0.99% | -0.63% | 141% | 103% |  |  |  |  |  |
| Class C | -0.74% | -0.79% | -0.95% | 138% | 107% |  |  |  |  |  |
| Class E | -0.35% | -0.67% | -0.78% | 135% | 109% |  |  |  |  |  |
| Overall | -0.90% | -0.84% | -0.77% | 138% | 106% |  |  |  |  |  |
| Class D | -0.52% | -0.61% | -0.95% | 135% | 109% |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.78% | -1.45% | -1.59% | 150% | 104% |  |  |  |  |  |
| Class C | -1.27% | -1.47% | -1.67% | 146% | 106% |  |  |  |  |  |
| Class E | -1.03% | -0.86% | -0.85% | 148% | 106% |  |  |  |  |  |
| Overall | -1.43% | -1.31% | -1.43% | 148% | 105% |  |  |  |  |  |
| Class D | -1.01% | -0.85% | -0.95% | 143% | 108% |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table1-3 show the separate gain for extended spatial merge candidates, merge offset extension and combined average merge candidates separately. Table 4 shows the performance of enhanced merge mode.

Reference

- BMS-1.0, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0

- J.Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, US, 10–20 Apr. 2018.

Patent rights declaration(s)

HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).