| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18July 2018 | Document: JVET-K0199 |
| --- | --- |

| Title: | CE9:Simplification of DMVR (Test 9.1.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xu Chen, Semih Esenlik, Jianhua Zheng | Tel: Email: | +86-13480990067 anderson.chen@hisilicon.com |
| Source: | HiSilicon Technologies, Huawei Technologies |  |  |

_____________________________

- Abstract

Asimplified decoder-side motion vector refinement algorithm is proposed. To make DMVR more familiar with hardware design and realization, we simplified DMVR by removing motion vectors update process. Compared to VTM-1.0, an average bit-rate savings of 1.46% gain for RA with 6% encoding time increase and 16% decoding time increase is achieved and about 0.59% BD-rate lost with 1% encoding time and 5% decoding time s is achieved over BMS-1.0 configuration.

Introduction

In current DMVR,a bilateral template is generated as the weighted combination (i.e. average) of the two prediction blocks, from the initial MV0 of list0 and MV1 of list1, respectively, as shown in Figure1. The template matching operation consists of calculating cost measures between the generated template and the sample region (around the initial prediction block) in the reference picture. For each of the two reference pictures, the MV that yields the minimum template cost is considered as the updated MV of that list to replace the original one.

The two new MVs, i.e., MV0′ and MV1′ as shown in Figure1, are used for generating new bi-prediction results. A sum of absolute differences (SAD) is used as the cost measure. Optionally, multi-iteration for the new MVs derivation is available to get the final bi-prediction results.

Figure1: DMVR based on bilateral template matching

Proposed DMVR

The DMVR method follows the block merging operation and modifies input motion vectors in order to improve the accuracy of the initial motion compensated prediction. The process is invoked if the following conditions are true:

- Current block is bi-predicted,

- One reference picture proceeds and other reference picture precedes the current picture in display order,

- Prediction mode is merge mode,

- AFFINE are signaled to be off.

The proposed DMVR is still only apply on integer pel motion vector refinement. The flowchart provides a high level overview of the operation:

Figure 3-32 Flowchart of DMVR process

Interpolation filtering:

During the MV search process the motion compensated predictions are obtained using the bilinear interpolation filtering. After the DMVR process finishes, the final predictions are obtained using regular interpolation filters.

Construction of motion vector refinement search space:

Integer search space is composed of 6 search points, 1 center point and 5 surrounding points, in total. In the first step 5 points (CENTER, UP, LEFT, DOWN and RIGHT) are searched. According to the matching cost obtained for the 4 surrounding points, one of the 4 corner points (TOP-LEFT, TOP-RIGHT, BOTTOM-LEFT and BOTTOM-RIGHT) is included in the search space. Gradient of the cost is used in the selection process.

The predictions corresponding to the candidate search points are generated using bilinear interpolation.

Construction of template and matching function:

Before beginning of MV0 refinement, the motion vectors are used to obtain the motion compensated predictions P1[CENTER] and P2[CENTER]. The template is constructed as T = P1[CENTER] + P2[CENTER]. Division, rounding or clipping are not applied, P1 and P2 are generated using bilinear interpolation.

The normalized SAD is used as the cost function, the matching cost is given by SAD(T – mean(T), 2 * P[x] – 2 * mean(P[x])).

Refined mv application (optional):

Refined MVsare only applied on current block motion compensation. No mv update process.

Simulation results

The proposed DMVR has been implemented on top of VTM-1.0 and BMS-1.0[1], all the tests were conducted under the common test conditions [2].

Table 1 shows the results ofoptimized DMVR without mv update process.

Table 1. OptimizedDMVR performance without mv update

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.62% | -1.32% | -1.45% | 107% | 117% | 0.80% | 0.60% | 0.73% | 98% | 96% |
| Class A2 | -2.47% | -2.49% | -2.54% | 106% | 118% | 1.11% | 1.07% | 1.13% | 99% | 95% |
| Class B | -1.23% | -1.22% | -1.21% | 106% | 116% | 0.32% | 0.35% | 0.41% | 98% | 94% |
| Class C | -0.88% | -1.03% | -1.01% | 106% | 113% | 0.39% | 0.50% | 0.54% | 99% | 94% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.46% | -1.44% | -1.47% | 106% | 116% | 0.59% | 0.58% | 0.65% | 99% | 95% |
| Class D | -0.77% | -0.91% | -1.02% | 107% | 115% | 0.44% | 0.48% | 0.65% | 98% | 93% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

It can be seen that the proposed DMVR without mv updatecan achieves averagely 1.46% BD-rate gain for VTM1.0 with 6% encoding time increase and 16% decoding time increase. When tested under BMS configuration, it is about 0.59% BD-rate lost with 1% encoding time and 5% decoding time saving.The lost is come from the remove of mv update process, which may be good for hardware realization and complexity reduction. It’s also supply performance of proposed DMVR with mv update list below.

Table 2. Optimized DMVR performance with mv update

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.50% | -2.55% | -2.89% | 108% | 118% |  |  |  |  |  |
| Class A2 | -4.86% | -4.73% | -4.80% | 106% | 121% |  |  |  |  |  |
| Class B | -2.30% | -2.04% | -2.12% | 107% | 117% |  |  |  |  |  |
| Class C | -1.37% | -1.65% | -1.67% | 106% | 114% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.80% | -2.58% | -2.69% | 107% | 117% |  |  |  |  |  |
| Class D | -1.15% | -1.31% | -1.54% | 108% | 116% |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

It can be seen that the proposed DMVR with mv updatecan achieves averagely 2.80% BD-rate gain for VTM1.0 with 7% encoding time increase and 17% decoding time increase. When tested under BMS configuration, it is aboutx.xx% BD-rate lost with x% encoding time and x% decoding time saving.

Conclusion

This contribution proposes anoptimizeddecoder-side motion vector refinement algorithm. In the VTM test, an average bit-rate savings of 1.46%/2.80% with 6%/7% encoding time 16%/17% decoding time increase for without and with mv update process respectively.ToBMS anchor with around 0.59%/x.xx% BD-rate lost with 1%/x% encoding time and 5%/x%decoding time saving for without and with mv update process respectively.

References

[1] BMS-1.0, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0

[2] J.Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010,San Diego, US, 10–20 Apr. 2018.

Patent rights declaration(s)

HiSilicon Technologies and Huawei Technologiesmay have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).