| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18July 2018 | Document: JVET-K0200 |
| --- | --- |

| Title: | CE9:DMVR extension based on template matching (Test 9.3.6) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xu Chen, Jianhua Zheng | Tel: Email: | +86-13480990067 anderson.chen@hisilicon.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

- Abstract

A decoder-side motion vector refinement extension algorithm is proposed. The algorithm is based on template matching and aims to reduce the bit-rate of motion vector by refining the motion vectors at decoder side. The proposed technologies option1 can provide 1.70%/0.94%/0.63% gain for RA/LB/LP respectively compared to VTM-1.0 anchor with around 56% encoding time and 22% decoding time increase, and can provide 0.71%/0.66%/0.36% gain for RA/LB/LP respectively compared to BMS-1.0 anchor with around 15% encoding time and 8% decoding time increase. For option2, it can provide 3.12%/1.60%/1.21% gain for RA/LB/LP respectively compared to VTM-1.0 anchor with around 59% encoding time and 19% decoding time increase, and can provide x.xx%/x.xx%/x.xx% gain for RA/LB/LP respectively compared to BMS-1.0 anchor with around xx% encoding time and x% decoding time increase.

Introduction

In current BMS, DMVR only apply for merge mode of bi-prediction with one from the reference picture in the past and the other from reference picturein the future.

In this contribution, DMVR extension method is proposed and can be applied on uni or bi-prediction based on template matching.

Proposed DMVR extension

The motion vector (MV) of list0 or MV of list1are further refined by a template matching process. As shown in Figure 1, template matching performs a distortion-based search between a template (top and/or left neighbouring blocks of the current CU) in the current picture and a blockin a reference picture in order to obtain a refined MV without the transmission of additional syntax element.

Fig. 1 Template matching

In VTM (BMS) merge case, MV refined based on template matching is only applied on the first candidate of merge candidate list. Only CU level search is applied.

DMVR extension method also be applied on another new prediction mode (AM mode). AM mode is a new bi-prediction mode that is the combination mode of Merge and AMVP, which one direction using Merge method to get the prediction block and the other direction use AMVP. In this case, MV refined is only applied on the Merge direction and only CU level search is applied.

Simulation results

The proposed method has been implemented on BMS-1.0[1], and allthe tests were conducted under the common testconditions[2]. The experimental results of thecombined average merge candidatescompared to VTM-1.0 and BMS-1.0 are shown in Table1.

Option1 setting:

Search Range: 1

Search Pattern: Square

Interpolation filter: DCTIF/8

Table 1. Experimental results of DMVR extension option1

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.94% | -1.84% | -2.05% | 143% | 115% | -0.60% | -0.61% | -0.60% | 111% | 105% |
| Class A2 | -2.20% | -2.50% | -2.51% | 153% | 123% | -0.83% | -0.96% | -1.07% | 116% | 106% |
| Class B | -1.84% | -2.02% | -1.81% | 159% | 124% | -0.83% | -0.87% | -0.83% | 116% | 110% |
| Class C | -0.98% | -1.21% | -1.46% | 164% | 123% | -0.54% | -0.63% | -0.74% | 117% | 110% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.70% | -1.86% | -1.90% | 156% | 122% | -0.71% | -0.77% | -0.81% | 115% | 108% |
| Class D | -0.76% | -0.98% | -1.19% | 176% | 130% | -0.52% | -0.66% | -0.74% | 119% | 113% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.00% | -1.14% | -1.24% | 179% | 123% | -0.59% | -1.13% | -0.63% | 124% | 121% |
| Class C | -0.82% | -0.96% | -1.08% | 178% | 129% | -0.62% | -0.53% | -0.83% | 125% | 115% |
| Class E | -1.02% | -1.32% | -1.52% | 194% | 128% | -0.82% | -1.09% | -0.78% | 131% | 126% |
| Overall | -0.94% | -1.13% | -1.26% | 182% | 126% | -0.66% | -0.92% | -0.73% | 126% | 120% |
| Class D | -0.69% | -1.12% | -1.30% | 180% | 138% | -0.60% | -0.44% | -0.74% | 129% | 118% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.49% | -0.26% | -0.50% | 108% | 120% | -0.24% | -0.15% | -0.19% | 102% | 120% |
| Class C | -0.51% | -0.42% | -0.69% | 109% | 127% | -0.35% | -0.30% | -0.28% | 101% | 116% |
| Class E | -1.02% | -1.08% | -0.86% | 107% | 123% | -0.58% | 0.30% | -0.98% | 102% | 131% |
| Overall | -0.63% | -0.52% | -0.65% | 108% | 123% | -0.36% | -0.09% | -0.42% | 102% | 121% |
| Class D | -0.35% | -0.90% | -0.61% | 110% | 132% | -0.33% | -0.20% | -0.19% | 103% | 116% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Option2 setting:

Search Range: 8

Search Pattern: Cross

Interpolation filter: Bilinear

Table 2. Experimental results of DMVR extension option2

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.34% | -2.89% | -3.07% | 147% | 115% |  |  |  |  |  |
| Class A2 | -4.44% | -4.22% | -4.34% | 156% | 124% |  |  |  |  |  |
| Class B | -3.10% | -2.88% | -2.75% | 163% | 119% |  |  |  |  |  |
| Class C | -1.98% | -2.02% | -2.20% | 167% | 119% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.12% | -2.92% | -2.98% | 159% | 119% |  |  |  |  |  |
| Class D | -1.89% | -1.74% | -1.94% | 180% | 123% |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.41% | -1.18% | -1.30% | 181% | 117% |  |  |  |  |  |
| Class C | -1.34% | -1.52% | -1.36% | 179% | 122% |  |  |  |  |  |
| Class E | -2.27% | -2.18% | -2.23% | 192% | 120% |  |  |  |  |  |
| Overall | -1.60% | -1.54% | -1.55% | 183% | 120% |  |  |  |  |  |
| Class D | -1.40% | -1.61% | -1.90% | 180% | 129% |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.91% | -0.69% | -0.62% | 107% | 112% |  |  |  |  |  |
| Class C | -0.92% | -0.80% | -0.97% | 107% | 116% |  |  |  |  |  |
| Class E | -2.11% | -1.92% | -1.64% | 105% | 111% |  |  |  |  |  |
| Overall | -1.21% | -1.03% | -0.99% | 107% | 113% |  |  |  |  |  |
| Class D | -0.75% | -0.85% | -0.79% | 108% | 118% |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

It can be seen that the proposed DMVR extension option1 can provide 1.70%/0.94%/0.63% gain for RA/LB/LP respectively compared to VTM-1.0 anchor with around 56% encoding time and 22% decoding time increase, and can provide 0.71%/0.66%/0.36% gain for RA/LB/LP respectively compared to BMS-1.0 anchor with around 15% encoding time and 8% decoding time increase. Option2 can provide 3.12%/1.60%/1.21% gain for RA/LB/LP respectively compared to VTM-1.0 anchor with around 59% encoding time and 19% decoding time increase, and can provide x.xx%/x.xx%/x.xx% gain for RA/LB/LP respectively compared to BMS-1.0 anchor with around xx% encoding time and x% decoding time increase.

Conclusion

This contribution proposes a new decoder-side motion vector refinement algorithm which is based on template matching and aims to reduce the bit-rate of motion vector by refining the motion vectors at decoder side. Option 1can provide 1.70%/0.94%/0.63% gain for RA/LB/LP respectively compared to VTM-1.0 anchor with around 56% encoding time and 22% decoding time increase, and can provide 0.71%/0.66%/0.36% gain for RA/LB/LP respectively compared to BMS-1.0 anchor with around 15% encoding time and 8% decoding time increase. Option2 can provide 3.12%/1.60%/1.21% gain for RA/LB/LP respectively compared to VTM-1.0 anchor with around 59% encoding time and 19% decoding time increase, and can provide x.xx%/x.xx%/x.xx% gain for RA/LB/LP respectively compared to BMS-1.0 anchor with around xx% encoding time and x% decoding time increase.

References

- BMS-1.0, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0

- J.Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010,San Diego, US, 10–20 Apr. 2018.

Patent rights declaration(s)

HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).