| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0201-v2 |
| --- | --- |

| Title: | Non-CE2.3 On SAO Parameters signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Jonathan Taquet Christophe Gisquet Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution presents a modification of the high level SAO parameters signaling. In addition to the SAO HEVC CTU level parameters derivation, the Frame, Line, Column, 2x2 CTU, 3x3 CTU, Temporal and Temporal 90° SAO parameters derivation are available at encoder side and signaling in the slice header. For the additional derivations, the traditional SAO Up and Left Merge flags are removed. In this contribution, only the parameters derivation is modified and the SAO filtering stays at CTU level. An average BDR YUV (14:1:1) gain is reported compared to VTM1.0 of -0.11%, -0.21%, -0.35%, -0.61% for respectively AI, RA, LDB and LDP configurations and an average BDR YUV (14:1:1) gain of -0.07%, -0.24%, -0.57%, -0.51% for respectively AI, RA, LDB and LDP configurations compared to BMS1.0.

# Introduction

In the current BMS and VTM softwares, SAO parameters derivation is defined at CTU level as depicted in Figure 1. The parameters can be inferred from the Left or the Up CTU thanks to the SAO merge flag signaling.

Figure 1  Current SAO parameters derivation.

This signaling can be improved by adding other SAO parameter derivation.

# Proposed method

In this contribution, we propose to modify the signaling of the SAO parameter derivation by adding new possible derivations as depicted in Figure 2.

| CTU | Frame |
| --- | --- |
| Line | Column |
| 2x2 CTU | 3x3 CTU |
| Previous frame | Temporal |
|  | Temporal 90° |

Figure 2 Proposed SAO parameters derivations.

The CTU level is exactly the same as the current derivation. For the Frame level, only one SAO parameters set is transmitted for the current slice. For Line level, one SAO parameters set is transmitted for each line. In the same way, for Column, 2x2 CTU and 3x3 CTU, one SAO parameters set is transmitted for respectively each column, each 2x2 CTU and each 3x3 CTU.

For the Temporal SAO parameters derivation, the SAO edge offset types are copied from the reference ref0 of List 0. For all the other CTUs a default SAO parameters set is used and transmitted once per slice. The Temporal 90° has the same functioning as the Temporal derivation expect that each SAO edge offset type from the previous frame is changed by its corresponding 90° SAO edge type. Of course the temporal derivation is applied only for Inter slices.

For all these new additional SAO parameters derivations, no SAO merge modes are transmitted.

Moreover, the SAO classification and filtering is not modified and stays at CTU level.

At encoder side, the proposed SAO parameters derivations don’t need additional SAO classifications compared to the current implementation. Indeed, each RD cost value is derived from the CTU level SAO statistics. Consequently, the RDcost for the additional SAO parameters derivations are obtained by some sums of existing tables in the VTM software.

# Experimental results

The proposed method is implemented on top of the BMS1.0 software and all tests were conducted according to the common test conditions [1]. Table 1 gives the experimental results of the proposed method compared to the BMS1.0 for VTM and BMS configuration.

Table 1 Results of proposed method, AI configuration

# Conclusion

In this contribution, additional SAO parameters derivations are competing at encoder side. The best one is signaled at slice header and used for the SAO filtering at decoder side. When the proposed modification is enabled a coding gain of 0.32% compared to the BMS1.0 is obtained for VTM configuration in average for all configurations and a coding gain of 0.37% compared to BMS configuration. The proposed method has no impact on encoding and decoding complexity. Considering the gain without impact on run time complexity, we recommend to consider these modifications for a core experiment.

# References

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010, San Diego, US, April 2018.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).