| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0202-v1 |
| --- | --- |

| Title: | Non-CE2.3 On SAO Edge offset classification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Jonathan Taquet Christophe Gisquet Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution presents a modification of the SAO Edge offset classification and offsets coding. The modification of the Edge offset classification is similar to those proposed in CE2-3.4 which consist in modifying the sign function used for the Edge offset category determination. In this contribution, when the same classification as CE2-3.4 is enabled, the peak and valley offsets are coded with an explicit sign signaling as Band offsets and the Luma offsets are predicted by a default value. Moreover, in a second modification, several sign functions are competing at encoder side and explicitly signaled in the bitstream. An average BDR YUV (14:1:1) gain is reported compared to VTM1.0 of ??%, ??%, ??%, ??% for respectively AI, RA, LDB and LDP configurations for the first modification and an average BDR YUV (14:1:1) gain of ??%, ??%, ??%, ??% for respectively AI, RA, LDB and LDP configurations for both modifications.

# Introduction

In CE2.3.4, 4 SAO modifications were proposed. One is related to the SAO classification and consists in changing the Edge offset category determination according to the rules given in the following table:

| EdgeIndex | Condition |
| --- | --- |
| 0 | c < a -3 && c < b -3 |
| 1 | ( c < a -3 && ¦c-b¦≤3) ¦¦ (¦c – a¦≤3 && c < b -3) |
| 3 | ( c > a +3 && ¦c-b¦≤3) ¦¦ (¦c – a¦≤3 && c > b +3 ) |
| 4 | c > a +3 && c > b +3 |
| 2 | None of the above |

Table 1 Edge offset classification rules.

This consists in changing the sign function used to determine the EdgeIndex in the following formula:

EdgeIndex = sign (C - Cn2) - sign (Cn1- C) +2

With CE2.3.4.a the sign function is the following:

sign(val) = (val < -3 ? -1 : (val > 3 ? 1 : 0))

And the definition of the function sign(.) is given by the following relationships:

sign(val)= -1 if “val” < -3

sign(val)= 0 if   -3<= “val” <= 3

sign(val)= +1 if “val” > 3

Unfortunately, the related CE2.3.4.a gives average coding efficiency losses for some testing configurations. This is due, first, to a non suitable coding of offsets when the sign function is changed and also to the value “3” inside the sign function which is not adapted for all inputs signal and quantization errors.

In this contribution, we propose to change the coding of the offsets related to EdgeIndex 0 and 4 (corresponding to the peak and valley) and to signal the sign function used for edge offset.

# Proposed method

## Edge Offsets coding

For both Luma and Chroma components, the sign (syntax element sao_offset_sign) is explicitly signaled for Edge offsets corresponding to EdgeIndex 0 and 4.

For Luma only, the offset corresponding to EdgeIndex 0 is predicted by the value 3 and the Edge corresponding to EdgeIndex 4 is predicted by the value -3

## Edge offset sign method signalling

For the second modification, we propose the following Edge offset classification rules where N is signaled.

| EdgeIndex | Condition |
| --- | --- |
| 0 | c < a -N && c < b -N |
| 1 | ( c < a -N && ¦c-b¦≤N) ¦¦ (¦c – a¦≤N && c < b -N) |
| 3 | ( c > a +N && ¦c-b¦≤N) ¦¦ (¦c – a¦≤N && c > b +N) |
| 4 | c > a +N && c > b +N |
| 2 | None of the above |

Table 2  Proposed Edge offset classification rules.

As for CE2.3.4.a, this new classification consists in changing the sign function used to determine the EdgeIndex in the following formula:

EdgeIndex = signN( C - Cn2, N) - signN (Cn1- C, N) +2

Where

signN(val, N ) = (val < -N ? -1 : (val > N ? 1 : 0))

Where the definition of the function signN(val, N) is given by the following relationships

sign(val, N)= -1 if “val” < -N

sign(val, N)= 0 if   - N <= “val” <= N

sign(val, N)= +1 if “val” > N

To limit the rate dedicated to the signaling of value N, only four values are available: 0, 2, 4 or 8. Consequently, this value is signaled with 2 bits.

The absolute default predictor value for the Luma offsets peak and valley categories is set equal to this value N.

# Experimental results

The proposed methods are implemented on top of the BMS1.0.

# Conclusion

In this contribution, two modifications of the Edge offset classification are proposed. The first one relates to the modification of the offsets signaling for Edge offsets peak and valley. The sign is explicitly signalled for these offsets and a default value predictor is used to predict these offsets for Luma component. The second method consists in signaling the sign method used to determine the Edge class.

When both methods are combined a coding gain of ??% compared to the BMS1.0 is obtained for VTM configuration. The proposed method has no impact on encoding and decoding complexity. Considering the gain without impact on run time complexity and without increase of the worst case complexity decoding, we recommend to consider these modifications for the VTM2.0.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).