| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0203-v1 |
| --- | --- |

| Title: | Non-CE2: higher-precision modifications to VVC deblocking filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christophe Gisquet Jonathan Taquet Patrice Onno Guillaume Laroche | Tel: Email: | christophe.gisquet@crf.canon.fr |
| Source: | Canon Research France |  |  |

_____________________________

# Abstract

This proposal describes three modifications to parameters of the deblocking filter.

It first asserts that the tC parameter, linearly derived according to bitdepth from a table, suffers from a lack of precision and thus proposes a bitdepth-dependent scalar multiplication approach.

Secondly, said parameter, used to clip the output of deblocking filters within a range of the filtered sample, is asserted to be too high compared to at least the distortion produced by the BMS. It then proposes to reduce the maximal value in the same fashion for all deblocking filters.

Finally, it proposes a new condition on whether to filter a chroma edge, dependent on the flatness of the luma as measured by the luma deblocking filter on the corresponding luma edge.

It reports achieving for Y -0.3%/-0.1%/-0.3% over the VTM anchor for respectively AI/RA/LDB, and -0.8%/-0.3%/-0.6% over the BMS anchor. For YUV, using 14:1:1 weights, these numbers are respectively -0.3%/-0.3%/-0.4% and -0.9%/-0.5%/-0.8%.

# Technical content

HEVC was primarily studied around 8 bits content, as even the Main10 mostly relied on such content left-shifted to 10 bits. Furthermore, BMS, and JEM, is producing content that has much lower distortion at the same input quantizer step. We consider that VVC suffers from the HEVC deblocking filter under the current settings, and can be simply tuned to prevent some of its negative downsides.

## Relation to bitdepth

### Problem statement

Table 8-12, “Derivation of threshold variables β′ and tC′ from input Q” in HEVC specifications was designed partially based on 8 bits content, and size considerations. One parameter, β, is thus obtained from said table, and scaled according to bitdepth. To the authors, β can be seen as a feature-detection parameter, e.g. the flatness of the sample value curves around an edge for the deblocking filter. It is therefore critical so as to not smear meaningful edges or features.

Especially compared to AVC, the tC parameter is interesting, because it clips the sample value output by the selected deblocking filter within a range around the value of the input (to be filtered) sample. To the authors, it is basically related to the (maximum) error after quantization, and its likelihood, but not only, as seen in the “dSam” decision process from the HEVC specifications.

The relation from the table values and the values used in filtering is laid out simply, for example for luma in equations (8-351) and (8-353): val = val’ * ( 1 << ( BitDepthY − 8 ) ).

In all instances, there is this proportional increase of the parameters according to bitdepth of the concerned component. We assert that this rule is sub-optimal, at least, and demonstrably, concerning tc.

Furthermore, the higher the bitdepth, the coarser the values. Indeed, table 8-12 is a quantized, fixed point representation of an ideal curve. Simple modelling under the shape exp(A*QP+B) yields A=0.11 and B=-2.64. From this, a higher precision content for table 8-12 can be derived for tC’.

As CTCs only mandate testing with an internal bitdepth of 10 bits, other bitdepths, and the related results, are obviously subject to proper testing conditions definition. For instance, for 12 bits, extended precision is worth being contemplated, and without patches, JEM is unable to encode 10 bits input as 8 bits (with related internal bitdepth), and the “NEXT” profile artificially limits VTM/BMS to 8 and 10 bits only (both through erroring on internal bitdepth or bitdepth constrain).

### Technical proposal

To achieve sufficient accuracy whatever the bitdepth, we propose the following derivation:

tc = ( tc’ * scale[ BitDepth ] ) >> N

The value of N and those in tables 8-12 (which provides tc’) and scale[ ] are co-dependant, but the core of the assertion comes from the values of the ‘scale’ table.

### Search for an “optimal” scale

Optimal is very debatable, in particular concerning the deblocking filter. The following may affect what that value is:

- Encoder configuration, e.g. All Intra vs Low-Delay vs Random Access;

- Quality evaluation, whether subjective (MOS, …) or objective (PSNR, SSIM, CIE*, …);

- Colorspace used, and the weighting of components thereof, if not accounted by the metrics;

- Sequences used.

During HEVC development, it was customary, such as in [1], to do a weighted average of the PSNR values of the Y/Cb/Cr components, with the weights being 6:1:1. Study of the impact of changing the chroma delta QP from [2] suggests a nice linear relation with weights of around 14:1:1. CfP response [3] even used 18:1:1 for some plots.

To alleviate initially the case for visual validation, an objective search for a given bitdepth can be performed according to any number of points from the above. In all cases, whatever the considered objective metrics and scenarios/configuration, the rule

scale[ BitDepth ] = 1 << ( BitDepth − 8 )

was found to be suboptimal. The next figure represents the variation observed for JEM, various objective metrics including MS-SSIM and PSNR and All Intra, Random Access and Low Delay P scenarios.

The figure below represents a scaling found during the development of JEM. First results also indicates a similar behaviour for BMS.

## Prediction and quantization errors

### Problem statement

The release of VTM1.0 and BMS1.0 (and of any subsequent version) underlined a fact: at any given QP, configuration or sequence, the PSNR of VTM1.0 (or BMS1.0) is up to 0.4dB (or 1.0dB) better than that of HM16. Such numbers are expected to increase (JEM7.0 was up to 2dB), so keeping the HEVC settings for the deblocking filter at a given QP is likely to hinder coding efficiency.

### Technical proposal

However, tC parameter currently controls several conditions, not all them directly linked to quantization errors. To reduce this, we simply divided by 2 the clipping range for all filters, while e.g. still maintaining the ratio between strong and weak filters, the conditions and filter coefficients. This is therefore a simplistic change, amounting to a 2-character change to the source code.

This allows not to lower as much the value of tc when used elsewhere, such as luma filter selection, and can therefore be mostly attributed to the correction of quantization error.

This was not tested against longer filters, which are new compared to HEVC, and for which there is no well-studied and agreed clipping range yet.

## Chroma filtering

### Problem statement

Although proposals were made for a more fine-grained chroma filtering, e.g. in the image of the luma filtering, it has remained relatively simpler. As a consequence, chroma filtering is often “all or nothing”, therefore with a conservative, weak, filter needed. This is a reasonable design, as the increased data flow required for any more precise filtering was not seen as a good trade-off. This trade-off is at the core of the proposed changes.

### Technical proposals

#### Luma-based conditional filtering

It could be argued that the relative behaviour between “first” (traditionally luma) component, and others (traditionally chroma), is often dependent on the colourspace, yet the “Chroma from Luma” method has been a staple of new techniques beyond HEVC’s. Concerning deblocking, however, if such high-detail content as the luma component suffers from blocking artifacts, then more than likely, chroma also does, and conversely if it does not.

The value ‘d’ computed in 8-301 to decide whether to filter a luma edge is simply reused to decide whether to filter the corresponding edge in chroma. As a consequence, a chroma edge is never filtered if its luma edge is not filtered, which certainly helps data flow.

#### Supplemental chroma samples filtering

HEVC chroma filter is unconditionally modified to filter 2 additional samples, albeit without accessing more samples than in the current design. Current filtering equations are as follows:

Δ = Clip3( −tC, tC, ( ( ( ( q0 − p0 )  <<  2 ) + p1 − q1 + 4 )  >>  3 ) )

p0′ = Clip1C( p0 + Δ )

q0′ = Clip1C( q0 − Δ )

The added filtering is:

Δ1 = Δ >> 1

p1′ = Clip1C( p1 + Δ1 )

q1′ = Clip1C( q1 – Δ1 )

Longer filters, or more adaptive, can obviously be combined with the idea, but we estimated the trade-off not to be worthwhile.

# Results for VTM1.1

# Conclusion

We recommend studying simple scaling in different places in the HEVC deblocking filter in the scope of CE2 to properly assess the gain provided by newer techniques such as longer or more adaptive filters, in particular in light of the deblocking filter changing behaviour depending on bitdepth. It is expected to then adopt a meaningful combination through a continuation of the current CE2.

# Acknowledgments

We would like to thank Huawei for conducting the cross-check for this test in contribution JVET-K04xx.

# References

- JCTVC-J0236, “Comparison of Compression Performance of HEVC Draft 7 with AVC High Profile,” Stockholm, SE, Oct. 2012

- JVET-H0045, “EE3: Test on Chroma QP offset,” Macau, CN, Oct. 2017

- JVET-J0024, “Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario,” San Diego, US, Apr. 2018

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).