| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-Kxxxx-v1 |
| --- | --- |

| Title: | CE1: Dual Tree in I Slices in ABT configuration (tests 5.2.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Fabrice Le Léannec, Tangi Poirier, Franck Galpin. | Tel: Email: | +33(0)299273198 fabrice.leleannec@technicolor.com tangi.poirier@technicolor.com franck.galpin@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports the results of Separated Tree for Luma and Chroma in I slices, combined with the Asymmetric Binary Tree (ABT) partitioning method, described in JVET-J0022 and implemented within the JVET BMS.

# Introduction

Asymmetric Binary Tree partitioning was proposed in JVET-J0022, in combination with the separation of coding tree between Luma and Chroma components in Intra slices. This contribution reports the results of separating the Luma/Chroma coding trees in Intra slices, in combination with the ABT partitioning method, when implemented in the BMS software with the Common Tests Conditions.

The CE1 tests reported in this document is the following one: 5.2.3.

# Description

The same partitioning method as in contribution JVET-K0197 is used. The ABT operating point chosen in this test is the same as the one from CE1 test 1.0.1. The only difference is that the Dual partitioning tree feature is used in Intra slices.

# Results

The performances, encoding times and decoding times of each of the following tests are reported: 5.2.3.

## Test 5.2.3: QTBT+ABT test 1.0.1, where luma/chroma coding trees are separated in Intra slices

Apart from the entropy coding and post-quantization scaling described in section 2.3, the overall VTM1.0 configuration parameters used in this test are the following ones.

- Minimum and maximum CTU size: same as VTM1.0

- Minimum and maximum TU size: 2 to 64

- Minimum and maximum PU size: 2 to 64 (no partitioning of CUs into PUs)

- Additional transform types and sizes:

  - VTM1.0: DCT2 is extended to sizes 6,12,24,48

  - BMS1.0: the EMT set of transforms is extended to sizes 6,12,24,48

### VTM-1.0

### BMS-1.0

# Discussion

…..

# Conclusion

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).