| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0206-v1 |
| --- | --- |

| Title: | Improved perceptually optimized QP adaptation and associated distortion measure |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET (AHG 10) |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christian Helmrich Heiko Schwarz Detlev Marpe Thomas Wiegand | Email: | christian.helmrich@hhi.fraunhofer.de heiko.schwarz@hhi.fraunhofer.de detlev.marpe@hhi.fraunhofer.de thomas.wiegand@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI, Einsteinufer 37, 10587 Berlin, Germany |  |  |

_____________________________

# Abstract

In JVET-H0047 [1], the authors proposed a CTU-wise subjectively optimized QP adaptation (QPA) along with a correspondingly weighted PSNR (WPSNR) distortion measure. This QPA approach was submitted [2] and evaluated [3] for JVET’s 2018 Call for Proposals [4] and is a part of the VTM/BMS 1 software [9].

This document proposes seven improvements to the QPA and, if applicable, the associated distortion metric:

- disable the CTU size and beating reduction for base QPs higher than 37 or GOP sizes lower than 5

- use a frame-wise instead of CTU-wise QPA for base QPs higher than 37 or input smaller than 480p

- use a more isotropic high-pass filter kernel with 9 instead of 5 taps in the visual activity computation

- adapt the lower activity limit amin to the modified high-pass filter and make it bit-depth dependent

- use β=12 instead of β=23 as activity weighting exponent for more conservative bit-rate distribution

- re-adjust the mean activity value apic for correct rate matching after changed filtering, amin, and β

- replace fixed chroma QP offsets with the transmission of picture-adaptive slice chroma QP offsets.

The subjective merit of these modifications is demonstrated via formal comparative subjective evaluation. It is suggested to adopt the proposed improvements to the QPA design in the next version of the VTM/BMS.

# Review of the Perceptual QP Adaptation Method

The perceptual QP adaptation (QPA) algorithm presented in [1] and employed in HHI’s response to JVET’s Call for Proposals (CfP) on video compression technology with capability beyond HEVC [4] is based on a subjectively motivated block-wise weighted distortion metric Dw derived from a local visual activity value:

Dpicw=kDkw=kwk⋅Dk=kwk∙x,y∈Bks[x,y]-s'[x,y]2,    wk=apicakβ,

where B denotes the sub-blocks (here CTUs) of the picture, indexed via k, wk is the perceptual weight (also called visual sensitivity measure), ak is the block’s visual activity, apic is the picture’s mean visual activity, and s, s' are the original and reconstructed pel values, respectively, of the picture’s luma component. Using DpicwSSE, the picture’s width W, height H, and component bit-depth BD, a weighted peak signal-to-noise ratio

WPSNR=10⋅log10W⋅H⋅2552⋅22BD-16DpicwSSE

can be obtained. Notice that, when wk= 1 for all blocks, this WPSNR definition reduces to the conventional PSNR metric used by the JVET. This is particularly true when β=  0. In [1], the use of β=23 was suggested.

During encoding, the perceptual QPA makes use of wk to adapt in each CTU at k the perceptually optimized QP and Lagrange parameter based on the default pre-assigned fixed QPslice and Lagrange parameter λslice:

QPk=QPslice-3 log2wk,    λk=λslicewk.

Using the adapted QPk and λk in each CTU block ensures that, in terms of visual quality and WPSNR, the coding distortion is optimally distributed within the given picture and within the pictures of the video signal.

Fraunhofer HHI’s response to the CfP included two versions of the NextSoftware-based codec, which were configured identically except that, in one version, the perceptual QPA was additionally activated. Thus, the two codec responses – the fixed-QP PSNR optimized and the adaptive-QP WPSNR optimized one – can be directly compared subjectively in order to assess the visual benefit of the QPA algorithm. In fact, subjective tests allowing for such a comparison have been conducted in the course of the CfP response evaluation [3].

Analyzing the mean opinion score (MOS) data collected for the CfP response evaluation [3] and deriving differential MOS (DMOS) values between the adaptive-QP and fixed-QP versions, the authors noticed that

- at the low-rate points R1 and R2, the QPA results in statistically significant quality degradations on some content, particularly Campfire, an ultra-high-definition (UHD) recording of a nightly bonfire,

- at the low-rate points, the DMOS variance is larger than at the high-rate points (especially rate R4).

# Proposed Changes to the Perceptual QPA Method

Based on the analysis of the subjective evaluation given in Section 1, the following seven modifications are proposed so as to optimize the visual performance of the QPA approach for a wide range of operating points.

## Disabled CTU size and beating reduction if base QP > 37 or GOP size < 5

In [2], the QPA method reduces the CTU size for HD input to minimize visible blocking artifacts within the decoded output. Moreover, a “beating reduction” scheme is applied especially in non-Intra frames to reduce time-variant loss of detail due to coarse quantization in low-activity image regions; see [1], [2] for specifics.

It was found that these algorithmic components lead to significant reductions in coding efficiency especially at very low bit-rates, i. e., very high base QPs above approximately 38, and in case of small group of picture (GOP) sizes, i. e., the short jointly coded picture groups used in Intra-only (GOP size 1) and some low-delay configurations (GOP sizes between 1 and 4). Thus, the authors recommend to disable said QPA components

- when the coding bit-rate is very low, as indicated by a user-specified base QP valued QPbase ≥ 38,

- when the coding configuration is all-Intra (AI) or low-delay (LD), as indicated by a GOP size ≤ 4.

## Frame-wise instead of CTU-wise QPA if base QP > 37 or video size < 480p

In [1] and [2], the basic QPA algorithm is applied unchanged for both high and low coding bit-rates as well as large and small video input dimensions. Low-resolution video sequences like those of JVET’s “class D” [5], [6], [7], however, comprise only a few CTUs per frame during coding. This makes it impossible for a CTU-wise QPA approach such as the present one to adapt to finely located spatial changes in visual activity. Moreover, Section 1 noted an increase in DMOS variance due to the QPA at very low rates, i. e., very high base QPs. Particularly at QPbase ≥ 38, strong quality gains as well as losses can be observed, which indicates that, due to the coarse quantization, reliable spatial distribution of the coding distortion becomes infeasible. Therefore, it is proposed to disable the CTU-wise delta-QP coding and to apply a “global” frame-wise QPA

- when the coding bit-rate is very low, as indicated by a user-specified base QP valued QPbase ≥ 38,

- when the coded input video sequence has a width ≤ 512 luma pels and a height ≤ 320 luma pels.

A frame-wise QPA can be realized by adapting the slice QP, i. e., for each frame with QPslice<QPmax, set

QPslice'=Clip30;QPmax;QPslice+x3β∙ log2aYapic,   aY=maxamin2;1|PY*|[x,y]∈PY*h[x,y]2,

where Y indicates the luma component, QPmax= 51 in VTM/BMS 1, and PY* is the picture buffer, excluding the boundary pel rows and columns for which no h[ ] values are available. If, in addition, luma-based QPA for HDR input is enabled (by providing, in addition to --PerceptQPA=1, a --LumaLevelToDeltaQPMode=1 configuration parameter), QPslice' is further modified according to the frame’s mean luma value (DC offset):

QPslice''=Clip30;QPmax;QPslice'+1-3JY2∙21-2BD,

with JY=1PY∙x,y∈PYsx,y being the mean sample value of the picture’s luma buffer, similarly to [2]. Using QPslice'' or QPslice' for HDR or SDR content, respectively, as the final slice QP for coding of the frame ensures a monotonous bit-rate progression when increasing or decreasing the base QP around QPbase=  38.

## Use of a more isotropic high-pass filter kernel with 9 instead of 5 taps

To determine the local visual activity ak of a block Bk at index k, the luma input samples s are subjected to a 5-tap plus-shaped high-pass filter approximating a negated discrete Laplace operator. While such high-pass filtering, with filter kernel coefficients [–1, –1, 4, –1, –1], exhibits very low complexity, it does show a bit of angular dependency. More specifically, equally strong edges with different spatial angles in the input image lead to edges of varying strength after the high-pass filtering. A more isotropic approximation of the Laplace operator and, thus, a more “robust” computation of ak, is achieved with a 9-tap square-shaped filter kernel having the coefficients [–1, –2, –1, –2, 12, –2, –1, –2, –1]. Figure 1 depicts the transfer functions of the negated 5-tap and 9-tap high-pass kernels to illustrate the more isotropic performance of the latter filter.

Given these observations, the authors suggest to modify the high-pass filter operation as follows. Instead of

hx,y=4⋅sx,y-sx-1,y-sx+1,y-sx,y-1-sx,y+1,

it is recommended to utilize a 9-component weighted summation to determine the high-pass output h[x,y]:

hx,y=12⋅sx,y-2⋅sx-1,y-2⋅sx+1,y-2⋅sx,y-1-2⋅sx,y+1

- sx-1,y-1-sx+1,y-1-sx-1,y+1-sx+1,y+1.

The spatial range of 0<x<W-1, 0<y<H-1 as well as the definition of Bk* stay the same as in [1].

Figure 1. Transfer function of 5-tap and 9-tap high-pass filters. Image courtesy of Jkrieger, 2006, from [8].

## Bit-depth dependent lower activity limit amin accounting for new filter gain

In [2], the visual activity measure ak for a CTU located at block Bk is defined as (see [1] and [2] for details)

ak=maxamin2; 1|Bk*|[x,y]∈Bk*h[x,y]2

with lower limit amin= 8 (note that the value 1 in [2] is a typographical error, it does not account for the gain of 8 due to the signal amplification caused by the integer high-pass filter implementation). This lower limit, however, was empirically derived using 8-bit coded video data and should be specified depending on the channel’s (here luma) coding bit-depth BD. Moreover, due to the increased gain of the 9-tap high-pass filter suggested in the previous section, the numeric constant in the declaration of amin should be redefined.

Since the 5-tap filter exhibits a gain of 8, while the 9-tap filter has a gain of 24, the authors suggest to apply

amin=2BD-4

instead of amin=  8 in the derivation of ak, yielding a value of 16 for 8-bit coding and 64 for 10-bit coding. This renders the subjective QPA behavior largely independent of the actual coding bit-depth and, as a convenient side-effect, reduces some QPA-related quality regressions in case of the BD= 10 in use with, e. g., JVET’s Common Test Condition (CTC) for SDR, HDR/WCG, and 360° content [5], [6], [7]; see Section 3.

## Use of β=12 instead of β=23  for a more conservative activity weighting

At the higher-rate operating points in the CfP evaluation [3], [4], the QPA was found to yield a slight overall improvement in subjective MOS performance. For a small number of sequences, however, the visual quality did not improve or was even degraded. The authors attribute this behavior of the QPA algorithm to a slightly too aggressive perceptual distribution of the coding rate within pictures with locally varying visual activity.

In the discussed QPA and associated WPSNR distortion measure, the parameter controlling the strength of the mapping between local visual activity ak and adapted QPk or weight wk is the weighting exponent β. It is proposed to slightly reduce its value from β=23 to β=12 to obtain a more conservative weighting in the visual sensitivity measure and, thereby, the QPA and WPSNR metric. Note that, on some hardware, the new value for β can be realized more easily since a power-of-12  can be implemented with a square-root operation.

## Re-adjusted mean activity value apic after changed filtering, amin, and β

In [2], the “global” mean visual activity value apic employed in the CTU-wise QPA was determined empirically for the underlying CfP response software (NextSoftware) and sequence configuration (HD and UHD videos only). Its purpose is to match, for a given base QP, the average bit-rate of the QPA coded bit-streams (determined across a large set of video sequences) with the average bit-rate obtained using fixed-QP coding.

Having changed the high-pass filtering, lower activity limit amin, and activity weighting exponent β above, it is necessary to update apic. For a large test set such as all CTC sequences [5], [6], [7], the authors suggest

apic=2BD∙ 16for UHD sequences>2048×1280 pels, 32for HD sequences>1024×640 pels, 64otherwise,

where BD denotes the coding bit-depth as in [1], [2] and above. Note that this modification of apic, which now includes a case for smaller-than-HD sequences, only represents a constant QP offset subtracted from all adapted QPs during coding. It does not change or extend the underlying principles of the QPA algorithm.

## Replacement of fixed PPS-wise with adaptive slice-wise chroma QP offsets

In HEVC and VTM/BMS version 1, a predefined luma-QP-to-chroma-QP mapping is employed during the quantization and reconstruction of 4:2:0 coded input (ChromaArrayType =1); see Table 8-10 in [10], Section 8.6.1 for details. This mapping effectively reduces the chroma QP – and, thereby, the chroma quantization distortion – at high luma QPs near QPmax, which may explain why, at low coding bit-rates, the use of positive chroma QP offsets via, e. g., the transmission of non-zero pps_cb_qp_offset and pps_cr_qp_offset values in the PPS header [10] has recently been suggested. In combination with the proposed subjective QPA, such a reduction of the chroma QP values relative to the luma QP values seems to be unnecessary in terms of visual reconstruction quality, particularly when a cross-component predictive coding technique such as the linear-model chroma (CCLM) predictor in the BMS and JEM [11] is utilized. In fact, excessive fixed redistribution of coding bits to the chroma channels at low rates potentially leads to visible quality reduction on individual video sequences with the VTM/BMS. Moreover, since the QPA method may increase or decrease the coding rate for a specific sequence and given base QP (but not on average across a large set of test sequences, see section 2.6), the different luma-to-chroma QP mapping at low and high QPs according to [10] complicates meaningful “direct” objective comparisons using Bjøntegaard delta-WPSNR (BD-WPSNR) measures [12].

Given the abovementioned disadvantages of a fixed input-independent luma-to-chroma QP adjustment, the authors propose to, when perceptual QPA is enabled, employ input-dependent adaptive luma-to-chroma QP offsets on a frame-by-frame or a slice-by-slice basis. Specifically, it is suggested to extend the luma-based QPA approach of [1] by the following backward-compatible HEVC syntax-compliant chroma QPA method:

- When the --PerceptQPA or --SliceChromaQPOffsetPeriodicity encoder configuration parameter is provided with a positive value, set pps_slice_chroma_qp_offsets_present_flag = 1 in the coded PPS.

- For each slice and chroma component c∈  [Cb, Cr] present in the coded image or video, determine a value QPc from the ratio between said chroma channel’s mean activity and the mean luma activity:

QPc= x3β∙ log2α∙acaY,α∙ac>aY,0,otherwise,    ak=maxamin2; 1|Pk*|[x,y]∈Pk*h[x,y]2,

where subscript Y denotes the luma component, β=12 as in Section 2.5, k∈  [Y, Cb, Cr], and Pk* is the component’s frame buffer, excluding the border pel rows and columns to prevent the high-pass filter from extending beyond the picture boundaries. Value α is set empirically and given by α=  4.

- Obtain the frame-wise mean luma QP already derived in Section 2.2, using the apic of Section 2.6:

QPY= QPslice''for HDR content, QPslice'otherwise.

- Define d as the difference between QPY and the luma-to-chroma mapping of QPY according to [10]:

d=QPY-QpCQPY,   QpC ≔chroma QP lookup via HEVC Table 8-10.

- Set Oc=minOmax+d;QPc+d with Omax=  3. This chroma QPA effectively lowers the coding bit-rate of the chroma channel when its mean visual activity is relatively high compared to the luma channel’s visual activity. At the same time, it compensates for the picture-averaged fixed luma-to-chroma QP mapping by QpC  to prevent undesired re-shifting of coding bits from luma to chroma.

The upper limit Omax is introduced to prevent very coarse chroma-channel quantization on some sequences at lower bit-rates. Within the CTC sequence set, it only affects the encoding of the ParkRunning3 sequence. It is worth mentioning that the slice-wise offsets Oc, which can be transmitted as the slice_cb_qp_offset and slice_cr_qp_offset elements in HEVC and VTM/BMS 1, can still be combined with non-zero PPS-wise QP chroma offsets (pps_cb_qp_offset and pps_cr_qp_offset) in the traditional fashion known from HEVC [10]. Moreover, it is worth repeating that no new syntax elements are introduced by the above algorithm, so the existing VTM/BMS software can decode the bit-streams encoded with the additional chroma QPA method.

# Subjective Evaluation of the Improved QPA Method

The improved perceptual QPA design proposed herein was evaluated using two comparative subjective A/B tests. The first test intended to assess the overall video coding quality of the proposal (called “test QPA” in the following) in comparison with an alternative perceptually optimized quantization parameter adaptation (named “other QPA” below) also submitted as a CfP response [4]. The second test assessed the visual performance of the test QPA relative to the authors’ existing QPA available in the VTM (termed “VTM QPA”).

For the two A/B tests, the 11 HD and UHD sequences of the SDR-category CTC set [5], listed in Table 1, were used as content. An extension of the tests to the HDR/WCG-category CTC set [6] is being considered. The bit-streams of the test QPA, other QPA, and VTM QPA conditions were rate matched to the bit-rates of the fixed-QP VTM encodings, where for each sequence a base QP of QPbase=  37 was utilized [5]. The QP step-size for this rate matching was 0.5, meaning that a base QP increase by 1 was allowed halfway through the encoding process. The target bit-rates for all sequences, as exhibited by the fixed-QP VTM 1 encodings, are provided in Table 1 and lead to a mean bit-rate of 2801 kbit/s across all sequences (median: 2312 kbit/s). The VTM QPA and fixed-QP VTM encodings were generated with PPS chroma QP offsets of 1 for fairness.

The subjective evaluation was done using direct A/B comparisons largely following the ITU-R BT.500 [13] methodology. Twelve subjects experienced in detecting video coding artifacts participated in each test. Each subject claimed to have normal color perception and took part in only one of the tests, but not in both. The decoded bit-stream stimuli of codec conditions A and B (which were randomly assigned) were presented in a temporally concatenated “GA – A – GB – B – GA – A – GB – B – GVote” manner, where G stands for a one-second mid-gray separating picture labeled with a black letter “A” or “B” at its center, depending on which coding conditions follows, or in case of the GVote, the word “VOTE” to indicate the 10-second voting period. A color-calibrated 65-inch Sony XE93 (KD-65XE9305) television with UHD panel resolution was used for presentation, on which the HD sequences were shown centered in 1:1 resolution on a mid-gray background.

The participants were asked to vote whether A or B showed slightly better overall coding quality, or whether both exhibited equal overall coding quality. Each subject’s verdict was recorded and translated to a number,

DOS=1if B has better quality than A,0if A and B have equal quality,-1if A has better quality than B,

which, given the expected subtle differences between the QPA variants, is interpreted directly as a difference opinion score (DOS). Averaging the DOS values results in a DMOS figure for codec B relative to codec A.

## Results of the first subjective A/B experiment (A: other QPA, B: test QPA)

The sequence-wise and overall DMOS values and associated 95% confidence intervals obtained from the first A/B test are enumerated in Table 2. Notice that, for 4 of the 11 sequences, the 12 subjects preferred the test QPA condition over the other QPA condition in terms of visual coding quality. It can also be concluded that averaged across all sequences, the test QPA method outperforms the other QPA method in visual quality with statistical significance (the 95% confidence interval around the DMOS value is positive in its entirety).

## Results of the second visual A/B experiment (A: VTM QPA, B: test QPA)

The sequence-wise and overall DMOS values and confidence intervals gathered from the second test are summarized in Table 3. It can be seen that, averaged over all sequences, the DMOS does not change significantly. Given that the VTM QPA runs with PPS chroma QP offsets of 1, this is a satisfactory result. In addition, for the Campfire input, i. e., the sequence on which a quality reduction due to the VTM QPA was observed during the CfP, the test QPA yields strong quality gains (all viewers preferred the test QPA condition). This, in effect reduces the DMOS variance of the QPA method at the tested low bit-rates, as desired.

Due to the more conservative tunings of Sections 2.1 and 2.2, a slightly reduced subjective quality, primarily due to stronger blocking artifacts in large CUs, is noted for the RitualDance sequence. Nonetheless, it was found that, in terms of visual quality, the test QPA encodings are still on par with the other QPA and fixed-QP encodings, and the authors believe that large-CU blocking artifacts are better addressed by re-adjusting the deblocking filter in the VTM/BMS, as already suggested within the JVET (see CE2). Thus, the improved QPA is considered a better tradeoff than the existing QPA for the future versions of the VTM/BMS software.

Table 1. Information on the sequences used for the subjective evaluation. fps: frames per second, *: frame count limitation as specified in the CTC configurations [5] to reduce the encoding workload. QPbase= 37.

| Sequence Name | Size (luma pels) | Duration (sec) | Bit-Rate (kbit/s) |
| --- | --- | --- | --- |
| BasketballDrive | 1920×1080 | 10 at 50 fps | 1239 |
| BQTerrace | 1920×1080 | 10 at 60 fps | 881 |
| Cactus | 1920×1080 | 10 at 50 fps | 1211 |
| Campfire | 3840×2160 | 10 at 30 fps | 5002 |
| CatRobot1 | 3840×2160 | 5 at 60 fps* | 2747 |
| DaylightRoad2 | 3840×2160 | 5 at 60 fps* | 2624 |
| FoodMarket4 | 3840×2160 | 5 at 60 fps* | 2431 |
| MarketPlace | 1920×1080 | 10 at 60 fps | 1264 |
| ParkRunning3 | 3840×2160 | 6 at 50 fps* | 9666 |
| RitualDance | 1920×1080 | 10 at 60 fps | 1433 |
| Tango2 | 3840×2160 | 5 at 60 fps* | 2312 |

Table 2. Results of the first subjective experiment with differential mean opinion score (DMOS) values and 95% confidence intervals (assuming a Student-t distribution). A: other QPA, B: test QPA. See text for details.

| Sequence Name | Overall DMOS | Conf. Interval | Stat. Significance? |
| --- | --- | --- | --- |
| BasketballDrive | 0.750 | ±0.287 | Yes, B > A |
| BQTerrace | 0.167 | ±0.456 | No |
| Cactus | –0.167 | ±0.596 | No |
| Campfire | 0.500 | ±0.428 | Yes, B > A |
| CatRobot1 | 0.500 | ±0.428 | Yes, B > A |
| DaylightRoad2 | –0.083 | ±0.425 | No |
| FoodMarket4 | –0.167 | ±0.366 | No |
| MarketPlace | 0.167 | ±0.366 | No |
| ParkRunning3 | 0.000 | ±0.542 | No |
| RitualDance | –0.167 | ±0.530 | No |
| Tango2 | 0.583 | ±0.327 | Yes, B > A |
| All | 0.189 | ±0.128 | Yes, B > A |

Table 3. Results of the second subjective experiment with differential mean opinion score (DMOS) data and 95% confidence intervals (assuming a Student-t distribution). A: VTM QPA, B: test QPA. See text for details.

| Sequence Name | Overall DMOS | Conf. Interval | Stat. Significance? |
| --- | --- | --- | --- |
| BasketballDrive | 0.000 | ±0.469 | No |
| BQTerrace | –0.083 | ±0.504 | No |
| Cactus | –0.167 | ±0.456 | No |
| Campfire | 1.000 | ±0.000 | Yes, B > A |
| CatRobot1 | –0.167 | ±0.367 | No |
| DaylightRoad2 | –0.417 | ±0.425 | No |
| FoodMarket4 | –0.250 | ±0.550 | No |
| MarketPlace | 0.167 | ±0.530 | No |
| ParkRunning3 | 0.083 | ±0.572 | No |
| RitualDance | –0.417 | ±0.327 | Yes, B < A |
| Tango2 | –0.083 | ±0.425 | No |
| All | –0.030 | ±0.133 | No |

# Summary and Conclusion

This input document gave a review and an analysis of the perceptual QP adaptation (QPA) method proposed in JVET-H0047 [1] and integrated into the VTM and BMS software [9], based on the results of its subjective evaluation for the Call for Proposals on video compression technology with capabilities beyond HEVC [3], [4]. In order to correct the visual quality issues observed with that version of the QPA implementation, seven modifications to the QPA design were proposed and explained. Formal subjective assessment of the modified QPA method in comparison with the original QPA version already integrated into the VTM 1 software [9] and with an alternative perceptual optimization approach recently proposed to the JVET confirms the benefit of the proposed changes without reducing the overall quality. The VTM 1 bit-streams generated for the visual assessment and for updated BD-WPSNR evaluation are available to JVET members upon request.

In light of the outcome of this investigation, the authors recommend the integration of the proposed modifications to the QPA method into the next version of the VTM and BMS software after the 11th JVET meeting in Ljubljana. The basic QPA code is already encapsulated inside a macro ENABLE_QPA, and the proposed improvements can be encapsulated inside a dependent macro ENABLE_QPA_IMPROVEMENTS if desired.

# References

- S. Bosse, C. Helmrich, H. Schwarz, D. Marpe, T. Wiegand, “Perceptually Optimized QP Adaptation and Associated Distortion Measure,” JVET-H0047, Macao, China, Oct. 2017.

- M. Albrecht et al., “Description of SDR, HDR, and 360° Video Coding Technology Proposal by Fraunhofer HHI,” JVET-J0014, San Diego, USA, Apr. 2018.

- V. Baroncini, “Results of Subjective Testing of Responses to the Joint CfP on Video Compression Technology with Capability beyond HEVC,” JVET-J0080, San Diego, USA, Apr. 2018.

- A. Segall, V. Baroncini, J. Boyce, J. Chen, T. Suzuki, “Joint Call for Proposals on Video Compression with Capability beyond HEVC,” JVET-H1002, Macao, China, Oct. 2017.

- J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET Common Test Conditions and Software Reference Configurations,” JVET-J1010, San Diego, USA, Apr. 2018.

- A. Segall, E. François, D. Rusanovskyy, “JVET Common Test Conditions and Evaluation Procedures for HDR/WCG Video,” JVET-J1011, San Diego, USA, Apr. 2018.

- P. Hanhart, J. Boyce, K. Choi, “JVET Common Test Conditions and Evaluation Procedures for 360° Video,” JVET-J1012, San Diego, USA, Apr. 2018.

- Wikipedia, “Laplace-Filter (in German),” 2006. Online: https://de.wikipedia.org/wiki/Laplace-Filter

- J. Chen and E. Alshina, “Algorithm Description for Versatile Video Coding and Test Model 1 (VTM 1),” JVET-J1002, San Diego, USA, Apr. 2018.

- ITU-T, recommendation H.265 and ISO/IEC, Int. Standard 23008-2, “High efficiency video coding,” Geneva, Switzerland, Feb. 2018. Online: https://www.itu.int/rec/T-REC-H.265

- J. Chen et al., “Algorithm Description of Joint Exploration Test Model 7 (JEM 7),” JVET-G1001, Torino, Italy, July 2017.

- G. Bjøntegaard, “Calculation of Average PSNR Differences between RD-Curves,” VCEG-M33, Austin, USA, Mar. 2001.

- ITU-R, Recommendation BT.500, “Methodology for the subjective assessment of the quality of television picture,” Geneva, Switzerland, Jan. 2012. Online: https://www.itu.int/rec/R-REC-BT.500

# Patent Rights Declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).