| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0207-v1 |
| --- | --- |

| Title: | CE4: Enhanced Chroma Interpolation Filter (Test 7.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Maxim Sychev Georgy Zhulikov Timophey Solovyev Jianle Chen | Tel: Email: | sychev.maxim@huawei.com |
| Source: | Huawei Technologies Co.,Ltd., Moscow, Russia |  |  |

_____________________________

# Abstract

This proposal describes an Enhanced Interpolation Filter (EIF) for Chroma inter prediction. The coefficients of 4-tap interpolation filter was derived from EIF described in JVET-J0024 and JVET-K0207 with light reduction of filter strength. The new filter coefficient allow to achieves ~ -0.6% of BD-rate U/V in RA without influence on complexity for the VTM configuration and ~ -0.35 of BD-rate U/V in RA for the BMS configuration.

# Introduction

Enhanced Interpolation Filter was introduced in JVET-J-0024 in San Diego in April 2018. Two filters was proposed based on bilinear interpolation with high-pass 3-tap filter in pixel precision and 5-tap filter in half-pixel precision. The convolution of bilinear interpolation filter and 5-tap high-pass filter form the basis of proposed 4-tap filter for Chroma Inter Prediction. Section 2 provides details of the proposed technique and section 3 reports detailed results.

# Enhanced Chroma Interpolation Filter

The derivation process to obtain filter coefficients is a simple convolution of bilinear interpolation with high-pass filter and performed according to the following formula for the first half of fractional offsets – other offsets obtained by mirroring first ones:

4tap=1-f*c0+ 1-f+12*c1, f*c0+ f+12*c1+1-f*c2+1-f+12*c1,f*c2+ f+12*c1+1-f*c0,f*c0

- Where c0,c1,c2,c1,c0 is 5-tap high-pass filter with coefficients -13, 26, 38, 26, -13, f – fractional offset.

The resulted coefficients provided in Table 1 for all fractional offsets with precision 1/32.

Table 1 Chroma filter coefficients

| 0 | 0 | 64 | 0 | 0 |  | 16 | -7 | 39 | 39 | -7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 62 | 2 | 0 |  | 17 | -6 | 36 | 40 | -6 |
| 2 | -1 | 61 | 5 | -1 |  | 18 | -6 | 34 | 42 | -6 |
| 3 | -1 | 59 | 7 | -1 |  | 19 | -5 | 31 | 43 | -5 |
| 4 | -2 | 58 | 10 | -2 |  | 20 | -5 | 29 | 45 | -5 |
| 5 | -2 | 56 | 12 | -2 |  | 21 | -4 | 26 | 46 | -4 |
| 6 | -2 | 54 | 14 | -2 |  | 22 | -4 | 24 | 48 | -4 |
| 7 | -3 | 53 | 17 | -3 |  | 23 | -4 | 22 | 50 | -4 |
| 8 | -3 | 51 | 19 | -3 |  | 24 | -3 | 19 | 51 | -3 |
| 9 | -4 | 50 | 22 | -4 |  | 25 | -3 | 17 | 53 | -3 |
| 10 | -4 | 48 | 24 | -4 |  | 26 | -2 | 14 | 54 | -2 |
| 11 | -4 | 46 | 26 | -4 |  | 27 | -2 | 12 | 56 | -2 |
| 12 | -5 | 45 | 29 | -5 |  | 28 | -2 | 10 | 58 | -2 |
| 13 | -5 | 43 | 31 | -5 |  | 29 | -1 | 7 | 59 | -1 |
| 14 | -6 | 42 | 34 | -6 |  | 30 | -1 | 5 | 61 | -1 |
| 15 | -6 | 40 | 36 | -6 |  | 31 | 0 | 2 | 62 | 0 |

# Results

According to the common test conditions in JVET-J1010 [3], simulations were performed for random-access (RA) and low-delay B (LDB) configuration with BMS-1.0 software in VTM and BMS configuration.

Table 2 Results for RA configuration

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.08% | 0.00% | 102% | 99% | 0.04% | -0.14% | -0.36% | 100% | 100% |
| Class A2 | -0.02% | -0.55% | -0.84% | 101% | 99% | 0.06% | -0.32% | -0.42% | 100% | 99% |
| Class B | 0.00% | -0.82% | -0.72% | 102% | 99% | -0.02% | -0.45% | -0.43% | 100% | 98% |
| Class C | -0.02% | -0.76% | -0.79% | 102% | 101% | 0.02% | -0.33% | -0.42% | 100% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.57% | -0.62% | 102% | 99% | 0.02% | -0.33% | -0.41% | 100% | 99% |
| Class D | -0.03% | -0.96% | -1.34% | 102% | 99% | 0.04% | -0.50% | -0.80% | 99% | 99% |
| Class F (optional) | 0.00% | -0.06% | -0.11% | 101% | 100% | 0.01% | 0.11% | -0.03% | 100% | 99% |

Table 3 Results for LDB configuration

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.02% | -0.02% | 0.15% | 100% | 98% | -0.02% | -0.78% | -0.26% | 100% | 104% |
| Class C | 0.02% | -0.69% | -0.90% | 101% | 98% | -0.04% | -0.84% | -1.20% | 100% | 103% |
| Class E | 0.13% | 2.17% | 2.18% | 100% | 98% | -0.04% | 1.52% | 1.71% | 100% | 102% |
| Overall | 0.03% | 0.31% | 0.31% | 100% | 98% | -0.03% | -0.22% | -0.08% | 100% | 103% |
| Class D | -0.03% | -1.09% | -1.28% | 101% | 99% | -0.01% | -1.15% | -2.18% | 100% | 102% |
| Class F (optional) | -0.10% | -0.40% | 0.01% | 100% | 98% | -0.07% | 0.34% | 0.38% | 100% | 102% |

# Conclusions

This contribution reports the results of new Chroma Interpolation Filter in JVET-J0024 to BMS1.0. Experimental results reportedly show on average 0.6% chroma BD-rate gain in RA configurations over to VTM anchor and show on average 0.35% chroma BD-rate gain over BMS configuration without influence on complexity.

We propose to adopt Enhanced Chroma Interpolation Filter into the test model.

# References

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVET-J1024, San Diego, USA, April 2018.

- A. Alshin, E. Alshina, K. Choi, N. Choi, W. Choi, S. Jeong, C. Kim, J. Min, J. H. Park, M. Park, M. W. Park, Y. Piao, A. Tamse, A. Dsouza, C. Pujara, H. Chen, J. Chen, R. Chernyak, S. Esenlik, A. Filippov, S. Gao, S. Ikonin, A. Karabutov, A. M. Kotra, X. Lu, X. Ma, V. Rufitskiy, T. Solovyev, V. Stepin, M. Sychev, T. Wang, Y.-K. Wang, W. Xu, H. Yang, V. Zakharchenko, H. Zhang, Y. Zhao, Z. Zhao, J. Zhou, C. Auyeung, H. Gao, I. Krasnov, R. Mullakhmetov, B. Wang, Y. F. Wong, G. Zhulikov, A. Abbas, D. Newman, J. An, X. Chen, Y. Lin, Q. Yu, J. Zheng, “Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario” JVET-J0024, San Diego, USA, April 2018.

- X-Check JVET-Kxxxx

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, April 2018.

- Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).