| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0208-v2 |
| --- | --- |

| Title: | CE4.3.6 Unicity in motion information candidate list from J0022 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | A. Robert, T. Poirier, F. LeLéannec | Tel: Email: | firstname.lastname@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports the results of the unicity in motion information candidate list as described in JVET-J0022 [1], implemented in the JVET VTM-1.0 and the BMS-1.0. The proposed unicity check in motion information candidate list gives, in RA configuration, an average luma BD-rate gain of -0.07% in the VTM code and of -0.05% in the BMS code, for 1% encoder and 2-3% decoder complexity increase. However, in the LDB configuration, it leads to an average luma BD-rate gain of -0.01% in the VTM code and of -0.19% in the BMS code, for 1% encoder complexity increase and 4% decoder complexity increase or decrease respectively.

# Introduction

J0022 [1] proposes a unicity check of motion information candidates during each of the motion prediction list construction.

# Unicity in motion information candidate list in J0022

All the inter coding modes need to apply a process for generating the lists of candidate motion information. In the current response, these processes are homogenized and designed to ensure the full diversity and unicity of candidates.

The first generic aspect consists in unicity checking before adding any new candidate to the list. If the new candidate is already present in the list, then it is not added. This check is performed for all inter coding modes: AMVP, affine AMVP, merge/skip (and template matching merge modes).

In some inter coding modes, other processes also improve the diversity of the lists, as explained below.

- In AMVP, the unicity check is also ported into the spatial candidate collection which constructs temporary candidate list. Furthermore, in AMVP and affine AMVP, when rounding and/or clipping operations are performed, these operations are achieved as soon as a new candidate is reached. The rounded and/or clipped candidate is checked before being added or not into the list.

- In Merge, whereas in the JEM only the spatial candidate redundancy was checked, the unicity check is applied to all candidate types except the ATMVP (and STMVP) ones.

- (Candidates used to build the Template Matching and Bilateral Matching lists are the same as the ones used to build the merge list. The main difference with the merge list derivation is that, during the unicity check, the IC flag of those candidates is not considered, because it is not part of the derived/predicted motion information in these modes.)

# Results

The tool described above is implemented within the BMS-1.0 software. The following tables report simulation results using VTM and BMS configuration files, as defined in the Common Test Conditions. The same binary is used (JEM_TOOLS macros activated). All tests were performed with SIMD=SSE42 options.

## VTM

Anchor is VTM and tested is VTM with the specific test from CE4.3.6.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.05% | 0.04% | 101% | 100% |
| Class A2 | -0.06% | -0.02% | -0.01% | 101% | 104% |
| Class B | -0.14% | -0.16% | 0.01% | 101% | 100% |
| Class C | -0.05% | -0.04% | -0.14% | 101% | 103% |
| Overall | -0.07% | -0.08% | -0.03% | 101% | 102% |
| Class D | -0.04% | -0.02% | -0.06% | 101% | 105% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.09% | -0.08% | 0.10% | 100% | 102% |
| Class C | -0.03% | -0.08% | 0.14% | 101% | 92% |
| Class E | 0.15% | -0.11% | -0.21% | 101% | 93% |
| Overall | -0.01% | -0.09% | 0.04% | 101% | 96% |
| Class D | -0.07% | -0.30% | -0.05% | 100% | 102% |

## BMS

Anchor is BMS and tested is BMS with specific test from CE4.3.6.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.19% | -0.06% | 101% | 103% |
| Class A2 | -0.04% | -0.05% | 0.01% | 100% | 104% |
| Class B | -0.11% | -0.14% | -0.08% | 101% | 102% |
| Class C | -0.03% | -0.08% | -0.10% | 101% | 103% |
| Overall | -0.05% | -0.12% | -0.06% | 101% | 103% |
| Class D | -0.04% | 0.01% | -0.02% | 101% | 104% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.14% | -0.23% | 0.15% | 100% | 104% |
| Class C | -0.11% | 0.03% | -0.30% | 101% | 105% |
| Class E | -0.35% | -0.44% | 0.16% | 101% | 103% |
| Overall | -0.19% | -0.20% | 0.00% | 100% | 104% |
| Class D | -0.11% | 0.34% | -0.54% | 101% | 104% |

# References

[1] E. François, F. Le Léannec, P. Bordes, A. Robert, T. Poirier, F. Racapé, F. Galpin and al., “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – medium complexity version”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J0022, 10th Meeting, San Diego, April 2018.

[2] H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1024, 10th Meeting, San Diego, April 2018.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).