| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0211-v1 |
| --- | --- |

| Title: | CE3: DC mode without divisions and modifications to intra filtering (Tests 1.2.1, 2.2.2 and 2.5.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon Monzastr. 4c 63225 Langen Germany | Tel: Email: | +49-6103-766240 virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This document presents the results of three CE3 tests:

- CE3-1.2.1: DC prediction using only bit shifts to avoid divisions

- CE3-2.2.2: Bilinear interpolation for extension of array of reference samples

- CE3-2.5.1: same as CE3-2.2.2 + Smoothing applied after extension of array of reference samples

Is it stated that CE3-1.2.1 results show that divisions for DC prediction can be avoided with very little impact on the performances of both VTM and BMS.

It is stated that the main design purpose for CE3-2.5.1 was to implement the smoothing process in intra prediction in a simpler way without major losses in performance. It is suggested to adopt at least one approach investigated in CE3 that removes the need to perform smoothing of the neighbouring pixels prior to preparing the array of reference samples for angular prediction.

# Introduction

This contribution proposes three small changes to the current intra prediction in VTM and BMS software. These three tests are investigated in the context of CE3 [1] on intra prediction. They were originally proposed in JVET-J0020 [2].

- DC prediction using only bit shifts to avoid divisions (CE3-1.2.1)

- Improved precision for extension of array of reference samples for some of the angular prediction modes (CE3-2.2.2)

- Smoothing filter applied to the extended array of reference samples instead of the row and column of already decoded pixels surrounding the block (CE3-2.5.1)

# DC prediction without divisions (CE-1.2.1)

## Description

The DC prediction in VTM and BMS is implemented using a division:

DCvalue= x=0widthpx,-1+y=0heightp-1,y+(width+height≫1)width+height

This code was probably inherited from HM, where it was not an issue because HEVC only had square blocks for intra prediction (width = height), so that the division was equivalent to a bit shift. Unfortunately, this is not true for VTM anymore where for some blocks width ≠ height, which introduces a division for the DC mode.

It is proposed to follow the design of previous standards and avoid divisions where they can easily be avoided. It is therefore proposed to change the current calculation for the DC intra prediction mode to the following one which uses only bit shifts (see an example for a rectangular block in Figure 1):

DCvalue=x=0widthp(x,-1)+(width≫1)≫log2(width)+y=0heightp(-1,y)+(height≫1)≫log2(height)≫1

Figure 1: Example of new DC mode for a 16x4 block

This modification is applied to both luma and chroma DC prediction.

## Test results

The proposed DC prediction calculation was implemented in BMS-1.0 and tested using both VTM and BMS configurations according to the common test conditions described in JVET-J1010 [3]. Table 1 shows the results for intra only and random access configurations. The modification to DC prediction has only negligible impact on the performance of both VTM and BMS. It is therefore suggested to adopt this new DC mode calculation into VVC in order to avoid unnecessary divisions.

Table 1: Results of CE3-1.2.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | -0.02% | 99% | 99% |
| Class A2 | 0.01% | 0.08% | 0.03% | 100% | 99% |
| Class B | 0.01% | 0.01% | 0.01% | 99% | 99% |
| Class C | 0.03% | 0.02% | -0.01% | 100% | 99% |
| Class E | 0.01% | 0.08% | 0.05% | 100% | 98% |
| Overall | 0.01% | 0.04% | 0.01% | 100% | 99% |
| Class D | 0.03% | 0.02% | 0.21% | 100% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.11% | 0.05% | 100% | 101% |
| Class A2 | 0.00% | 0.07% | -0.04% | 101% | 103% |
| Class B | 0.03% | 0.06% | -0.05% | 100% | 101% |
| Class C | -0.01% | 0.11% | 0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.08% | -0.01% | 100% | 101% |
| Class D | 0.06% | 0.13% | -0.35% | 102% | 101% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 BMS config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.11% | -0.03% | 102% | 100% |
| Class A2 | 0.02% | 0.04% | -0.05% | 99% | 99% |
| Class B | 0.01% | 0.07% | 0.02% | 98% | 97% |
| Class C | 0.01% | 0.13% | 0.10% | 102% | 102% |
| Class E | -0.01% | 0.07% | 0.03% | 101% | 100% |
| Overall | 0.01% | 0.09% | 0.02% | 100% | 99% |
| Class D | 0.01% | 0.24% | 0.09% | 101% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 BMS config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.04% | 0.00% | 101% | 100% |
| Class A2 | 0.03% | 0.06% | 0.01% | 100% | 101% |
| Class B | 0.00% | 0.20% | 0.07% | 100% | 100% |
| Class C | 0.04% | 0.14% | -0.28% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.13% | -0.05% | 100% | 101% |
| Class D | 0.02% | -0.09% | 0.04% | 100% | 101% |

# Bilinear interpolation for extension of array of reference samples (CE3-2.2.2)

## Description

In HEVC, VTM and BMS, the array of reference samples is extended in case of angular intra prediction as described below (from HEVC specification):

– If intraPredAngle is less than 0, the main reference sample array is extended as follows:

– When ( nTbS * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 ][ −1 + ( ( x * invAngle + 128 )  >>  8 ) ],

with x = −1..( nTbS * intraPredAngle )  >>  5

This roughly corresponds to a projection of the left column to the upper row in case of a vertical prediction mode, i.e. when the index of the prediction mode is greater than or equal to the index of the diagonal up left prediction mode, or to a projection of the upper row to the left column in case of a horizontal prediction mode, i.e. when the index of the prediction mode is less than the index of the diagonal up left prediction mode. However, the best projected position may not correspond to an integer position as illustrated in Figure 2. It is therefore proposed to improve the projection by using bilinear interpolation. The equation above is modified as follows:

ref[x] = ( 256 – ( ( x * invAngle ) & 255 ) ) * p[-1][-1 + ( ( x* invAngle ) >> 8 ) ] + ( ( x * invAngle ) & 255 ) * p[-1][-1 + ( ( x* invAngle ) >> 8 ) + 1 ]

This modification is applied to the extension of the array of reference samples for both luma and chroma angular prediction.

Figure 2: Projection of left column to the upper row for a vertical prediction mode

## Test results

The proposed bilinear interpolation for extension of the array of reference samples was implemented in BMS-1.0 and tested using both VTM and BMS configurations according to the common test conditions described in JVET-J1010 [3]. Table 2 shows the results for intra only and random access configurations. Unfortunately, the results are rather disappointing compared to the results that were obtained using JEM-7.0 as answer to the call for proposals. The results for this proposal are therefore provided here for information only.

Table 2: Results of CE3-2.2.2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.15% | -0.13% | 100% | 99% |
| Class A2 | -0.06% | -0.09% | -0.06% | 101% | 100% |
| Class B | -0.06% | -0.08% | -0.05% | 99% | 98% |
| Class C | 0.04% | -0.01% | -0.05% | 103% | 102% |
| Class E | -0.10% | -0.11% | -0.14% | 101% | 99% |
| Overall | -0.05% | -0.08% | -0.08% | 101% | 100% |
| Class D | -0.02% | 0.03% | -0.01% | 103% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.19% | 0.06% | 101% | 100% |
| Class A2 | -0.05% | 0.07% | 0.02% | 101% | 102% |
| Class B | -0.05% | 0.07% | -0.11% | 102% | 104% |
| Class C | 0.01% | 0.05% | -0.21% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.01% | -0.08% | 101% | 102% |
| Class D | 0.01% | 0.06% | -0.12% | 101% | 100% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 BMS config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.05% | -0.11% | 100% | 99% |
| Class A2 | -0.02% | -0.04% | -0.10% | 99% | 99% |
| Class B | -0.02% | 0.05% | 0.03% | 100% | 99% |
| Class C | 0.11% | 0.08% | 0.15% | 101% | 100% |
| Class E | -0.06% | -0.03% | -0.19% | 100% | 98% |
| Overall | 0.00% | 0.01% | -0.03% | 100% | 99% |
| Class D | 0.04% | 0.02% | 0.05% | 100% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 BMS config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | -0.01% | 100% | 100% |
| Class A2 | 0.02% | 0.06% | -0.01% | 100% | 100% |
| Class B | -0.02% | -0.10% | 0.02% | 100% | 100% |
| Class C | 0.03% | 0.08% | -0.14% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | -0.03% | 100% | 101% |
| Class D | 0.04% | -0.33% | -0.27% | 100% | 100% |

# Smoothing applied after extension of array of reference samples (CE3-2.5.1)

## Description

In HEVC, VTM and BMS software, reference samples for angular intra prediction that are located to the left of and above the block are sometimes smoothed with filter [1 2 1]/4 (see Figure 3). The smoothing filter is applied or not based on the angular intra prediction mode used (MDIS). After this smoothing operation, and depending on the angular prediction mode used, an array of reference samples is prepared that is subsequently used to calculate the prediction signal for the block. This may include an extension process (see description in section 3.1 above).

Figure 3: Smoothing of reference samples in HEVC

Two modifications are proposed here: the projection process of reference samples is improved to be more accurate (same as described above in section 3.1), and the smoothing filter is applied after projection rather than before. The same smoothing filter as in HEVC is applied to the extended array of reference samples, thereby smoothing only the samples used for prediction (see Figure 4). Smoothing of the array of reference samples is applied according to the same rules as MDIS in HEVC and applies only to luma intra prediction. This proposal reduces the complexity of the smoothing process by removing the necessity to switch between rows and columns of decoded pixels, since the samples to smooth are all on one row or column.

Figure 4: Proposed smoothing after extension of the array of reference samples

## Test results

The proposed modifications to smoothing and extension of the array of reference samples were implemented in BMS-1.0 and tested using both VTM and BMS configurations according to the common test conditions described in JVET-J1010 [3]. Table 3 shows the results for intra only and random access configurations. Smoothing the reference samples after extension of the array of reference samples has disappointing results compared to the results that were obtained using JEM-7.0. However, since the complexity of the smoothing process is decreased, it is suggested to either adopt this approach in VVC, or to adopt a similar approach that consists in combining the smoothing filter together with the interpolation filters, as proposed for example in CE3-2.10.1.

Table 3: Results of CE3-2.5.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.12% | -0.13% | 100% | 99% |
| Class A2 | -0.02% | -0.08% | -0.06% | 102% | 99% |
| Class B | 0.00% | -0.13% | -0.09% | 102% | 99% |
| Class C | 0.08% | -0.05% | -0.11% | 104% | 101% |
| Class E | -0.09% | -0.17% | -0.26% | 102% | 101% |
| Overall | -0.01% | -0.11% | -0.13% | 102% | 100% |
| Class D | 0.01% | -0.03% | -0.13% | 104% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.22% | -0.10% | 100% | 100% |
| Class A2 | 0.02% | -0.07% | 0.07% | 100% | 100% |
| Class B | 0.00% | 0.02% | -0.09% | 102% | 101% |
| Class C | 0.03% | -0.09% | 0.03% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.08% | -0.03% | 101% | 101% |
| Class D | 0.02% | -0.11% | -0.25% | 101% | 100% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 BMS config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.00% | -0.12% | 101% | 99% |
| Class A2 | 0.00% | 0.02% | -0.06% | 99% | 98% |
| Class B | 0.01% | 0.01% | -0.03% | 99% | 99% |
| Class C | 0.12% | 0.17% | 0.19% | 100% | 100% |
| Class E | -0.05% | -0.18% | -0.29% | 100% | 99% |
| Overall | 0.02% | 0.01% | -0.04% | 100% | 99% |
| Class D | 0.06% | 0.16% | 0.12% | 101% | 99% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 BMS config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.12% | -0.13% | 100% | 100% |
| Class A2 | 0.04% | 0.14% | 0.01% | 100% | 100% |
| Class B | 0.00% | -0.07% | -0.13% | 100% | 99% |
| Class C | 0.10% | 0.15% | -0.17% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.02% | -0.12% | 100% | 100% |
| Class D | 0.06% | 0.10% | -0.16% | 100% | 100% |

# Conclusions

This document presents the results of three CE3 [1] tests:

- CE3-1.2.1: DC prediction using only bit shifts to avoid divisions

- CE3-2.2.2: Bilinear interpolation for extension of array of reference samples

- CE3-2.5.1: same as CE3-2.2.2 + Smoothing applied after extension of array of reference samples

CE3-1.2.1 results show that divisions can be avoided with hardly any impact on the performances of both VTM and BMS. It is therefore suggested to adopt this proposal into VVC.

CE3-2.2.2 results are rather disappointing, so that the results are only presented for information.

CE3-2.5.1 results show that the smoothing process in intra prediction can be implemented in a simpler way without major losses in performance. It is suggested to adopt either the approach presented here of applying smoothing after extension of the array of reference samples or to adopt similar proposals that combine smoothing and interpolation filters at the prediction stage, as investigated in CE3 [1].

# References

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3: Intra Prediction and Mode Coding”, JVET-J1023, JVET 10th Meeting: San Diego, US, 10–20 Apr. 2018.

- T. Toma, T. Nishi, K. Abe, et al., “Description of SDR video coding technology proposal by Panasonic”, JVET-J0020, JVET 10th Meeting: San Diego, US, 10–20 Apr. 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010, JVET 10th Meeting: San Diego, US, 10–20 Apr. 2018.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).