| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET- K0212-v1 |
| --- | --- |

| Title: | Improved Cyclic Intra Refresh |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jean-Marc Thiesse, Didier Nicholson VITEC 99 rue Pierre Semard 92320 Chatillon, France | Tel: Email: | +33 1 46 73 06 06 jean-marc.thiesse@vitec.com didier.nicholson@vitec.com |
| Source: | VITEC |  |  |

_____________________________

# Abstract

This contribution proposes a technique to improve the use of Cyclic Intra Refresh, a technique that is commonly used for real-time implementation of HEVC video encoders. Intra Refresh provides several desirable features, such as bit-rate smoothing, maintaining Intra data bit-rate under control, low delay and natural error resilience limiting temporal error propagation along frames. For video encoder’s manufacturers this feature is considered as mandatory for real-life applications.

In VVC Requirements, as contained in VCEG-BD03 and WG11/ N17074, low latency and error resilience requirements are present but have not been addressed by the responses to the CfP. One big factor of latency apart picture ordering and related dependencies, is the importance of Intra picture data amongst other, with an increasing Intra picture data size versus Inter picture data size ratio at each video codec generation. Cyclic Intra Refresh can be used for fulfilling these requirements.

But this feature, as applied by encoder manufacturer in previous standards, implies some encoding restrictions, limiting the coding efficiency, as well as Random Access capability. The proposed technique through appropriate signaling enables to leverage these limitations, allowing less coding restrictions. In this document results provided show an improvement toward classic use of Cyclic Intra Refresh together with usual LD configuration as a reference. Authors recommend considering the proposed technique for a potential inclusion in VVC future standard with further study to be conducted in an appropriate Core Experiment.

# Introduction

In VVC Requirements [1][2], low latency and error resilience requirements are present have not been addressed by the responses to the CfP. One big factor of latency apart picture ordering and related dependencies, is the importance of Intra picture data amongst other, with an increasing Intra picture data size versus Inter picture data size ratio at each video codec generation. This phenomenon is due to the fact that reached coding gain on Intra pictures is less than on Inter pictures. The bigger size of Intra picture is generating a latency issue when sent to a fixed data rate network, and furthermore can generate network congestions and packet losses. For reducing the impact of Intra picture size it is common on practical real time implementation of video encoders to use Cyclic Intra Refresh [3] to spread Intra data along the Inter frames, as depicted in the figure 1, smoothing bitrate variation. In addition to solving instantaneous high rate issue, it provides additional error robustness capabilities [4].

Figure 1: Intra Refresh principle.

This is represented on figures 2 and 3 extracted from an HEVC analyzer which respectively shows the sizes of pictures for a classical IB structure with and without Intra Refresh. The Intra pictures are in red while B pictures are in green. In figure 2, the third I pictures is 8 times higher than following B pictures taking much more time to be send and requiring a bigger buffer which increase the latency. In figure 3, using Cyclic Intra Refresh, the picture size is homogenous and the latency is reduced.

Figure 2: Size of picture in a classical IB structure.

Figure 3: Size of picture in a classical IB structure with Intra Refresh.

But Intra Refresh, due to many restrictions in coding, decreases coding efficiency. In HEVC, this limitation is due to the fact that no particular normative signaling is used together with Intra coded CUs, enabling a more efficient use of the coding tools.

This document proposes an improved Cyclic Intra Refresh method, including some signalling that allows keeping good coding efficiency.

# Proposed Improved Intra Refresh

## Description of the method

The proposed solution adds the minimum signalization needed to determinate the position of Intra Refreshed pixels and then exploits this position knowledge to improve the quality within this low-latency use-case. The modifications at the encoder side involved:

- Avoid the transmission of pred_mode_flag for Coding Unit with at least one pixel within the Intra Refresh Area.

- Forcing splits of CU to reach the Intra Refresh Area.

- Enable the Constrained Intra Prediction (CIP) tool adaptively depending on the position and direction of the Intra Refresh Area.

- Take care of the Intra Refresh position for the motion vectors derivation.

- Define two deltas QP by slice depending on the Intra Refresh Area position.

## Signalization

The proposed additional signalization is:

- at PPS level:

  - intra_refresh_enabled_flag

  - intra_refresh_mode

  - intra_refresh_size

  - intra_refresh_delta_qp

- at Slice level:

  - intra_refresh_direction

  - intra_refresh_position

Figure 4: Proposed signalization and modification.

# Results

## Intra Refresh against Reference

## Normative Intra Refresh

# Recommendation

In this document, a method for an improved Cyclic Intra Refresh method was proposed, including some signalling that allows keeping good coding efficiency. It is recommended to consider it for a potential inclusion in VVC future standard with further study to be conducted in an appropriate Core Experiment.

# Acknowledgement

The research leading to this work has been supported by the FUI23 EFIGI project, BPIFrance and Region Ile de France.

# References

- “Requirements for Future Video Coding (FVC)”, ITU-TSG16/Q6 VCEG56th meeting, Torino, Italy, July2017, Doc. VCEG-BD03 (available at http://wftp3.itu.int/av-arch/video-site/1707_Tor/).

- “Requirements for a Future Video Coding Standard v5”, ISO/IEC JTC1/SC29/WG11 MPEG, 119th meeting, Torino, Italy, July 2017, Doc.N17074(available at http://mpeg.chiariglione.org/meetings/119).

- Tri D. Tran, Lurng-Kuo Liu, Peter H. Westering: “Low-delay MPEG-2 video coding”, Proc. SPIE - Int. Soc. Opt. Eng. (USA), USA: SPIE-Int. Soc. Opt. Eng, vol.3309, 1997 pp. 510-16.

- Haoming Chen, Chen Zhao, Ming-Ting Sun a, Aaron Drake, “Adaptive intra-refresh for low-delay error-resilient video coding”, J. Vis. Commun. Image R. 31 (2015) 294–304

# Patent rights declaration(s)

VITEC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).