| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0213-v1 |
| --- | --- |

| Title: | CE10.2.3 Generalized OBMC from J0022 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | A. Robert, T. Poirier, F. LeLéannec | Tel: Email: | firstname.lastname@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports the results of generalized OBMC as described in JVET-J0022 [1], implemented in the JVET VTM and the BMS.

# Introduction

J0022 [1] proposes a new version of OBMC which is generic and allows better adaptations to the partitioning.

# Generalized OBMC in J0022

The OBMC process is carried out with 4x4 sub-blocks. In the proposed solution, it has been unified so that the decision to filter 2 or 4 lines/columns of pixels is no more taken a-priori depending on the CU coding mode, but on-the-fly depending on motion vectors located at opposite 4x4 sub-block borders.

Instead of performing OBMC successively on left, above, right then bottom sub-block borders, these borders are considered by pairs: horizontal orientation (left, right) and vertical orientation (top, bottom). For each 4x4 sub-block, motion vectors corresponding to the pair of borders in the considered orientation are retrieved. If both are available and different from the motion vector of the current 4x4 sub-block, then OBMC filtering is performed on 2 lines/columns of pixels from each border. If only one is available or if only one is different from the motion vector of the current sub-block, OBMC filtering is achieved on 4 lines/columns of pixels on the border where the motion vector is different. If none is available or both motion vectors are equal to the current sub-block motion vector, no filtering is applied.

This proposed unified OBMC allows filtering a different number of samples for horizontal and vertical directions and having a better adaptation to motion transitions within a CU, in particular with merge ATMVP CUs as illustrated in Figure 1.

Figure 1. a) prior art OBMC and b) proposed unified OBMC applied on a 32x16 CU in Merge ATMVP mode
(dashed lines denote the splits retrieved from ATMVP, i.e. the motion transitions).

Furthermore, the OBMC process in the JEM limits filtering to 2 lines/columns for small CUs whose surface is lower than 64 samples. In the proposed unified OBMC, since the filtering size is decoupled between orientations, this limitation has been adapted. If the width of the CU is less than 8, only 2 columns of pixels are filtered. If the height is less than 8, only 2 lines of pixels are filtered. This allows a better adaptation of the OBMC filtering for thin CUs as shown in Figure 2.

Figure 2. OBMC applied on a 32x4 CU a) as achieved in JEM and b) as in proposed unified OBMC.

# Results

All tests are done with SIMD=SSE42 options and VTM/BMS 1.0.

## VTM

Anchor is VTM and tested is VTM with OBMC and he specific test from CE10.2.3.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.41% | -1.65% | -1.91% | 109% | 117% |
| Class A2 | -1.04% | -1.98% | -1.88% | 113% | 129% |
| Class B | -0.96% | -1.89% | -1.83% | 114% | 123% |
| Class C | -1.50% | -2.54% | -2.70% | 117% | 130% |
| Overall | -1.01% | -2.03% | -2.09% | 114% | 125% |
| Class D | -1.43% | -2.05% | -2.04% | 122% | 137% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -1.24% | -1.83% | -1.93% | 117% | 128% |
| Class C | -1.67% | -2.18% | -2.32% | 119% | 119% |
| Class E | -1.31% | -1.77% | -1.95% | 115% | 111% |
| Overall | -1.40% | -1.93% | -2.07% | 117% | 120% |
| Class D | -1.92% | -2.18% | -1.67% | 120% | 136% |

## BMS

Anchor is BMS and tested is BMS with OBMC and specific test from CE10.2.3.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.48% | -1.16% | -1.38% | 104% | 120% |
| Class A2 | -1.22% | -2.19% | -2.73% | 107% | 138% |
| Class B | -1.22% | -2.29% | -2.20% | 107% | 132% |
| Class C | -1.83% | -2.96% | -3.05% | 107% | 134% |
| Overall | -1.24% | -2.22% | -2.37% | 107% | 131% |
| Class D | -1.87% | -2.95% | -2.92% | 108% | 137% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -1.51% | -2.73% | -2.49% | 108% | 135% |
| Class C | -2.18% | -3.20% | -3.34% | 108% | 138% |
| Class E | -2.33% | -3.11% | -2.48% | 109% | 134% |
| Overall | -1.94% | -2.98% | -2.77% | 109% | 136% |
| Class D | -2.54% | -2.55% | -2.77% | 111% | 148% |

# Conclusion

The results show the advantages of the proposed generalized OBMC. In VTM code, it gives -1.01% bdrate gain in RA configuration and it leads to -1.40% bdrate gain in LDB configuration, for 14% and 17% encoder complexity increase, and 25% and 20% decoder complexity increase respectively. In BMS code, it gives -1.24% bdrate gain in RA configuration and leads to -1.94% bdrate gain in LDB configuration, for 7% and 9% encoder complexity increase, and 31% and 36% decoder complexity increase respectively.

# References

[1] E. François, F. Le Léannec, P. Bordes, A. Robert, T. Poirier, F. Racapé, F. Galpin and al., “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – medium complexity version”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J0022, 10th Meeting, San Diego, April 2018.

[2] Y.-W. Huang, C.-W. Hsu, M. Winken, X. Xiu, “Description of Core Experiment 10 (CE10): Combined and multi-hypothesis prediction”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1030, 10th Meeting, San Diego, April 2018.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).