| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0214-v1 |
| --- | --- |

| Title: | CE9.3.4 Template matching from J0022 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | A. Robert, T. Poirier, F. LeLéannec | Tel: Email: | firstname.lastname@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports the results of Template matching described in JVET-J0022 [1], implemented in the JVET VTM and the BMS.

# Introduction

J0022 [1] proposes a modified version of the Template Matching proposed in J0021 ([2] 8.3.3).

First, the candidates, called template matching unilateral motion vectors, are completely removed. This reduces complexity at the encoder and decoder sides.

Then, in AMVP, instead of limiting the studied candidates to the AMVP ones, other candidates as the merge ones and the neighboring spatial candidates are considered. The number of candidates in that case is restricted to two to limit the complexity impact.

# Template matching in J0022

## Unilateral motion vectors removal

In the JEM, the candidates, called template matching unilateral motion vectors, are always constructed at the beginning of each slice. In the proposed solution, these candidates are completely removed, i.e. no more constructed nor used.

## AMVP candidate limitation

In the AMVP process described in the proposed solution, the studied candidates in template matching are restricted to two (2AMVP, 1AMVP+1other, or 2others) to limit the complexity. And, since original AMVP candidates are considered by the template matching, it is possible to assume that the best MVP candidate is the FRUC refined one. In that case, the single considered MVP candidate in the RDO process is the FRUC one and the MVP index has then not to be transmitted.

# Results

All tests are done with SIMD=SSE42 options and VTM/BMS 1.0.

## VTM

Anchor is VTM and tested is VTM with FRUC tool from BMS activated and the specific test from CE9.3.4.

### Test 9.3.4 A

Unilateral motion vectors removal 9.3.4 a).

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -6.50% | -5.97% | -6.60% | 157% | 259% |
| Class A2 | -8.61% | -8.14% | -8.43% | 164% | 321% |
| Class B | -5.76% | -5.23% | -5.13% | 170% | 279% |
| Class C | -5.27% | -5.32% | -5.64% | 181% | 329% |
| Overall | -6.35% | -5.98% | -6.22% | 169% | 295% |
| Class D | -5.79% | -5.27% | -5.53% | 192% | 358% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -2.56% | -2.74% | -2.54% | 221% | 229% |
| Class C | -2.60% | -3.16% | -3.58% | 229% | 240% |
| Class E | -3.87% | -3.83% | -4.35% | 202% | 208% |
| Overall | -2.90% | -3.15% | -3.34% | 219% | 227% |
| Class D | -2.75% | -3.22% | -3.65% | 228% | 274% |

### Test 9.3.4 B

Template matching candidates limited to 2 in AMVP.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -6.69% | -6.16% | -6.81% | 168% | 292% |
| Class A2 | -8.90% | -8.41% | -8.74% | 179% | 357% |
| Class B | -5.95% | -5.48% | -5.41% | 176% | 306% |
| Class C | -5.57% | -5.63% | -5.89% | 183% | 351% |
| Overall | -6.59% | -6.24% | -6.48% | 177% | 324% |
| Class D | -6.14% | -5.65% | -5.83% | 194% | 377% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -3.04% | -3.12% | -3.07% | 230% | 285% |
| Class C | -3.14% | -3.74% | -3.98% | 230% | 276% |
| Class E | -4.26% | -4.15% | -4.47% | 212% | 291% |
| Overall | -3.38% | -3.58% | -3.72% | 225% | 283% |
| Class D | -3.44% | -3.98% | -4.50% | 229% | 308% |

### Test 9.3.4 C

Test a) and b) together.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -6.65% | -6.05% | -6.78% | 160% | 259% |
| Class A2 | -8.89% | -8.38% | -8.72% | 166% | 320% |
| Class B | -5.95% | -5.51% | -5.34% | 172% | 279% |
| Class C | -5.53% | -5.62% | -5.86% | 182% | 329% |
| Overall | -6.57% | -6.22% | -6.44% | 171% | 295% |
| Class D | -6.12% | -5.55% | -5.83% | 192% | 357% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -3.02% | -3.12% | -3.27% | 222% | 229% |
| Class C | -3.08% | -3.53% | -4.16% | 228% | 239% |
| Class E | -4.42% | -4.60% | -4.76% | 203% | 209% |
| Overall | -3.39% | -3.62% | -3.94% | 219% | 227% |
| Class D | -3.41% | -3.56% | -4.37% | 229% | 274% |

## BMS

Anchor is BMS and tested is BMS with FRUC tool and the specific test from CE9.3.4.

### Test 9.3.4 A

Unilateral motion vectors removal 9.3.4 a).

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.19% | -3.10% | -3.55% | 126% | 156% |
| Class A2 | -3.94% | -3.74% | -3.94% | 131% | 165% |
| Class B | -3.23% | -2.98% | -2.91% | 133% | 167% |
| Class C | -3.56% | -3.58% | -3.78% | 136% | 200% |
| Overall | -3.45% | -3.32% | -3.48% | 132% | 173% |
| Class D | -3.86% | -3.71% | -3.80% | 138% | 214% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -2.34% | -2.94% | -3.33% | 173% | #NUM! |
| Class E | -2.90% | -2.29% | -2.42% | 158% | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -2.28% | -1.98% | -2.69% | 179% | #NUM! |

### Test 9.3.4 B

Template matching candidates limited to 2 in AMVP.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.33% | -3.21% | -3.73% | 130% | 172% |
| Class A2 | -4.21% | -4.01% | -4.23% | 139% | 182% |
| Class B | -3.41% | -3.22% | -3.13% | 135% | 183% |
| Class C | -3.81% | -3.85% | -3.98% | 137% | 213% |
| Overall | -3.66% | -3.55% | -3.70% | 135% | 188% |
| Class D | -4.12% | -4.03% | -4.21% | 139% | 222% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -2.72% | -3.44% | -3.95% | 173% | #NUM! |
| Class E | -3.28% | -2.36% | -2.64% | 163% | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -2.75% | -2.98% | -3.21% | 179% | #NUM! |

### Test 9.3.4 C

Test a) and b) together.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.31% | -3.16% | -3.71% | 127% | 155% |
| Class A2 | -4.18% | -3.96% | -4.20% | 132% | 166% |
| Class B | -3.40% | -3.21% | -3.15% | 134% | 168% |
| Class C | -3.81% | -3.84% | -3.93% | 136% | 201% |
| Overall | -3.65% | -3.52% | -3.68% | 133% | 173% |
| Class D | -4.13% | -3.86% | -4.11% | 139% | 216% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -2.74% | -3.21% | -3.62% | 173% | #NUM! |
| Class E | -3.44% | -2.89% | -2.69% | 158% | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -2.85% | -2.81% | -4.08% | 179% | #NUM! |

# Conclusion

The results show the advantages of the proposed unilateral candidates removal and AMVP candidates limitation. For the VTM code, it gives -6.57% bdrate gain in RA configuration and leads to -3.39% bdrate gain in LDB configuration, for 71% and 119% encoder complexity increase, and 195% and 127% decoder complexity increase respectively. In BMS code, it gives -3.65% bdrate gain in RA and leads to -3.xx% bdrate gain in LDB, for 33% and xx% encoder complexity increase, and 73% and xx% decoder complexity increase respectively.

# References

[1] E. François, F. Le Léannec, P. Bordes, A. Robert, T. Poirier, F. Racapé, F. Galpin and al., “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – medium complexity version”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J0022, 10th Meeting, San Diego, April 2018.

[2] S. Esenlik, Y. W. Chen, H. Yang, “Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1029, 10th Meeting, San Diego, April 2018.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).