| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0217_v1 |
| --- | --- |

| Title: | CE9: DMVR with Bilateral Matching (Test2.9) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Ivan Krasnov, Zhijie Zhao, Ma Xiang, Haitao Yang, Jianle Chen (Huawei), Yue Li (USTC) | Email: | semih.esenlik@huawei.com ivan.krasnov@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. University of Science and Technology of China (USTC) |  |  |

_____________________________

# Abstract

A decoder side motion vector refinement (DMVR) method using bilateral matching is proposed, which is evaluated in CE9. In the proposed method a search pattern with less calculation is used and mean removed sum of absolute differences (MRSAD) is used as the matching cost. In the contribution document experimental results for search range as 1, 2 and 4 samples are presented with and without half-pel search. Moreover supplementary results are provided where the spatial MV prediction from refined MV is disabled.

According to one of the configurations of the proposal (with search range of 2 samples and maximum 13 MRSAD computations), -4.59% luma coding gain with 107%, encoding time and 132% decoding time over VTM Anchor (RA), and -1.29% luma coding gain with 98%, encoding time and 101% decoding time over BMS Anchor (RA) is achieved.

According to the supplementary configuration (with search range of 2 samples and maximum 13 MRSAD computations), when refined motion vectors from spatial predictors are disabled, -2.80% luma coding gain with 109%, encoding time and 132% decoding time; over VTM Anchor (RA), and -0.3% luma coding gain with 100%, encoding time and 102% decoding time over BMS Anchor (RA) is achieved

# Introduction

The proposal presents a method of motion vector refinement by calculation of MRSAD between predictions in both reference pictures, located at opposite sides of the current picture, without any template construction. During integer precision search only 6 points (instead of 9 in DMVR JEM) are checked, which are chosen according to an adaptive search pattern method. After integer search process is completed the half sample precision search is to be done if half-pel search is enabled. In the half sample search only 4 points are to be checked (in plus shape).

# Proposed Method

In the proposed method DMVR is applied only in Merge and Skip modes, if the following condition is true:

(POC - POC0)*(POC - POC1) < 0,

where POC – picture order count of current to be encoded picture, POC0 and POC1 – picture order counts of the references for the current picture.

The signaled merge candidate pair is used as input to DMVR process and are denoted initial motion vectors (MV0, MV1). The search points that are searched by DMVR obey the motion vector difference mirroring condition. In other words any point that is checked by DMVR, denoted by candidate motion vector pair (MV0’, MV1’) obey the following two equations:

MV0'=MV0+MVdiff

MV1'=MV1-MVdiff

Where MVdiff represents the points in the search space in one of the reference pictures.

After the construction of the search space the uni-lateral predictions are constructed using regular 8-tap DCTIF interpolation filter. Bilateral matching cost function is calculated by using MRSAD between the two predictions (Fig. 1) and the search point resulting in the minimum cost is selected as the refined MV pair. For the MRSAD calculation 16 bit precision of samples is used (which is the output of the interpolation filtering), and no clipping and no rounding operations are applied before MRSAD calculation. The reason for not applying rounding and clipping is to reduce internal buffer requirement.

Fig. 1: bilateral matching with 6 points search

In the proposed method the integer precision search points are chosen by the Adaptive pattern method.  The cost, corresponding to the central points (pointed by the initial motion vectors) is calculated firstly. The other 4 costs (in sign shape) is calculated by the two predictions, located at the opposite sides of each other by the central point. Last 6th point at the angle is chosen by the gradient of the previous calculated costs (Fig. 2).

Fig. 2: adaptive integer search pattern (left), half sample search pattern (right)

The output of the DMVR process is the refined motion vector pair corresponding to the minimal cost.

If after one iteration the minimum cost is achieved at the central point of the search space, i.e. the motion vectors are not changed, and the refinement process is terminated. Otherwise, the best cost further is regarded as center, and the process continues, while the minimal cost does not correspond to the central point and the search range is not exceeded.

Half sample precision search is applied only if application of half-pel search does not exceed the search range. In this case only 4 MRSAD calculations are performed, corresponding to plus shape points around the central one, which is chosen as the best during the integer precision search. At the end the refined motion vector pair is output that correspond to the minimal cost point.

# Comments about complexity

In the proposed method the MRSAD computations are applied on the output of the unilateral predictions from refPic0 and refPic1 directly, which are 16 bit in precision. After the refined motion vector is determined, the final prediction is obtained by merely averaging the unilateral predictions that are already generated. The rounding and clipping operations are therefore delayed to final weighted averaging stage in order not to increase internal buffer requirement.

# Experiment results

## CE9.2.9 with refined motion vectors for all predictors

Table 1: Experiment results of proposed method in CE9.2.9 with refined motion vectors for all predictors

| VTM |  |  |  |  | BMS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Max # of MRSAD calculation | Max. Search Range | Half Pel |
| -3.18% | -2.95% | -3.09% | 103% | 115% | -0.38% | -0.33% | -0.35% | 97% | 96% | 6 | 1 sample | Off |
| -3.68% | -3.17% | -3.33% | 106% | 124% | -0.68% | -0.48% | -0.52% | 98% | 98% | 10 | 1 sample | On |
| -3.80% | -3.57% | -3.70% | 105% | 119% | -0.79% | -0.73% | -0.78% | 99% | 97% | 9 | 2 samples | Off |
| -4.59% | -3.97% | -4.14% | 107% | 132% | -1.29% | -1.02% | -1.10% | 98% | 101% | 13 | 2 samples | On |
| -4.07% | -3.86% | -4.03% | 105% | 121% | -0.97% | -0.96% | -0.98% | 99% | 97% | 15 | 4 samples | Off |
| -4.92% | -4.31% | -4.50% | 109% | 136% | -1.50% | -1.28% | -1.35% | 99% | 102% | 19 | 1 sample | On |

## CE9.2.9 for non-refined motion vectors of spatial candidates in a grid 32x32 samples

Table 2: Experiment results of proposed method in CE9.2.9 with non-refined motion vectors as spatial candidates in a grid 32x32 samples

| VTM |  |  |  |  | BMS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Max # of MRSAD calculation | Max. Search Range | Half Pel |
| -2.72% | -2.48% | -2.56% | 103% | 118% | -0.18% | -0.06% | -0.05% | 98% | 97% | 6 | 1 sample | Off |
| -3.11% | -2.67% | -2.76% | 105% | 125% | -0.41% | -0.22% | -0.21% | 99% | 100% | 10 | 1 sample | On |
| -3.38% | -3.13% | -3.23% | 106% | 121% | -0.59% | -0.49% | -0.50% | 99% | 98% | 9 | 2 samples | Off |
| -4.08% | -3.47% | -3.62% | 109% | 133% | -1.03% | -0.78% | -0.78% | 100% | 101% | 13 | 2 samples | On |
| -4.47% | -3.88% | -4.02% | 111% | 138% | -1.28% | -1.04% | -1.06% | 100% | 103% | 19 | 4 samples | On |

## Non-refined motion vectors of spatial candidates in a whole frame

Table 3: Experiment results of proposed method in CE9.2.9 with non-refined motion vectors as spatial candidates in a whole frame

| VTM |  |  |  |  | BMS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Max # of SAD calculation | Max. SR | Half Pel |
| -2.80% | -2.57% | -2.63% | 109% | 132% | -0.30% | -0.23% | -0.20% | 100% | 102% | 13 | 2 samples | On |

The proposed method is implemented top of the VTM 1.0 and BMS 1.0 software. The experiment results were obtained under the common test conditions. The results of VTM 1.0 software were obtained by switching DMVR tool on in the config file.

The complete results are provided as attachment to this document and are uploaded under the svn repository:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE9/CE9-2.9/

# Conclusion

A bilateral matching based DMVR method using adaptive search pattern and half-pel sample precision search in CE9.2.9 is proposed to achieve better performance and less complexity in DMVR process.

According to one of the configurations of the proposal (with search range of 2 samples and maximum 13 MRSAD computations), -4.59% luma coding gain with 107%, encoding time and 132% decoding time; over VTM Anchor (RA), and -1.29% luma coding gain with 98%, encoding time and 101% decoding time over BMS Anchor (RA) is achieved. The contribution is proposed to be adopted in the VVC.

# References

[1] JVET common test conditions and software reference configurations, Jill Boyce, doc. JVET-J1010-v1, April 2018

[2] Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation, Semih Esenlik, doc. JVET-J1021, April 2018

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

University of Science and Technology of China, Peking University, Harbin Institute of Technology, and Wuhan University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).