| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0218-v2 |
| --- | --- |

| Title: | CE4.1.4 Affine mode enhancement from J0022 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | A. Robert, T. Poirier, F. LeLéannec | Tel: Email: | firstname.lastname@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports the results of the Affine mode described in JVET-J0022 [1], implemented in the JVET VTM-1.0 and the BMS-1.0. The proposed affine MVP candidates and affine model storage method (test 4.1.4 D), for the VTM code, gives an average luma BD-rate gain of -3.91% and up to -8.72% for affine classes in RA configuration, for 48% and 53% encoder and 14-15% decoder complexity increase. In the BMS code, it leads to an average luma BD-rate gain of -2.72% and up to -7.82% for affine classes in RA, for 13% and 20% encoder, and 6 and 9% decoder complexity increase.

# Introduction

J0022 [1] proposes a modified version of the Affine motion vector prediction proposed in J0021 described in CE4.1.3 [2] (2.2.6 and 2.2.7).

The affine model seeds are stored at the CU level allowing merge mode to be applied on CU of size 4 in width or height.

Two further spatial affine positions are explored to retrieve affine model for the current CU. These neighbors are {a2, b3} as presented on Figure 1 and associated derived affine model, if exist, are inserted after the {b2} affine candidate.

The complete list of MVP candidates is presented in Table 2.

# Affine motion vector prediction adaptations in J0022

## Shared modifications

All tests of the proposed CE4.1.4 share three modifications:

- A fix of the divisions by zero in the affine motion estimation.

- An affine cleaning that uses only three specific functions to derive affine models (derivation from a neighboring affine model, computations of the third affine model seed from the two first ones, or of the second one from the first and last ones).

- The affine model derivations are all performed in integer precision (instead of double).

Following Table 1 shows that these shared modifications have a negligible impact on the BD-rates and associated complexity.

Table 1 Results of the three shared modifications (one intra period).

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class C | 0.05% | -0.01% | 0.16% | 100% | 99% |
| Class D | -0.06% | -0.12% | -0.18% | 100% | 100% |
| Overall | 0.00% | -0.06% | -0.01% | 100% | 99% |

## Affine model storage

In the JEM, the affine model seeds are stored in the top-left, top-right and bottom-left 4x4 sub-blocks in the considered CU. In the proposed solution, the affine model seeds are stored separately as a motion information associated to the whole CU. The motion model is thus decoupled from the motion vectors used for actual motion compensation at the 4x4 block level. This storage allows preserving the complete motion vector field at the 4x4 sub-block level. It also allows using affine motion compensation for block of size 4 in width or height.

## Affine MVP candidate derivation

Table 2 Affine AMVP mode characteristics.

|  | JEM | Proposed |
| --- | --- | --- |
| Motion information predictor composition | inherited MV triplet | inherited MV triplet |
| Candidate list | 2 candidates, among: - constructed affine models from spatial neighbors - classical AMVP candidates | 2 candidates, among: - spatial Affine positions {b1, a1, a0, b0, b2, a2, b3} - improved constructed affine models from spatial neighbors - classical AMVP candidates - zero MVs |
| Signaling | For each reference picture: - reference picture idx - predictor idx - 2 MVDs Affine flag Residual is coded | For each reference picture: - reference picture idx - predictor idx - 2 MVDs Affine flag Residual is coded |

In the MVP process described in J0021, a candidate list is created as presented in Table 1. In the proposed solution, the affine candidates are enriched with new ones. The spatial affine candidates set in J0021 {a1, b1, a0, b0, b2} becomes {a1, b1, b0, a0, b2, a2, b3} in this contribution.

Figure 1. Positions of spatial candidates of motion information.

# Results

The tools described above are implemented within the BMS-1.0 software. The following tables report simulation results using VTM and BMS configuration files, as defined in the Common Test Conditions. The same binaries are used (JEM_TOOLS macros activated). All tests were performed with SIMD=SSE42 options. The test 4.1.4 is added on top of 4.1.3 (affine motion prediction, with 4-6 parameters model, slice switching).

## VTM

Anchor is VTM and tested is VTM with affine and high precision motion vector tool from BMS activated, CE4.1.3 tools and the specific test from CE4.1.4.

### Test 4.1.4 A

Added affine model storage 4.1.4 a).

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.61% | -2.20% | -2.30% | 135% | 111% |
| Class A2 | -8.22% | -6.67% | -6.33% | 161% | 127% |
| Class B | -4.02% | -2.79% | -2.95% | 152% | 114% |
| Class C | -1.46% | -0.92% | -1.02% | 144% | 109% |
| Overall | -3.90% | -2.95% | -2.98% | 148% | 115% |
| Class D | -1.85% | -1.11% | -1.22% | 139% | 113% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -2.90% | -2.11% | -1.86% | 210% | 116% |
| Class C | -1.52% | -0.58% | -0.48% | 178% | 100% |
| Class E | -2.64% | -2.17% | -2.63% | 276% | 103% |
| Overall | -2.38% | -1.61% | -1.59% | 213% | 107% |
| Class D | -2.93% | -1.96% | -2.15% | 157% | 113% |

### Test 4.1.4 B

Add 2 new affine spatial candidates in AMVP.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.59% | -2.14% | -2.31% | 134% | 110% |
| Class A2 | -8.22% | -6.69% | -6.33% | 160% | 128% |
| Class B | -4.02% | -2.74% | -2.89% | 151% | 114% |
| Class C | -1.45% | -0.95% | -0.98% | 144% | 110% |
| Overall | -3.89% | -2.93% | -2.95% | 148% | 115% |
| Class D | -1.84% | -1.01% | -1.24% | 140% | 112% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -2.92% | -2.03% | -2.05% | 210% | 116% |
| Class C | -1.51% | -0.65% | -0.59% | 180% | 100% |
| Class E | -2.62% | -2.00% | -2.41% | 275% | 103% |
| Overall | -2.37% | -1.56% | -1.65% | 213% | 107% |
| Class D | -2.92% | -1.69% | -2.23% | 157% | 112% |

### Test 4.1.4 C

This test has been withdrawn.

### Test 4.1.4 D

Test a) and b) together. (Instead of test a), b) and c) together, since test c) has been withdrawn).

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.59% | -2.18% | -2.28% | 136% | 110% |
| Class A2 | -8.24% | -6.74% | -6.40% | 160% | 128% |
| Class B | -4.04% | -2.82% | -2.94% | 151% | 114% |
| Class C | -1.49% | -0.87% | -1.06% | 144% | 110% |
| Overall | -3.91% | -2.96% | -3.00% | 148% | 115% |
| Class D | -1.86% | -1.24% | -1.22% | 139% | 114% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -2.91% | -2.03% | -2.16% | 211% | 117% |
| Class C | -1.51% | -0.64% | -0.50% | 178% | 101% |
| Class E | -2.69% | -2.10% | -2.76% | 277% | 103% |
| Overall | -2.39% | -1.59% | -1.75% | 213% | 108% |
| Class D | -2.96% | -1.83% | -2.30% | 157% | 114% |

### Test 4.1.4 D on affine class

The affine class is the same as in [1], except for the Shields sequence (see attached excel files).

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Affine 1080p | -4.40% | -1.86% | -1.71% | 125% | 114% |
| Affine 720p | -15.19% | -11.28% | -11.18% | 206% | #NUM! |
| Overall | -8.72% | -5.63% | -5.50% | 153% | #NUM! |

## BMS

Anchor is BMS without affine tool and tested is BMS with CE4.1.3 tools and the specific test from CE4.1.4.

### Test 4.1.4 A

Added affine model storage 4.1.4 a).

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.61% | -1.20% | -1.28% | 110% | 106% |
| Class A2 | -5.60% | -4.25% | -4.11% | 118% | 108% |
| Class B | -2.82% | -2.04% | -2.03% | 114% | 106% |
| Class C | -1.26% | -0.88% | -0.97% | 112% | 107% |
| Overall | -2.72% | -2.01% | -2.02% | 113% | 107% |
| Class D | -1.69% | -1.49% | -1.13% | 109% | 108% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 without affine |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -3.03% | -2.15% | -2.19% | 138% | 109% |
| Class C | -1.52% | -0.97% | -1.43% | 129% | 110% |
| Class E | -2.47% | -2.70% | -2.72% | 170% | 108% |
| Overall | -2.39% | -1.89% | -2.07% | 142% | 109% |
| Class D | -2.99% | -2.61% | -3.05% | 124% | 113% |

### Test 4.1.4 B

Add 2 new affine spatial candidates in AMVP.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.58% | -1.24% | -1.33% | 110% | 105% |
| Class A2 | -5.59% | -4.18% | -4.10% | 118% | 109% |
| Class B | -2.81% | -2.02% | -2.01% | 114% | 105% |
| Class C | -1.25% | -0.98% | -1.08% | 112% | 106% |
| Overall | -2.71% | -2.02% | -2.05% | 113% | 106% |
| Class D | -1.71% | -1.47% | -1.27% | 110% | 107% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 without affine |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -3.01% | -2.09% | -2.50% | 139% | 109% |
| Class C | -1.55% | -1.21% | -0.96% | 129% | 110% |
| Class E | -2.48% | -2.83% | -2.66% | 171% | 111% |
| Overall | -2.39% | -1.98% | -2.03% | 143% | 110% |
| Class D | -2.99% | -2.65% | -2.88% | 125% | 110% |

### Test 4.1.4 C

This test has been withdrawn.

### Test 4.1.4 D

Test a) and b) together.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 without affine |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.59% | -1.26% | -1.31% | 110% | 105% |
| Class A2 | -5.61% | -4.25% | -4.10% | 118% | 108% |
| Class B | -2.83% | -1.94% | -2.06% | 113% | 105% |
| Class C | -1.27% | -0.98% | -0.92% | 112% | 105% |
| Overall | -2.72% | -2.01% | -2.01% | 113% | 106% |
| Class D | -1.74% | -1.38% | -1.19% | 109% | 105% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 without affine |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -3.03% | -2.17% | -2.03% | 139% | 109% |
| Class C | -1.54% | -1.11% | -1.22% | 129% | 108% |
| Class E | -2.57% | -2.64% | -2.37% | 170% | 107% |
| Overall | -2.42% | -1.93% | -1.85% | 142% | 108% |
| Class D | -3.11% | -2.60% | -3.01% | 124% | 109% |

### Test 4.1.4 D on affine class

The affine class is the same as in [1], except for the Shields sequence (see attached excel files).

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 without affine |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Affine 1080p | -4.39% | -2.10% | -1.94% | 114% | 109% |
| Affine 720p | -12.96% | -9.81% | -10.41% | 129% | 110% |
| Overall | -7.82% | -5.18% | -5.33% | 120% | 109% |

# References

[1] E. François, F. Le Léannec, P. Bordes, A. Robert, T. Poirier, F. Racapé, F. Galpin and al., “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – medium complexity version”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J0022, 10th Meeting, San Diego, April 2018.

[2] H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1024, 10th Meeting, San Diego, April 2018.

[3] “Affine Transform prediction for next generation video coding”, COM16-C1016R1, ITU, Sept. 2015.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).