| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0219-v1 |
| --- | --- |

| Title: | CE4.2.4 Merge mode enhancement from J0022 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | F. Galpin , A. Robert, T. Poirier, F. Leleannec | Tel: Email: | firstname.lastname@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports the results of Merge mode enhancement, described in JVET-J0022, implemented within the JVET VTM-1.0 and BMS-1.0. For test 4.2.4C, simulation results reportedly show an average luma BD-rate gain of -4.08%, resp. -2.67%, in Random Access (RA), resp. Low-delay (LDB) configuration, compared with TM, and -2.86%, resp. -2.57%, in RA, resp. LDB configuration, compared to BMS. Moreover, additional tests on affine class sequences show an average luma BD-rate gain in RA of -11.75% for TM, resp. -xxxx%, for BMS.

# Introduction

J0022 proposes a modified version of the Merge/Skip mode proposed in J0021 (2.9) [1].

The affine model seeds are stored at the CU level as presented in J0022 [2] “2.1.7.4.1	Affine Motion Compensation adaptations”.

Two further spatial affine positions are explored to retrieve affine model for the current CU. These neighbors are {a2, b3} as presented on Figure 1 and associated derived affine model, if exist, are inserted after the {b2} affine candidate.

Furthermore, several temporal affine candidates are also added to the merge candidate list. The derivation of such temporal affine models that are get from {curr, c} positions is described in J0022. These temporal affine candidates are inserted after the classical temporal candidate.

The complete list of merge candidates (normal/affine) is presented in Table 1.

# Affine motion vector prediction adaptations in J0022

## Affine model storage

In the JEM, the affine model seeds are stored in the top-left, top-right and bottom-left 4x4 sub-blocks in the considered CU. In the proposed solution, the affine model seeds are stored separately as a motion information associated to the whole CU. The motion model is thus decoupled from the motion vectors used for actual motion compensation at the 4x4 block level. This storage allows preserving the complete motion vector field at the 4x4 sub-block level. It also allows using affine motion compensation for block of size 4 in width or height.

## Affine candidate derivation

Table 1 Merge mode characteristics.

|  | JEM | Proposed |
| --- | --- | --- |
| Motion information predictor composition | - inherited reference picture index - inherited associated MV - inherited associated IC flag | - inherited reference picture index - inherited associated MV (triplet) - inherited associated IC flag - inferred affine flag |
| Candidate list | 7 candidates, among: - spatial positions in {a1, b1, b0, a0} - ATMVP (is 1 candidate, where CU is divided into 4x4 sub-CUs) - STMVP (is 1 candidate, where CU is divided into 4x4 sub-CUs) - spatial positions in {b2} - a temporal position {c} - bi-directional combinations - zero MV for each reference picture | 17 candidates with up to 8 Affine ones, among: - spatial positions in {a1, b1, b0, a0} - spatial affine positions in {a1, b1, b0, a0} - ATMVP (is 1 candidate, where CU is divided into 4x4 sub-CUs) - STMVP (is 1 candidate, where CU is divided into 4x4 sub-CUs) - constructed affine candidate based on spatial non-affine positions - spatial positions in {b2} - spatial affine positions in {b2, a2, b3} - a temporal position {c} - temporal affine positions in {curr, c} - non-adjacent spatial positions - non-adjacent spatial affine positions - bi-directional combinations - zero MV for each reference picture |
| Signaling | For each reference picture: - predictor idx Residual is coded if mode != skip | For each reference picture: - predictor idx Residual is coded if mode != skip |

In the merge process described in J0021, a candidate list is created as presented in Table 1. The list contains classical merge candidates as well as affine ones. In the proposed solution, the affine candidates are enriched with new ones. The spatial affine candidates set in JEM {a1, b1, a0, b0, b2} becomes {a1, b1, b0, a0, b2, a2, b3} in this contribution.

Furthermore, affine does not benefit from the temporal candidates introduced in the proposed solution. To that aim, every available reference picture of each list is explored at the current CU position {curr} and at the bottom-right position {c} to get affine model seeds. When a temporal candidate was coded with an affine mode, its associated affine model seeds are retrieved. If the corresponding temporal reference picture is also available for the current CU in same reference picture list, then the current CU inherits the reference picture of the temporal candidate. Otherwise the first reference picture in the considered list is chosen for current CU. As for classical temporal candidates, the retrieved affine model seeds are scaled according to the temporal distances between pictures and their reference pictures. The same process as for spatial candidates is applied, i.e. the current affine model seeds are derived from the scaled temporal candidate seeds according to the following equations (also used in JEM).

  - v0=v3+v5-v3Ycurr-YneighbHneighb+v4-v3Xcurr-XneighbWneighb

  - v1=v0+(v4-v3)WcurrWneighb

  - v2=v0+(v5-v3)HcurrHneighb

Equation 1. Affine model seeds derivation process from CU to CU.

Figure 1. Positions of spatial candidates of motion information.

# Results

The tools described above are implemented within the BMS-1.0 software. The following tables report simulation results using VTM and BMS configuration files, as defined in the Common Test Conditions. The same binaries are used (JEM_TOOLS macros activated). All tests were performed with SIMD=SSE42 options. The test 4.2.4 is added on top of 4.2.3 (unified merge affine, with 4 parameters model, r606 from the svn repository).

## VTM

Anchor is VTM and tested is VTM with affine and high precision motion vector tool from BMS activated.

### Test 4.2.4A

Added affine model storage similar to 4.1.4 a). Temporal motion vector storage is the VTM default (16x16).

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.08% | -2.73% | -2.90% | 139% | 114% |
| Class A2 | -6.73% | -5.93% | -5.64% | 145% | 127% |
| Class B | -4.67% | -3.36% | -3.40% | 148% | 115% |
| Class C | -1.93% | -1.61% | -1.57% | 150% | 113% |
| Overall | -4.03% | -3.28% | -3.26% | 146% | 117% |
| Class D | -2.16% | -1.57% | -1.78% | 153% | 115% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -3.19% | -2.04% | -1.75% | 174% | 105% |
| Class C | -1.59% | -0.60% | -0.61% | 167% | 97% |
| Class E | -2.55% | -1.91% | -2.55% | 213% | 97% |
| Overall | -2.50% | -1.53% | -1.57% | 181% | 100% |
| Class D | -2.86% | -2.21% | -2.18% | 159% | 108% |

### Test 4.2.4B

Add new affine candidates: 2 spatial candidates + temporal candidates. Temporal motion vector storage is the VTM default (16x16), but the affine temporal use 4x4 motion vector storage.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.00% | -2.64% | -2.86% | 140% | 117% |
| Class A2 | -6.68% | -5.89% | -5.63% | 148% | 130% |
| Class B | -4.65% | -3.35% | -3.44% | 149% | 118% |
| Class C | -2.05% | -1.56% | -1.60% | 151% | 114% |
| Overall | -4.03% | -3.24% | -3.27% | 148% | 119% |
| Class D | -2.36% | -1.78% | -1.98% | 156% | 118% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -3.25% | -2.16% | -1.85% | 177% | 114% |
| Class C | -1.71% | -0.80% | -0.70% | 168% | 100% |
| Class E | -2.70% | -2.23% | -2.55% | 213% | 97% |
| Overall | -2.60% | -1.72% | -1.64% | 182% | 105% |
| Class D | -3.20% | -1.96% | -2.60% | 159% | 111% |

### Test 4.2.4C

Test A and B together. Temporal motion vector storage is VTM default (16x16) and the affine model is stored at CU level.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.07% | -2.80% | -2.95% | 143% | 126% |
| Class A2 | -6.74% | -5.99% | -5.69% | 149% | 140% |
| Class B | -4.69% | -3.41% | -3.49% | 152% | 128% |
| Class C | -2.07% | -1.71% | -1.71% | 154% | 126% |
| Overall | -4.08% | -3.35% | -3.35% | 150% | 129% |
| Class D | -2.47% | -1.65% | -1.89% | 159% | 127% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -3.28% | -2.19% | -2.04% | 182% | 125% |
| Class C | -1.77% | -0.83% | -0.78% | 170% | 106% |
| Class E | -2.87% | -2.69% | -2.83% | 220% | 118% |
| Overall | -2.67% | -1.86% | -1.82% | 187% | 117% |
| Class D | -3.29% | -2.15% | -2.64% | 162% | 117% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Affine 1080p | -9.58% | -3.42% | -2.46% | 132% | 117% |
| Affine 720p | -15.00% | -10.58% | -12.08% | 199% | 162% |
| Overall | -11.75% | -6.29% | -6.31% | 156% | 133% |

The affine class is the same as in [3], except for the Shields sequence (see attached excel files).

## BMS

Anchor is BMS without the affine tool.

### 4.2.4A

Added affine model storage similar to 4.1.4 a). Temporal motion vector storage is the BMS default (4x4).

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 without affine |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.81% | -1.39% | -1.43% | 117% | 98% |
| Class A2 | -4.44% | -3.60% | -3.58% | 121% | 100% |
| Class B | -3.45% | -2.61% | -2.62% | 121% | 100% |
| Class C | -1.70% | -1.33% | -1.50% | 118% | 98% |
| Overall | -2.85% | -2.22% | -2.28% | 119% | 99% |
| Class D | -1.92% | -1.74% | -1.52% | 118% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 without affine |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -3.22% | -2.21% | -2.12% | 130% | 104% |
| Class C | -1.66% | -1.19% | -1.16% | 125% | 103% |
| Class E | -2.28% | -2.04% | -2.22% | 157% | 105% |
| Overall | -2.47% | -1.83% | -1.83% | 134% | 104% |
| Class D | -2.97% | -2.72% | -2.80% | 124% | 103% |

### Test 4.2.4B

Add new affine candidates: 2 spatial candidates + temporal candidates. Temporal motion vector storage is the BMS default (4x4).

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 without affine |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.76% | -1.22% | -1.40% | 116% | 100% |
| Class A2 | -4.36% | -3.56% | -3.57% | 120% | 102% |
| Class B | -3.39% | -2.55% | -2.60% | 120% | 102% |
| Class C | -1.67% | -1.29% | -1.41% | 113% | 101% |
| Overall | -2.80% | -2.15% | -2.24% | 117% | 101% |
| Class D | -1.98% | -1.78% | -1.46% | 112% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 without affine |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -3.27% | -2.17% | -2.63% | 133% | 105% |
| Class C | -1.74% | -1.52% | -1.38% | 126% | 105% |
| Class E | -2.32% | -2.55% | -2.18% | 157% | 107% |
| Overall | -2.52% | -2.05% | -2.10% | 136% | 105% |
| Class D | -3.24% | -2.75% | -3.34% | 123% | 103% |

### Test 4.2.4C

Test A and B together. Temporal motion vector storage is BMS default (4x4) and the affine model is stored at CU level.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 without affine |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.78% | -1.36% | -1.44% | 117% | 105% |
| Class A2 | -4.43% | -3.58% | -3.63% | 120% | 108% |
| Class B | -3.46% | -2.61% | -2.66% | 121% | 108% |
| Class C | -1.75% | -1.35% | -1.46% | 119% | 107% |
| Overall | -2.86% | -2.22% | -2.29% | 119% | 107% |
| Class D | -2.06% | -1.77% | -1.76% | 119% | 107% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 without affine |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -3.29% | -2.50% | -2.31% | 132% | 111% |
| Class C | -1.80% | -1.53% | -1.55% | 127% | 111% |
| Class E | -2.40% | -2.25% | -1.85% | 159% | 118% |
| Overall | -2.57% | -2.11% | -1.94% | 136% | 113% |
| Class D | -3.31% | -2.85% | -3.57% | 125% | 110% |

### Test 4.2.4C on affine class

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 (with affine) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Affine 1080p |  |  |  |  |  |
| Affine 720p | -12.92% | -9.12% | -10.13% | 125% | 114% |
| Overall |  |  |  |  |  |

# References

[1] “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – low and high complexity versions”, JVET-J0021, April 2018.

[2] “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – medium complexity version”, JVET-J0022r1, April 2018.

[3] “Affine Transform prediction for next generation video coding”, COM16-C1016R1, ITU, Sept. 2015.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).