| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0221-v1 |
| --- | --- |

| Title: | CE3 Related: Additional results of JVET-J1023 Core Experiments 5.2.3, 5.2.4 and 5.2.5 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Steve Keating Karl Sharman | Email: | steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe |  |  |

_____________________________

# Abstract

Results are reported relating to configurations of JVET-J1023 multi-reference line intra prediction expanding on Core Experiments CE3-5.2.x.  This contribution is for the purpose of providing additional information.

# Introduction

This document details additional results for JVET-J1023 Core Experiments 5.2.3 regarding the use of multiple (in this case, 2) lines of reference samples for intra prediction. This use of two lines of reference samples was originally described in JVET-J0028.

# Results –VTM All Intra

This experiment was designed to test the effect of disabling the use of multiple reference lines for the first line in a CTU in order to remove the requirement to add extra line store(s). The result with disabling for the first line is:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (75:25) all angles sample repeat for last new ref. sample except 1st line of CTU (CE3-5.2.3) Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.38% | -0.49% | 119% | 110% |
| Class A2 | -0.22% | -0.24% | -0.16% | 114% | 107% |
| Class B | -0.32% | -0.36% | -0.42% | 116% | 107% |
| Class C | -0.19% | -0.48% | -0.53% | 115% | 109% |
| Class E | -0.24% | -0.62% | -0.61% | 117% | 108% |
| Overall | -0.26% | -0.42% | -0.44% | 116% | 108% |
| Class D | -0.01% | -0.20% | -0.22% | 113% | 108% |
| Class F (optional) | 2.37% | 2.21% | 2.38% | 115% | 110% |

And for reference, the same processing but enabled for all lines in a CTU:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (75:25) all angles sample repeat for last new ref. sample including 1st line of CTU Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.39% | -0.40% | -0.60% | 117% | 108% |
| Class A2 | -0.25% | -0.21% | -0.24% | 114% | 107% |
| Class B | -0.35% | -0.43% | -0.44% | 116% | 108% |
| Class C | -0.23% | -0.50% | -0.65% | 114% | 107% |
| Class E | -0.29% | -0.76% | -0.66% | 116% | 108% |
| Overall | -0.30% | -0.46% | -0.52% | 115% | 108% |
| Class D | -0.03% | -0.17% | -0.28% | 115% | 110% |
| Class F (optional) | 2.41% | 2.19% | 2.30% | 116% | 111% |

Note that this reference case is not exactly the same as that tested in CE3-5.2.1 due to implementation differences which are:

- CE3-5.2.1 implementation adds another sample to the second reference line, increasing its length to height+width+2.  This can only affect the bottom right predicted sample for angles near 45 degrees so is unlikely to have much effect on BD-rate.  Adding this extra sample does require more checks for sample availability which may affect run time.

- CE3-5.2.1 implementation disables the use of the second reference line for purely horizontal or vertical angles.  This has a negligible effect on overall BD-rate (0.01% in All Intra) but does improve class F (2.4%  1.0%).

The results for CE3-5.2.1 are shown below for comparison:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (75:25) all angles except H and V additional sample assignment for last new ref. sample including 1st line of CTU (CE3-5.2.1) Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.39% | -0.47% | -0.61% | 129% | 110% |
| Class A2 | -0.25% | -0.27% | -0.26% | 121% | 107% |
| Class B | -0.35% | -0.48% | -0.50% | 123% | 107% |
| Class C | -0.23% | -0.52% | -0.66% | 122% | 109% |
| Class E | -0.32% | -0.81% | -0.69% | 125% | 108% |
| Overall | -0.31% | -0.51% | -0.54% | 124% | 108% |
| Class D | -0.04% | -0.24% | -0.34% | 121% | 109% |
| Class F (optional) | 1.02% | 0.71% | 0.74% | 124% | 110% |

# Comment on results

It can be seen that disabling the use of the second row of reference samples for the first line in a CTU does reduce the BD-rate improvement very slightly (-0.26% vs. -0.30% for Intra).

Disabling the use of the second row for pure H and V angles as done in CE3-5.2.1 does reduce the loss for class F (1.02% vs. 2.41%) but disabling for small blocks with a 50:50 weighting as in CE3-5.2.5 actually gives a small gain in class F (-0.19%).

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).