| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0221-v2 |
| --- | --- |

| Title: | CE3 Related: Additional results of JVET-J1023 Core Experiments 5.2.3, 5.2.4 and 5.2.5 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information and Core Experiment |  |  |
| Author(s) or Contact(s): | Steve Keating Karl Sharman | Email: | steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe |  |  |

_____________________________

# Abstract

Results are reported for Core Experiments CE3-5.2.3, 5.2.4 and 5.2.5 multi-reference line intra prediction, additional results are also included expanding on Core Experiments CE3-5.2.x.

# Introduction

This document details results for JVET-J1023 Core Experiments 5.2.3, 5.2.4 and 5.2.5 regarding the use of multiple (in this case, 2) lines of reference samples for intra prediction. Additional results are also included expanding on Core Experiments CE3-5.2.x.

# Description

In JEM, a single row or column of reference samples is projected at a range of angles (65) to generate a range of angular predictions.  The use of two or more rows (or columns) of samples can provide a better angular prediction in most cases.  In the diagram below the blue arrow shows how a reference sample is projected to generate a predicted sample:

Where the light blue squares are reference samples and the white squares are samples to be predicted.

In addition, a second row of reference samples is used to make an alternate angular prediction value, shown below by the green arrow:

These two prediction values are then combined by simple (CE3-5.2.5) or weighted (CE3-5.2.3, CE3-5.2.4) averaging to generate a prediction value that is less affected by noise in the reference samples.

The second prediction value (green arrow) may be less accurate then the first (blue arrow) as it is derived from samples spatially further away.  This is most pronounced in small blocks so use of the second prediction values is avoided for small blocks (CE3-5.2.5).

The second prediction value may also be unused for blocks at the top of the CTU as the second row of reference samples would not be available in a hardware implementation without requiring significant extra storage (CE3-5.2.3).

# Results

## CE3-5.2.3

This experiment was designed to test the effect of disabling the use of multiple reference lines for the first line in a CTU in order to remove the requirement to add extra line store(s). The results with disabling for the first line are:

### VTM

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (75:25) except 1st line of CTU (CE3-5.2.3) Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.38% | -0.49% | 119% | 110% |
| Class A2 | -0.22% | -0.24% | -0.16% | 114% | 107% |
| Class B | -0.32% | -0.36% | -0.42% | 116% | 107% |
| Class C | -0.19% | -0.48% | -0.53% | 115% | 109% |
| Class E | -0.24% | -0.62% | -0.61% | 117% | 108% |
| Overall | -0.26% | -0.42% | -0.44% | 116% | 108% |
| Class D | -0.01% | -0.20% | -0.22% | 113% | 108% |
| Class F (optional) | 2.37% | 2.21% | 2.38% | 115% | 110% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (75:25) except 1st line of CTU (CE3-5.2.3) Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.32% | -0.31% | 101% | 99% |
| Class A2 | -0.12% | -0.04% | -0.02% | 101% | 100% |
| Class B | -0.17% | -0.28% | -0.24% | 101% | 100% |
| Class C | -0.12% | -0.35% | -0.25% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.15% | -0.26% | -0.21% | 101% | 100% |
| Class D | 0.02% | 0.23% | -0.04% | 103% | 101% |
| Class F (optional) | 1.37% | 1.53% | 1.74% | 104% | 101% |

### BMS

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (75:25) except 1st line of CTU (CE3-5.2.3) Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | 0.05% | -0.16% | 106% | 103% |
| Class A2 | -0.20% | -0.07% | -0.12% | 105% | 104% |
| Class B | -0.23% | -0.21% | -0.20% | 105% | 104% |
| Class C | -0.18% | -0.36% | -0.33% | 105% | 105% |
| Class E | -0.08% | -0.33% | -0.20% | 106% | 103% |
| Overall | -0.18% | -0.20% | -0.21% | 105% | 104% |
| Class D | 0.05% | 0.00% | -0.11% | 105% | 105% |
| Class F (optional) | 1.74% | 1.60% | 1.22% | 105% | 104% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (75:25) except 1st line of CTU (CE3-5.2.3) Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.12% | -0.15% | 103% | 100% |
| Class A2 | -0.09% | -0.10% | -0.08% | 102% | 101% |
| Class B | -0.12% | -0.06% | -0.07% | 103% | 100% |
| Class C | -0.06% | -0.18% | -0.55% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.11% | -0.22% | 103% | 100% |
| Class D | 0.05% | -0.55% | -0.32% | 102% | 101% |
| Class F (optional) | 1.04% | 0.95% | 1.02% | 103% | 101% |

## CE3-5.2.4

This experiment was designed to test the effect of using a simple 50:50 weighting for the prediction from the original and extra reference samples:

### VTM

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (50:50) including 1st line of CTU (CE3-5.2.4) Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.41% | -0.51% | -0.74% | 117% | 108% |
| Class A2 | -0.13% | -0.09% | -0.06% | 115% | 109% |
| Class B | -0.26% | -0.42% | -0.47% | 114% | 106% |
| Class C | 0.28% | -0.17% | -0.36% | 117% | 110% |
| Class E | -0.04% | -0.80% | -0.83% | 112% | 104% |
| Overall | -0.11% | -0.39% | -0.48% | 115% | 107% |
| Class D | 0.65% | 0.21% | 0.03% | 113% | 108% |
| Class F (optional) | 3.25% | 3.02% | 3.12% | 117% | 111% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (50:50) including 1st line of CTU (CE3-5.2.4) Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | -0.42% | -0.49% | 111% | 114% |
| Class A2 | -0.08% | -0.10% | 0.00% | 110% | 112% |
| Class B | -0.14% | -0.33% | -0.33% | 111% | 109% |
| Class C | 0.13% | -0.30% | -0.40% | 111% | 118% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.29% | -0.31% | 111% | 113% |
| Class D | 0.32% | -0.06% | -0.19% | 110% | 127% |
| Class F (optional) | 1.96% | 1.95% | 2.12% | 112% | 119% |

### BMS

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (50:50) including 1st line of CTU (CE3-5.2.4) Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.13% | -0.25% | 107% | 104% |
| Class A2 | -0.17% | -0.14% | -0.08% | 103% | 103% |
| Class B | -0.16% | -0.29% | -0.32% | 104% | 103% |
| Class C | 0.19% | -0.27% | -0.18% | 103% | 103% |
| Class E | 0.23% | -0.16% | -0.02% | 105% | 103% |
| Overall | -0.03% | -0.21% | -0.19% | 104% | 103% |
| Class D | 0.60% | 0.20% | 0.07% | 106% | 107% |
| Class F (optional) | 2.48% | 2.35% | 2.15% | 104% | 103% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (50:50) including 1st line of CTU (CE3-5.2.4) Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.21% | -0.25% | 103% | 100% |
| Class A2 | -0.10% | -0.22% | -0.09% | 102% | 100% |
| Class B | -0.09% | -0.27% | -0.35% | 101% | 99% |
| Class C | 0.15% | -0.03% | -0.51% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.18% | -0.32% | 102% | 100% |
| Class D | 0.29% | -0.19% | -0.39% | 102% | 101% |
| Class F (optional) | 1.54% | 1.58% | 1.66% | 102% | 100% |

## CE3-5.2.5

This experiment was designed to test the effect of disabling the use of the extra reference samples for blocks smaller than or equal to 4xN or Nx4, a 50:50 weighting is used for larger blocks

### VTM

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (50:50) including 1st line of CTU Except for small blocks (CE3-5.2.5) Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.56% | -0.56% | -0.68% | 116% | 109% |
| Class A2 | -0.39% | -0.26% | -0.22% | 110% | 106% |
| Class B | -0.46% | -0.28% | -0.28% | 113% | 107% |
| Class C | -0.33% | -0.28% | -0.26% | 113% | 108% |
| Class E | -0.38% | -0.40% | -0.31% | 113% | 107% |
| Overall | -0.42% | -0.34% | -0.34% | 113% | 107% |
| Class D | -0.22% | 0.01% | -0.20% | 111% | 108% |
| Class F (optional) | -0.19% | -0.16% | -0.02% | 112% | 109% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (50:50) including 1st line of CTU Except for small blocks (CE3-5.2.5) Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.51% | -0.58% | 100% | 99% |
| Class A2 | -0.20% | -0.24% | -0.17% | 101% | 100% |
| Class B | -0.23% | -0.21% | -0.26% | 101% | 100% |
| Class C | -0.18% | -0.09% | -0.07% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.23% | -0.24% | -0.25% | 101% | 100% |
| Class D | -0.09% | -0.05% | -0.34% | 102% | 100% |
| Class F (optional) | -0.08% | -0.02% | -0.16% | 103% | 102% |

### BMS

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (50:50) including 1st line of CTU Except for small blocks (CE3-5.2.5) Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | -0.10% | -0.22% | 105% | 102% |
| Class A2 | -0.26% | 0.03% | 0.03% | 104% | 102% |
| Class B | -0.34% | -0.02% | -0.02% | 103% | 102% |
| Class C | -0.30% | -0.10% | -0.03% | 104% | 104% |
| Class E | -0.21% | -0.05% | 0.15% | 105% | 103% |
| Overall | -0.28% | -0.05% | -0.02% | 104% | 103% |
| Class D | -0.14% | 0.06% | 0.21% | 105% | 106% |
| Class F (optional) | -0.17% | 0.12% | -0.06% | 105% | 105% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (50:50) including 1st line of CTU Except for small blocks (CE3-5.2.5) Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.10% | -0.24% | 103% | 100% |
| Class A2 | -0.13% | 0.10% | 0.05% | 102% | 101% |
| Class B | -0.18% | 0.13% | -0.14% | 102% | 99% |
| Class C | -0.07% | 0.19% | -0.20% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | 0.09% | -0.14% | 103% | 100% |
| Class D | 0.01% | 0.05% | -0.21% | 103% | 101% |
| Class F (optional) | -0.06% | 0.19% | 0.21% | 103% | 101% |

# Additional Results –VTM All Intra

## Baseline

A reference baseline for CE3-5.2.3, 5.2.4 and 5.2.5 is given below:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (75:25) all angles sample repeat for last new ref. sample including 1st line of CTU Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.39% | -0.40% | -0.60% | 117% | 108% |
| Class A2 | -0.25% | -0.21% | -0.24% | 114% | 107% |
| Class B | -0.35% | -0.43% | -0.44% | 116% | 108% |
| Class C | -0.23% | -0.50% | -0.65% | 114% | 107% |
| Class E | -0.29% | -0.76% | -0.66% | 116% | 108% |
| Overall | -0.30% | -0.46% | -0.52% | 115% | 108% |
| Class D | -0.03% | -0.17% | -0.28% | 115% | 110% |
| Class F (optional) | 2.41% | 2.19% | 2.30% | 116% | 111% |

Note that this reference case is not exactly the same as that tested in CE3-5.2.1 due to implementation differences which are:

- CE3-5.2.1 implementation adds another sample to the second reference line, increasing its length to height+width+2.  This can only affect the bottom right predicted sample for angles near 45 degrees so is unlikely to have much effect on BD-rate.  Adding this extra sample does require more checks for sample availability which may affect run time.

- CE3-5.2.1 implementation disables the use of the second reference line for purely horizontal or vertical angles.  This has a negligible effect on overall BD-rate (0.01% in All Intra VTM) but does improve class F (2.4%  1.0%).

The results for CE3-5.2.1 are shown below:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (75:25) all angles except H and V additional sample assignment for last new ref. sample including 1st line of CTU (CE3-5.2.1) Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.39% | -0.47% | -0.61% | 129% | 110% |
| Class A2 | -0.25% | -0.27% | -0.26% | 121% | 107% |
| Class B | -0.35% | -0.48% | -0.50% | 123% | 107% |
| Class C | -0.23% | -0.52% | -0.66% | 122% | 109% |
| Class E | -0.32% | -0.81% | -0.69% | 125% | 108% |
| Overall | -0.31% | -0.51% | -0.54% | 124% | 108% |
| Class D | -0.04% | -0.24% | -0.34% | 121% | 109% |
| Class F (optional) | 1.02% | 0.71% | 0.74% | 124% | 110% |

## Run time

It was noticed that a significant contribution to the increase in run time was filling the additional reference sample array, this part of the code was tidied up by combining the new assignments into the original loops etc. giving an improvement in run times (113%  107% Enc VTM):

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (50:50) including 1st line of CTU Except for small blocks (CE3-5.2.5) Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.56% | -0.56% | -0.68% | 108% | 104% |
| Class A2 | -0.39% | -0.26% | -0.22% | 107% | 103% |
| Class B | -0.46% | -0.28% | -0.28% | 107% | 103% |
| Class C | -0.33% | -0.28% | -0.26% | 107% | 105% |
| Class E | -0.38% | -0.40% | -0.31% | 107% | 104% |
| Overall | -0.42% | -0.34% | -0.34% | 107% | 104% |
| Class D | -0.22% | 0.01% | -0.20% | 108% | 106% |
| Class F (optional) | -0.19% | -0.16% | -0.02% | 106% | 104% |

# Comment on results

It can be seen that disabling the use of the second row of reference samples for the first line in a CTU does reduce the BD-rate improvement very slightly (-0.26% vs. -0.30% for Intra VTM).

Disabling the use of the second row of reference samples for pure H and V angles as done in CE3-5.2.1 does reduce the loss for class F (1.02% vs. 2.41%) but disabling for small blocks with a 50:50 weighting as in CE3-5.2.5 actually gives a small gain in class F (-0.19%) whilst improving the gain for all other classes.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).