| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0223-v1 |
| --- | --- |

| Title: | CE1.1.0.19: Context modeling for coding CU split decisions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Shawmin Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of Core Experiment 1.1.0.19 (CE1.1.0.19). The proposed method modifies the context modeling scheme for entropy coding the coding unit (CU) split decisions. The proposed method reportedly reduces the total number of contexts by 7 and only needs to access two spatial neighboring CUs for context selection. The proposed method reportedly leads to 0.01%, 0.01%, 0.04% luma BD-rates for the AI, RA, and LB settings, respectively, under the VTM-1.0 CTCs (with all BMS tools off). The proposed method reportedly leads to 0.01%, 0.02%, -0.03% luma BD-rates for the AI, RA, and LB settings, respectively, under the BMS-1.0 CTCs (with all BMS tools on).

# Introduction

This contribution reports the results of CE1.1.0.19 for the proposed context modeling scheme for entropy coding the CU split decisions. In the Versatile Video Coding (VVC) draft as specified in [1], each coding tree unit (CTU) can be partitioned into one or multiple smaller size CUs by a quadtree with nested multi-type tree using binary and ternary split. A syntax element qt_split_cu_flag is signaled to indicate whether a coding unit is split into coding units with half horizontal and vertical size. A syntax element mtt_split_cu_flag is signaled to indicate whether a coding unit is split into CUs by one of multi-type tree spliting modes. A syntax element mtt_split_cu_vertical_flag is signaled to indicate whether a CU is split horizontally or vertically. A syntax element mtt_split_cu_binary_flag is signaled to indicate whether a CU is partitioned by binary or ternary split.

In the reference software VTM-1.0 for VVC Test Model 1 (VTM1) [2], context selection for entropy coding the syntax element qt_split_cu_flag first evaluates the quadtree depths of two spatial neighboring CUs from the above neighboring row and the left neighboring column in comparison with the quadtree depth of the current CU. The conetxt increment index is set equal to the number of the neighboring CUs having qudtree depth higher than the current CU. It is the same method that is employed for entropy coding quadtree split decisions in HEVC. However, VTM-1.0 further adds two contexts related to the quadtree depths of four spatial neighboring CUs from the above neighboring row and the left neighboring column. The conetxt increment index is set to 3 when the current quadtree depth is less than the minimum quadtree depth among the four neighboring CUs. The conetxt increment index is set to 4 when the current quadtree depth is larger than the maximum quadtree depth among the four neighboring CUs.

In VTM-1.0, six contexts are assigned to entropy coding the syntax element mtt_split_cu_binary_flag. The context selection is conditioned on the block shape and the split direction of the current block.

# Proposed method

The proposed method targets at lowering the complexity for context modeling for entropy coding the CU split decisions. In the proposed method for entropy coding qt_split_cu_flag, we remove both four-neighbor dependent contexts and employ the original HEVC two-neighbor contexts for coding split_cu_flag. In this way, we only need to access two neighboring CUs, instead of four neighboring CUs in VTM-1.0, for context selection. The total number of contexts can be reduced by 2 for coding qt_split_cu_flag.

In the proposed method for entropy coding mtt_split_cu_binary_flag, we employ a single context and reduce the total number of contexts by 5.

# Experimental results

The proposed method was integrated into the BMS-1.0 reference software [3]. The coding results generated by the proposed method were evaluated against the anchor results generated by the unmodified BMS-1.0 reference software. All the coding experiments were conducted according to the CE1 test conditions specified in [4, 5].

Table 1 summarizes the simulation results of CE.1.1.0.19 under the VTM-1.0 configurations (with all BMS tools off) and the BMS-1.0 configurations (with all BMS tools on), respectively. It is shown that the proposed method has almost no impact on BD-rates.

The detailed simulation results can be found in the attached spreadsheets.

Table 1: Summary of the average BD bitrate and run time results for CE1.1.0.19.

# Conclusions

The proposed method for entropy coding the CU split decisions can reduce the total number of contexts by 7. It only needs to access two spatial neighboring CUs for context selection, as compared to four spatial neighboring CUs in VTM-1.0. Experimental results show the proposed method exhibit no performance loss in compression while lowering the complexity for context modeling.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- B. Bross, “Versatile Video Coding (Draft 1),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1001, 10th Meeting: San Diego, US, 10–20 Apr. 2018.

- The software VTM-1.0: https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_VTM/tags/VTM-1.0 .

- The software BMS-1.0: https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/.

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1010, 10th Meeting: San Diego, US, 10–20 Apr. 2018.

- J. Ma, F. Le Léannec, and M. W. Park, “Description of Core Experiment 1 (CE1): Partitioning,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1021, 10th Meeting: San Diego, US, 10–20 Apr. 2018.