| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0224-v1 |
| --- | --- |

| Title: | CE1.2.0.10: CU partitioning along picture boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Shawmin Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of Core Experiment 1.2.0.10 (CE1.2.0.10). The proposed method reportedly supports to select either quadtree (QT) split or binary-tree (BT) split for partitioning the out-of-bounds coding units (CUs), which are not entirely contained within the current picture region. The proposed method is reportedly involved minor modifications to the current partitioning rules for handling the regular in-bounds CUs. It reportedly does not need to utilize a separate set of the constraint parameters dedicated to handling the CUs partitioned from the out-of-bounds coding tree units (CTUs). The proposed method reportedly achieves average luma BD-rate reductions of 0.05%, 0.30%, and 0.49% for the AI, RA, and LB, respectively, under the VTM-1.0 common test conditions (CTCs) with all BMS tools off. The proposed method reportedly achieves average luma BD-rate reductions of 0.05%, 0.34%, and 0.67% for the AI, RA, and LB, respectively, under the BMS-1.0 CTCs with all BMS tools on.

# Introduction

This contribution reports the results of CE1.2.0.10 for the proposed method for partitioning the CUs that are not entirely contained within the current picture region. In the Versatile Video Coding (VVC) draft as specified in [1], each coding tree unit (CTU) can be partitioned into one or multiple smaller size CUs by a quadtree with nested multi-type tree using binary and ternary split. However, when the current CU is not entirely contained in the current picture region, it is inferred to be always partitioned by quadtree split. This method for handling the out-of-bounds CUs may lead to many small-size square CUs distributed along pictures, inefficient for compression.

# Proposed method

In the proposed method [2], when a CU corresponds to a region across a current picture boundary, it is inferred to be further split into smaller-size child-CUs using QT split or BT split. QT split is inferred when the current CU is across both the right picture boundary and the bottom picture boundary or the current CU size is larger than the maximum allowed CU size for BT split. BT split is inferred when the BT/TT depth of the current CU is larger than zero or the QT depth of the current CU is not less than the maximum allowed QT depth. When BT split is selected, horizontal split is inferred if the current CU is across the bottom picture boundary and vertical split is inferred if the current CU is across the right picture boundary.

# Experimental results

The proposed method was integrated into the BMS-1.0 reference software [3]. The coding results generated by the proposed method were evaluated against the anchor results generated by the unmodified BMS-1.0 reference software. All the coding experiments were conducted according to the CE1 test conditions specified in [4, 5].

Table 1 summarizes the simulation results of CE.1.2.0.10 under the VTM-1.0 configurations (with all BMS tools off) and the BMS-1.0 configurations (with all BMS tools on), respectively. It is shown that the proposed method can substantially improve BD rate performance in the random access and low delay settings. The average BD rate savings are particularly higher for coding the test sequences from Class B in HD resolution.

The detailed simulation results can be found in the attached spreadsheets.

Table 1: Summary of the average BD bitrate and run time results for CE1.2.0.10.

# Conclusions

This contribution reports the results of CE1.2.0.10 for the proposed method for partitioning the CUs that are not entirely contained within the current picture region. The experimental results show that the proposed method can substantially improve efficiency in compression with only minor modifications to the existing CU partitioning rules for handling the out-of-bounds CUs. The improvement is more significant for coding the test sequences in HD resolution.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- B. Bross, “Versatile Video Coding (Draft 1),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1001, 10th Meeting: San Diego, US, 10–20 Apr. 2018.

- C.-W. Hsu, et al, “Description of SDR video coding technology proposal by MediaTek,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J0018, 10th Meeting: San Diego, US, 10–20 Apr. 2018.

- The software BMS-1.0, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1010, 10th Meeting: San Diego, US, 10–20 Apr. 2018.

- J. Ma, F. Le Léannec, and M. W. Park, “Description of Core Experiment 1 (CE1): Partitioning,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1021, 10th Meeting: San Diego, US, 10–20 Apr. 2018.