| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–20 July 2018 | Document: JVET-K0225-v1 |
| --- | --- |

| Title: | CE1.3.5: Maximum TT split depth restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution proposes a maximum ternary-tree (TT) depth restriction method. The proposed method configures the maximum allowed TT depth as the maximum allowed BT depth minus a threshold. When the threshold is set to 1 under the common test condition (i.e., the maximum TT depth is 2, the allowed BT depth is 3), simulation results reportedly show 21% encoding time saving with 0.38%, 0.72%, and 0.78% luma BD-rates for All Intra (AI), Random Access (RA), and Low-delay B (LB) respectively in VTM-1.0 tests and 0.31%, 0.71%, and 0.65% luma BD-rates for AI, RA, and LB respectively in BMS-1.0 tests. When the threshold is set to 2 (i.e., the maximum TT depth is 1, the allowed BT depth is 3), simulation results reportedly show 40% encoding time saving with 0.83%, 1.56%, and 1.62% luma BD-rates for AI, RA, and LB respectively in VTM-1.0 tests and 0.68%, 1.52%, and 1.46% luma BD-rates for AI, RA, and LB respectively in BMS-1.0 tests.

Introduction

The multi-type-tree structure (MTT) proposed in JVET-D0117 [1] introduced ternary tree (TT) CU structure by extending the quad tree (QT) and binary tree (BT) structures proposed in m37524/COM16–C966 [2]. In MTT, a CTU is first partitioned by the QT structure, where a CTU block can be non-split or recursively split into 4 equal-size child-blocks. Then, the QT leaf nodes can be further partitioned by the BT or TT structure. The BT and TT partitioning can be interleaved and recursively applied. However, once BT or TT partitioning is applied, QT partitioning is disabled for the subsequent nodes. There are two splitting types (i.e., horizontal split and vertical split) in both BT and TT partitioning. For BT partitioning, the two splitting types are both symmetric. For TT partitioning, the splitting is in a ratio of 1:2:1.

In VTM-1.0 and BMS-1.0 reference software [4], BT and TT structures share the same maximum allowed tree depth syntax. To reduce the complexity of TT structure, an encoder-only fast algorithm can be applied to early terminate the TT test before maximum allowed tree depth for TT structure is reached. However, the TT structure syntax still needs to be signaled even when TT test is stopped. This contribution proposes a maximum TT split depth restriction method by indicating a threshold value in SPS to configure additional maximum allowed tree depth for TT structure.

Proposed method

In JVET-J0018 [3], the maximum allowed TT depth (MaxTTDepth) can be restricted or controlled separately. The TT partitioning is allowed for splitting the current block when the multi-type tree depth (MTTDepth) is smaller than the maximum allowed BT depth (MaxBTDepth) minus a threshold value, and the threshold is signaled as a SPS syntax element. Besides, the TT structure syntax is absent when the MTTDepth exceeds the MaxBTDepth minus the given threshold. In summary, the maximum TT depth for intra-slice, chroma intra-slice, and inter-slice are:

- MaxTTDepthI = MaxBTDepthI – ThresholdI

- MaxTTDepthIChroma = MaxBTDepthIChroma – ThresholdIChroma

- MaxTTDepthInter = MaxBTDepthInter – ThresholdInter

To configure the TT depth thresholds, the following SPS syntax elements are introduced:

- delta_tt_depth_flag: The on/off control of the proposed maximum allowed TT depth restriction method.

- delta_tt_depth_i_slice_minus1: Indicates the TT depth threshold for intra-slice (i.e., ThresholdI), is (delta_tt_depth_i_slice_minus1 + 1).

- delta_tt_depth_i_slice_chroma_minus1: Indicates the TT depth threshold for chroma intra-slice (i.e., ThresholdIChroma) is (delta_tt_depth_i_slice_chroma_minus1 + 1).

- delta_tt_depth_minus1: Indicates the TT depth threshold for inter-slice (i.e., ThresholdInter) is (delta_tt_depth_minus1 + 1).

Experimental results

The proposed maximum TT depth restriction method has been implemented on BMS-1.0 [4], and the results by setting the maximum TT split depth to 2 and 1 of AI, RA, and LB are summarized in Table 1 and Table 2, respectively. The simulations were performed following the JVET common test conditions [5]. More detail results can be found in the accompanying Excel files.

Table 1. Set the TT depth threshold to 1 for configuring the maximum TT depth to 2

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.24% | 0.13% | 0.13% | 78% | 100% | 0.14% | 0.08% | 0.14% | 78% | 100% |
| Class A2 | 0.39% | 0.56% | 0.54% | 79% | 99% | 0.30% | 0.42% | 0.50% | 78% | 100% |
| Class B | 0.33% | 0.42% | 0.55% | 79% | 99% | 0.27% | 0.48% | 0.49% | 78% | 99% |
| Class C | 0.44% | 0.62% | 0.67% | 78% | 100% | 0.35% | 0.66% | 0.75% | 77% | 100% |
| Class E | 0.55% | 0.73% | 0.63% | 81% | 100% | 0.47% | 0.67% | 0.56% | 80% | 100% |
| Overall | 0.38% | 0.49% | 0.52% | 79% | 99% | 0.31% | 0.47% | 0.50% | 78% | 100% |
| Class D | 0.29% | 0.67% | 0.54% | 78% | 100% | 0.26% | 0.63% | 0.76% | 78% | 100% |
| Class F (optional) | 0.37% | 0.57% | 0.52% | 81% | 100% | 0.31% | 0.63% | 0.54% | 80% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.64% | 0.68% | 0.92% | 76% | 100% | 0.61% | 0.80% | 0.93% | 75% | 98% |
| Class A2 | 0.88% | 1.24% | 1.25% | 79% | 100% | 0.86% | 1.09% | 1.07% | 79% | 101% |
| Class B | 0.71% | 1.12% | 0.94% | 79% | 99% | 0.67% | 1.07% | 0.85% | 78% | 100% |
| Class C | 0.68% | 1.01% | 1.04% | 80% | 100% | 0.72% | 1.29% | 0.80% | 78% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.72% | 1.03% | 1.02% | 79% | 100% | 0.71% | 1.08% | 0.90% | 77% | 100% |
| Class D | 0.55% | 0.94% | 0.86% | 82% | 100% | 0.56% | 0.75% | 0.68% | 80% | 100% |
| Class F (optional) | 0.55% | 0.95% | 0.95% | 82% | 99% | 0.52% | 0.79% | 0.76% | 80% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.66% | 1.14% | 1.32% | 77% | 100% | 0.62% | 0.49% | 1.13% | 78% | 101% |
| Class C | 0.69% | 0.98% | 1.02% | 78% | 99% | 0.68% | 0.80% | 0.59% | 80% | 100% |
| Class E | 1.07% | 1.47% | 1.44% | 81% | 98% | 0.67% | 1.04% | 0.86% | 84% | 101% |
| Overall | 0.78% | 1.17% | 1.25% | 78% | 99% | 0.65% | 0.73% | 0.88% | 80% | 101% |
| Class D | 0.45% | 0.34% | 0.48% | 81% | 100% | 0.54% | 0.93% | 0.14% | 82% | 100% |
| Class F (optional) | 0.56% | 0.96% | 0.92% | 81% | 100% | 0.74% | 0.92% | 1.04% | 81% | 100% |

Table 2. Set the TT depth threshold to 2 for configuring the maximum TT depth to 1

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.58% | 0.29% | 0.39% | 56% | 98% | 0.37% | 0.30% | 0.36% | 56% | 99% |
| Class A2 | 0.83% | 1.21% | 1.15% | 58% | 99% | 0.68% | 1.06% | 1.14% | 57% | 99% |
| Class B | 0.73% | 0.91% | 1.07% | 58% | 99% | 0.61% | 1.01% | 1.06% | 58% | 99% |
| Class C | 0.93% | 1.48% | 1.49% | 57% | 100% | 0.78% | 1.49% | 1.70% | 57% | 100% |
| Class E | 1.13% | 1.49% | 1.36% | 61% | 100% | 0.98% | 1.58% | 1.31% | 60% | 100% |
| Overall | 0.83% | 1.08% | 1.11% | 58% | 99% | 0.68% | 1.10% | 1.14% | 58% | 99% |
| Class D | 0.71% | 1.30% | 1.30% | 57% | 100% | 0.56% | 1.45% | 1.46% | 57% | 101% |
| Class F (optional) | 0.77% | 1.16% | 1.13% | 62% | 100% | 0.71% | 1.30% | 0.98% | 60% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 1.37% | 1.52% | 1.87% | 58% | 99% | 1.28% | 1.68% | 1.92% | 55% | 99% |
| Class A2 | 1.92% | 2.83% | 2.80% | 62% | 100% | 1.88% | 2.73% | 2.56% | 60% | 100% |
| Class B | 1.54% | 2.24% | 2.10% | 61% | 99% | 1.46% | 2.37% | 2.25% | 59% | 101% |
| Class C | 1.44% | 2.19% | 2.30% | 61% | 101% | 1.49% | 2.43% | 2.07% | 59% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 1.56% | 2.20% | 2.25% | 61% | 99% | 1.52% | 2.32% | 2.20% | 58% | 100% |
| Class D | 1.13% | 1.87% | 1.74% | 64% | 103% | 1.17% | 1.89% | 1.89% | 61% | 100% |
| Class F (optional) | 1.26% | 1.88% | 1.78% | 65% | 99% | 1.21% | 1.93% | 1.80% | 62% | 101% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 1.42% | 2.27% | 2.38% | 57% | 100% | 1.35% | 1.88% | 2.53% | 58% | 100% |
| Class C | 1.35% | 2.20% | 2.33% | 58% | 100% | 1.34% | 2.17% | 2.02% | 59% | 100% |
| Class E | 2.28% | 2.97% | 3.15% | 65% | 100% | 1.80% | 2.87% | 3.13% | 69% | 100% |
| Overall | 1.62% | 2.42% | 2.56% | 59% | 100% | 1.46% | 2.22% | 2.51% | 61% | 100% |
| Class D | 1.07% | 1.62% | 1.41% | 61% | 102% | 1.14% | 1.67% | 1.33% | 62% | 100% |
| Class F (optional) | 1.65% | 2.32% | 2.43% | 63% | 100% | 1.70% | 2.73% | 2.13% | 62% | 101% |

Conclusions

This contribution proposes a maximum TT depth restriction method. With the proposed method, a threshold value is indicated in SPS to configure the maximum allowed tree depth for TT structure. When the threshold is set to 1 (i.e., the maximum TT depth is 2, the allowed BT depth is 3), simulation results reportedly show 21% encoding time saving with 0.38%/0.72%/0.78% luma BD-rate for AI/RA/LB in VTM-1.0 tests and 0.31%/0.71%/0.65% luma BD-rate for AI/RA/LB in BMS-1.0 tests. When the threshold is set to 2 (i.e., the maximum TT depth is 1, the allowed BT depth is 3), simulation results reportedly show 40% encoding time saving with 0.83%/1.56%/1.62% luma BD-rate for AI/RA/LB in VTM-1.0 tests and 0.68%/1.52%/1.46% luma BD-rate for AI/RA/LB in BMS-1.0 tests.

References

- X. Li, H.-C. Chuang, J. Chen, M. Karczewicz, “Multi-Type-Tree,” JVET-D0117, Oct. 2016.

- J. An, Y.-W. Chen, K. Zhang, H. Huang, Y.-W. Huang, S. Lei, “Block partitioning structure for next generation video coding,” MPEG doc. m37524 and ITU-T SG16 Doc. COM16–C966, Oct. 2015.

- C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, H. Huang, S.-T. Hsiang, C.-C. Chen, M.-S. Chiang, C.-Y. Lai, C.-M. Tsai, Y.-C. Su, Z.-Y. Lin, Y.-L. Hsiao, J. Klopp, I.-H. Wang, Y.-W. Huang, S.-M. Lei, “Description of SDR video coding technology proposal by MediaTek,” JVET-J0018, April 2018.

- BMS-1.0: https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, April 2018.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).