| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0226-v1 |
| --- | --- |

| Title: | CE1.5.1: Separate coding tree partitioning for luma and chroma in I slices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Tzu-Der Chuang, Shawmin Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, peter.chuang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of Core Experiments 1.5.1.1 and 1.5.1.2 (CE1.5.1.1 and CE1.5.1.2) for using the two separate coding trees respectively for partitioning luma and chroma in I slices, or referred to as the dual-tree mode. CE1.5.1.1 evaluates the performance using the basic dual-tree coding tool. CE1.5.1.2 evaluates the performance by further integrating other dual-tree related improvements. CE1.5.1.1 reportedly achieves average BD-rates of +0.57%, -5.82%, and -7.12% for luma, cb, and cr, respectively, and an encoding run time saving of 12% for the AI configuration setting under the VTM-1.0 common test conditions (CTCs) with all BMS-1.0 tools off and reportedly achieves average BD-rates of -0.59%, -9.16%, and -9.00% for luma, cb, and cr, respectively, and an encoding run time saving of 40% for the AI configuration setting under the BMS-1.0 CTCs with all BMS tools on. CE1.5.1.2 reportedly achieves average BD-rates of +0.05%, -7.21%, and -8.48% for luma, cb, and cr, respectively, and an encoding run time saving of 18% for the AI configuration setting under the VTM-1.0 CTCs with all BMS-1.0 tools off and reportedly achieves average BD-rates of -0.88%, -9.95%, and -9.81% for luma, cb, and cr, respectively, and an encoding run time saving of 41% for the AI configuration setting under the BMS-1.0 CTCs with all BMS-1.0 tools on.

# Introduction

In the Versatile Video Coding (VVC) draft as specified in [1], each coding tree unit (CTU) can be partitioned into one or multiple smaller size coding units (CUs) by a quadtree with nested multi-type tree using binary and ternary split. The partitioning represented by a single coding tree is applied to all three color components in a CTU.

This contribution reports the results of CE1.5.1.1 and CE1.5.1.2 for using the two separate coding trees for partitioning luma and chroma, respectively, in I slices.

# Proposed methods

In the proposed method [2], each CTU in a I slice can signal a separate partitioning structure for the chroma components, with its own set of the configuration parameters such as the maximum CU size for BT/TT partitioning, the maximum depth for BT/TT partitioning, and the minimum CU size for QT split.

CE1.5.1.1 evaluates the performance using the basic dual-tree coding tool. It just corresponds to setting the encoder option DualITree equal to 1.

CE1.5.1.2 evaluates the performance by further integrating other dual-tree coding tools. In this test, we further set the encoder option MDMS equal to 1 for enabling the multiple direct modes for chroma intra coding [3]. We further enable the use of the 64x64 transform on the chroma blocks. A chroma CU size constraint is further applied: The quadtree depth of the current chroma CU is constrained not to be larger than the quadtree depth of the corresponding luma CU.

# Experimental results

The proposed method was integrated into the BMS-1.0 reference software [4]. The coding results generated by the proposed method were evaluated against the anchor results generated by the unmodified BMS-1.0 reference software. All the coding experiments were conducted according to the CE1 test conditions specified in [5, 6].

Table 1 summarizes the simulation results for CE1.5.1.1 under the VTM-1.0 configurations (with all BMS tools off) and the BMS-1.0 configurations (with all BMS tools on), respectively. It is shown that the proposed method using the basic dual-tree coding tool can provide significant BD rate savings for chroma components and suffer some BD rate increases for the luma component under the VTM-1.0 configurations. The BD rate saving is further improved under the BMS-1.0 configurations. It is also shown that the proposed method can substantially reduce encoding run time under both VTM-1.0 and BMS-1.0 configurations.

Table 2 summarizes the simulation results for CE1.5.1.2 under the VTM-1.0 configurations (with all BMS tools off) and the BMS-1.0 configurations (with all BMS tools on), respectively. It is shown that the proposed dual-tree coding tool can be further improved in both compression efficiency and encoding run time by integrating other dual-tree related modifications.

The detailed simulation results can be found in the attached spreadsheets.

Table 1.  Summary of the average BD bitrate and run time results for CE1.5.1.1

Table 2.  Summary of the average BD bitrate and run time results for CE1.5.1.2

# Conclusions

This contribution reports the results of CE1.5.1.1 and CE1.5.1.2 for using the proposed dual-tree mode for coding I slices. The experimental results show that the proposed dual-tree coding tool integrated with other dual-tree related modifications can significantly reduce BD rate for the chroma components with almost no BD rate increase for the luma component while speeding up encoding.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- B. Bross, “Versatile Video Coding (Draft 1),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1001, 10th Meeting: San Diego, US, 10–20 Apr. 2018.

- C.-W. Hsu, et al, “Description of SDR video coding technology proposal by MediaTek,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J0018, 10th Meeting: San Diego, US, 10–20 Apr. 2018.

- L. Zhang, et al, “EE5: Multiple Direct Modes for chroma intra coding” JVET-E0062, Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-E0062, 5th Meeting: Geneva, CH, 12–20 January 2017.

- The software BMS-1.0, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1010, 10th Meeting: San Diego, US, 10–20 Apr. 2018.

- J. Ma, F. Le Léannec, and M. W. Park, “Description of Core Experiment 1 (CE1): Partitioning,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1011, 10th Meeting: San Diego, US, 10–20 Apr. 2018.