| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0227-v1 |
| --- | --- |

| Title: | CE1.6.1: Coding large size CUs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, ChingYeh Chen, Chen-Yen Lai, Yu-Wen Huang, Shawmin Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, chingyeh.chen, jenny.lai, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of CE1.6.1.1 and CE1.6.1.2 for handling a coding unit (CU) of width or height larger than the maximum transform size. The proposed methods reportedly remove extra implementation for processing the CUs containing more than one transform unit (TU), which is employed for coding the CUs of width or height larger than the maximum transform size. In CE1.6.1.1, the CU is inferred to be further partitioned by a quadtree split when the width or height of the current CU is larger than the maximum transform size. In CE1.6.1.2, the CU is inferred to have zero residual in a B or P slice when the width or height of the current CU is larger than the maximum transform size. It is reported that each CU always contains one transform unit (TU) and extra implementation to process the CUs containing more than one TU is not needed using the proposed methods in both CE tests. CE1.6.1.1 reportedly leads to -0.01%, 0.90%, and 0.64% luma BD-rates for the AI, RA, and LB settings, respectively, under the VTM-1.0 common test conditions (CTCs) with all BMS tools off. CE1-6.1.2 reportedly leads to -0.01%, 0.07%, and 0.02% luma BD-rates for the AI, RA, and LB settings, respectively, under the VTM-1.0 CTCs with all BMS tools off.

# Introduction

In the Versatile Video Coding (VVC) draft as specified in [1], each coding tree unit (CTU) can be partitioned into one or multiple smaller size CUs by a quadtree with nested multi-type tree using binary and ternary split. The size of the CU can be as large as the CTU size. The maximum supported transform size is 64 samples. When both width and height of the current CU is less than or equal to the maximum transform size, the resulting CU partition is also employed for prediction and transform processing. That is, each CU only contains one prediction unit (PU) and one TU. When the width or height of the current CU is larger than the maximum transform size, the CU is inferred to be further partitioned into multiple TUs by tiling with the maximum transform blocks.

# Proposed methods

This contribution reports the results of CE1.6.1.1 and CE1.6.1.2 proposed in JVET-J0018 [2] for handling a CU of width or height larger than the maximum transform size. The desirable feature of the proposed methods lie in that each CU always contains only one TU. There is no need to add extra implementation to process the CUs containing more than one TU.

In CE1.6.1.1, when the width or height of the current CU is larger than the maximum transform size, the current CU is inferred to be further partitioned by quadtree split without explicitly signaling. The maximum allowed CU size is equal to the maximum allowed transform block size.

In CE1.6.1.2, when the width or height of the current CU is larger than the maximum transform size in a B or P slice, the current CU is inferred to have zero residual without explicitly signaling. The CU is inferred to be coded in either skip mode or AMVP mode. The syntax element pred_mode_flag is thus inferred to be equal to 0 without explicitly signaling. The syntax element merge_flag is inferred to be equal to 0 without explicitly signaling after the syntax element cu_skip_flag is signaled to be equal to 0. The maximum allowed CU size is equal to the CTU size. When the width or height of the current CU is larger than the maximum transform size in an I slice, the current CU is inferred to be further partitioned by quadtree split, the same method as CE1.6.1.1.

# Experimental results

The proposed method was integrated into the BMS-1.0 reference software [3]. The coding results generated by the proposed method were evaluated against the anchor results generated by the unmodified BMS-1.0 reference software. All the coding experiments were conducted according to the CE1 test conditions specified in [4, 5].

Table 1 summarizes the simulation results of CE1.6.1.1 under the VTM-1.0 configurations (with all BMS tools off) and the BMS-1.0 configurations (with all BMS tools on), respectively. It is shown that this proposed method can reduce encoding run time substantially but suffers some performance loss in compression efficiency.

Table 2 summarizes the simulation results of CE1.6.1.2 under the VTM-1.0 configurations (with all BMS tools off) and the BMS-1.0 configurations (with all BMS tools on), respectively. It is shown that this proposed method suffers slight loss only for coding sequences from the Class A1 category and exhibit no significant loss in compression efficiency for coding sequences from all other categories. The performance in encoding run time is slightly improved under the Random Access and Low Delay B settings.

The detailed simulation results can be found in the attached spreadsheets.

Table 1: Summary of the average BD bitrate and run time results for CE1.6.1.1

Table 2: Summary of the average BD bitrate and run time results for CE1.6.1.2.

# Conclusions

The proposed methods provide the solutions to remove extra implementation to process the CUs containing more than one TU for handling the CUs of width or height larger than the maximum transform size. Experimental results show the proposed method in CE1.6.1.1 can speed up encoding run time substantially while suffering some performance loss in compression efficiency. The proposed method in CE1.6.1.2 exhibit no significant loss in compression efficiency while slightly improving performance in encoding run time under the Random Access and Low Delay B settings.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- B. Bross, “Versatile Video Coding (Draft 1),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1001, 10th Meeting: San Diego, US, 10–20 Apr. 2018.

- C.-W. Hsu, et al, “Description of SDR video coding technology proposal by MediaTek,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J0018, 10th Meeting: San Diego, US, 10–20 Apr. 2018.

- The software BMS-1.0, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1010, 10th Meeting: San Diego, US, 10–20 Apr. 2018.

- J. Ma, F. Le Léannec, and M. W. Park, “Description of Core Experiment 1 (CE1): Partitioning,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1021, 10th Meeting: San Diego, US, 10–20 Apr. 2018.