| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0228-v1 |
| --- | --- |

| Title: | CE 4-2.1: Adding non-adjacent spatial merge candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruoyang Yu Per Wennersten Rickard Sjöberg | Tel: Email: | +46725298793 ruoyang.yu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution reports the results for Core Experiment (CE) 4-2.1 on adding non-adjacent spatial merge candidates. The tool modifies the merge candidate list generation process such that spatial candidates can be derived from non-adjacent neighboring positions. The BD-rate luma results over VTM-1.0 under random access (RA), low delay B (LB), and low delay P (LP) common test configurations are -0.45%, -0.12%, and -0.47% respectively. The BD-rate luma results over BMS-1.0 are -0.22% for RA, -0.09% for LB, and -0.22% for LP.

# Technology description

As described in JVET-J0012 [1] section 2.1.7.3, the proposed tool derives additional spatial candidates from two non-adjacent neighboring positions. One from the closest non-neighboring block in straight horizontal spatial distance and the other one from the closest non-neighboring block in straight vertical spatial distance, as illustrated in Figure 1. The blocks are limited within a maximum distance of 1 CTU to the current block. The fetching process of non-adjacent neighboring candidates starts with tracing the previous decoded blocks in the vertical direction. The vertical inverse tracing stops when an inter block is encountered or the traced back distance reaches 1 CTU size. The fetching process then traces the previous decoded blocks in the horizontal direction. The criterion for stopping the horizontal fetching process depends on whether there is a vertical non-adjacent candidate successfully being fetched or not. If no vertical non-adjacent candidate is fetched, the horizontal fetching process stops when an inter is encountered or the traced back distance exceed one CTU size threshold. If there is a vertical non-adjacent candidate fetched, then the horizontal fetching process stops when an inter block which contains different MV from the vertical non-adjacent candidate is encountered or the traced back distance exceed on CTU size threshold.

Figure 1. Illustration of fetching non-adjacent spatial merge candidates VN and HN

The fetched non-adjacent neighboring candidates are added before the TMVP candidate in the merge candidate list when the number of already added spatial candidates is less than a threshold. The threshold for adding the non-adjacent neighboring candidate is set to 3 in the current implementation. Figure 2 illustrates the modified merge candidate list generation process.

Figure 2. Modified merge candidate list generation process

# Simulation results

The tests were defined in the CE description document JVET-J1024 [2]. The simulations were conducted by following the JVET common test conditions [3]. The software was compiled using GCC 6.2.0. The SIMD configuration was AVX2.

## Random access configuration

## Low delay B configuration

## Low delay P configuration

# References

- R. Sjöberg, K. Andersson, R. Yu, Z. Zhang, P. Wennersten, J. Lainema, A. Hallapuro, A. Aminlou, M. Hannuksela, R. Ghaznavi-Youvalari, J. Ridge, JVET-J0012, “Description of SDR and HDR video coding technology proposal by Ericsson and Nokia”, April. 2018.

- H. Yang, S. Liu, K. Zhang, JVET-J1024, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, April. 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, JVET-J1010, “JVET common test conditions and software reference configurations”, April. 2018.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).