| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0229-v1 |
| --- | --- |

| Title: | CE1.7.0.1: Signaling maximum CU size for BT/TT split |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Shawmin Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of CE1.7.01. In the proposed method, the maximum allowed coding unit (CU) size for binary and ternary split is signaled for the slice header in a B or P slice. The proposed method reportedly leads to 0.06% and 0.16% luma BD-rates and 10% and 12% encoding run time savings for the RA and LB settings, respectively, under the VTM-1.0 common test conditions (CTCs) with all BMS tools off.

# Introduction

In the Versatile Video Coding (VVC) draft as specified in [1], each coding tree unit (CTU) can be partitioned into one or multiple smaller size CUs by a quadtree with nested multi-type tree using binary and ternary split. The maximum allowed CU size for binary split is signaled in the slice header for a B or P slice. The maximum allowed CU size for ternary split is pre-defined to be equal to 128x128.

# Proposed method

This contribution reports the results of CE1.7.01. In the proposed method [2], the maximum allowed CU size for multi-type tree partitioning is signaled in the slice header for a B or P slice. The signaled maximum CU size constraint is applied to both binary and ternary split. The encoder can determine the constraint parameter value considering the block partitioning statistics of previously coded slices.

# Experimental results

The proposed method was integrated into the BMS-1.0 reference software [3]. The coding results generated by the proposed method were evaluated against the anchor results generated by the unmodified BMS-1.0 reference software. All the coding experiments were conducted according to the CE1 test conditions specified in [4, 5].

Table 1 summarizes the simulation results of CE1.7.0.1 under the VTM-1.0 configurations (with all BMS tools off) and the BMS-1.0 configurations (with all BMS tools on), respectively. It is shown the proposed method can reduce encoding run time substantially without significant sacrifice in compression efficiency. The run time savings are achieved by skipping the test on the ternary split for the CUs of width or height larger than the signaled maximum size for multi-type tree partitioning.

The detailed simulation results can be found in the attached spreadsheets.

Table 1: Summary of the average BD bitrate and run time results for CE1.7.0.1.

# Conclusions

This contribution proposes a modified method for signaling the constraint on the maximum allowed CU size for binary and ternary split. Experimental results show the proposed method can substantially speed up encoding without significant sacrifice in compression efficiency.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- B. Bross, “Versatile Video Coding (Draft 1),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1001, 10th Meeting: San Diego, US, 10–20 Apr. 2018.

- C.-W. Hsu, et al, “Description of SDR video coding technology proposal by MediaTek,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J0018, 10th Meeting: San Diego, US, 10–20 Apr. 2018.

- The software BMS-1.0, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1010, 10th Meeting: San Diego, US, 10–20 Apr. 2018.

- J. Ma, F. Le Léannec, and M. W. Park, “Description of Core Experiment 1 (CE1): Partitioning,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1021, 10th Meeting: San Diego, US, 10–20 Apr. 2018.

# Text Modifications

The text provided in this section is based on JVET-J1001. The added text sections are marked in blue.

### Slice header syntax

[Ed. (BB): Preliminary basic slice header, subject to further study and pending further specification development.]

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| slice_address | u(v) |
| slice_type | ue(v) |
| if ( slice_type != I ) |  |
| log2_diff_ctu_max_bt_size log2_diff_ctu_max_mtt_size | ue(v) |
| byte_alignment( ) |  |
| } |  |

### Slice header semantics

log2_diff_ctu_max_mtt_size log2_diff_ctu_max_bt_size specifies the difference between the luma CTB size and the maximum luma size (width or height) of a coding block that can be split using a binary split or a ternary split. The value of log2_diff_ctu_max_mtt_size log2_diff_ctu_max_bt_size shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive.

When log2_diff_ctu_max_mtt_size log2_diff_ctu_max_bt_size is not present, the value of log2_diff_ctu_max_bt_size log2_diff_ctu_max_bt_size is inferred to be equal to 2.

The variables MinQtLog2SizeY, MaxBtLog2SizeY, MinBtLog2SizeY, MaxTtLog2SizeY, MinTtLog2SizeY, MaxBtSizeY, MinBtSizeY, MaxTtSizeY, MinTtSizeY and MaxMttDepth are derived as follows:

MinQtLog2SizeY  =  ( slice_type  = =  I )  ?  MinQtLog2SizeIntraY  :  MinQtLog2SizeInterY	(7-26)

MaxBtLog2SizeY  =  CtbLog2SizeY − log2_diff_ctu_max_bt_size	(7-27)

MaxBtLog2SizeY  =  ( slice_type  = =  I )  ?  5 :  CtbLog2SizeY − log2_diff_ctu_max_mtt_size 	(7-27)

MinBtLog2SizeY  =  MinCbLog2SizeY		(7-28)

MaxTtLog2SizeY  =  ( slice_type  = =  I )  ?  5  :  7	(7-29)

MaxTtLog2SizeY  =  ( slice_type  = =  I )  ?  5  : CtbLog2SizeY − log2_diff_ctu_max_mtt_size	(7-29)