| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0231-v1 |
| --- | --- |

| Title: | CE2.1.2: Bilateral filter - spatial filter strength adjustment |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chi Su, ChingYeh, Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd, Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yu-chi.su, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek.inc |  |  |

_____________________________

# Abstract

This contribution proposes a method to adjust spatial filter in bilateral filter based on coding block (CB) area size. Under VTM-1.0, the proposed bilateral filter achieves -0.10%, -0.40%, and -0.37% luma BD-rates with 3%, 2%, and 1% encoding time increases and 9%, 3%, and 1% decoding time increases for AI, RA, and LB, respectively. Under BMS-1.0, it achieves -0.26%, -0.44%, and -0.46% luma BD-rates with 3%, 4%, and 3% encoding time increases and 4%, 0%, and 1% decoding time increases for AI, RA, and LB, respectively.

# Introduction

Bilateral filter after inverse transform is proposed in JVET-D0069 [1]. In JVET-D0069, the bilateral filter is applied to the reconstructed block to form a smoothed-filtered reconstructed block in both the encoder and the decoder. Figure 1 shows the position of bilateral filter at the decoder.

Figure 1. The positon of bilateral filter in decoder.

The bilateral filter is a five-tap filter in the shape of a plus sign. Each luma sample in the reconstructed transform block (TB) is replaced by a weighted average of itself and its neighbours within the TB. The weights are calculated based on the distance from the center sample as well as the difference in sample values. A sample located at (i, j), is filtered using its neighbouring sample (k, l). The weight ωi,j,k,l is the weight assigned for sample (k, l) to filter the sample(i, j), and it is defined as

ωi,j,k,l=e- i-k2+j-l22σd2 - Ii,j-Ik,l22σr2                			(1)

Here, I(i, j ) and I(k, l) are the original reconstructed intensity values of samples (i, j) and (k,l) respectively. σd is the spatial parameter, and σr is the range parameter.

In JEM, the strength of the bilateral filter is controlled by σd and σr. σr is determined by the quantization parameter (QP) used for the current block. The formula is shown as in Equation 2.

σr=maxQP-172, 0.01                                  (2)

σd is determined by the prediction mode and the minimal length between TB height and width. The formula is shown as in Equation 3 and Equation 4.

σd=0.92-minTB width, TB height40,	for intra blocks			(3)

σd=0.72 – minTB width,  TB height40,	for inter blocks			(4)

In order to reduce the number of calculations, the bilateral filter in the JEM has been implemented using a look-up table (LUT) [2] [3]. The center weight values that determine the spatial filter strength based on the prediction mode and the TB length in JEM are shown in Table 1. In this table, a large center weight value means weaker filtering. The bilateral filtering is turned off for inter blocks of size 16×16 and larger.

Table 1. Center weight values based on the prediction mode and the TB length in JEM.

# Proposed method

The proposed center weight values are shown in Table 2.

Table 2. Center weight values based on the prediction mode and CB area.

# Experimental results

The proposed method was implemented on top of BMS-1.0 reference software [4]. Table 3 shows the simulation results.

Table3. Proposed bilateral filter under VTM-1.0 (left) and BMS-1.0 (right).

# Conclusion

In this contribution, a spatial filter strength adjustment based on CU area size for bilateral filter is proposed. Under VTM-1.0, the proposed bilateral filter achieves -0.10%/-0.40%/-0.37% luma BD-rate with 3%/2%/1% encoding time increase and 9%/3%/1% decoding time increase for AI/RA/LB. Under BMS-1.0, the proposed bilateral filter achieves -0.26%/-0.44%/-0.46% luma BD-rate with 3%/4%/3% encoding time increase and 4%/0% and 1% decoding time increase for AI/RA/LB. We recommend to integrate the proposed spatial filter adjustment method for bilateral filter into the next release of VTM and BMS software.

# Patent rights declaration

MediaTek, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J. Ström, P. Wennersten, Y. Wang, K. Andersson, J. Samuelsson, “Bilateral filter after inverse transform,” JVET-D0069, Chengdu, China, October 2016.

[2] J. Ström, K. Andersson, P. Wennersten, M. Pettersson, J. Enhorn, R. Sjöberg, “EE2-JVET-E0032 bilateral filter Test1, Test2,” JVET-F0034, Hobart, AU, April 2017.

[3] J. Ström, K. Andersson, P. Wennersten, M. Pettersson, J. Enhorn, R. Sjöberg, “EE2-JVET related: Division-free bilateral filter,” JVET-F0096, Hobart, AU, April 2017.

[4] BMS-1.0 reference software, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/