| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0232-v1 |
| --- | --- |

| Title: | CE2.2.1.3: Long deblocking filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Tzu-Der Chuang, Chih-Wei Hsu, Ching-Yeh Chen Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, peter.chuang, cw.hsu, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to add three long strong filter sets in addition to the HEVC strong filter set for deblocking. In addition, a deblocking bugfix is also included to apply deblocking to maximum transform block (TB) boundaries that do not coincide with any coding block (CB) boundaries. It is reported that the proposed long strong filtering and the bugfix together lead to negligible changes in BD-rate and encoding time, and decoding time increase is 3% and 1% for VTM-1.0 and BMS-1.0, respectively. It is claimed that blocky artifacts at maximum TB boundaries that do not coincide with any CB boundaries are reduced because of the bugfix. It is asserted that the overall subjective quality at high QPs is improved because of the long strong filter sets.

# Introduction

HEVC deblocking filters (DF) consist of two main parts: filtering decisions and filtering processes. The filtering decisions are to determine filter type (e.g., no filtering, weak filtering, or strong filtering) for block edge. The filtering processes are to filter samples at both sides of block edge based on filtering decisions. However, as popular picture resolutions become higher and higher, HEVC DF cannot effectively reduce blocky artifacts at large block boundaries in high resolution videos (e.g., UHD). In this contribution, longer filters are added to DF. The proposed DF is modified from the DF in JVET-J0018 [1] with a deblocking bugfix.

Some definitions in this contribution are described as follow. The left side and right side of a vertical block edge are called p-side and q-side, respectively. The upper side and lower side of a horizontal block edges are called p-side and q-side, respectively. In addition, each block edge is of 4-sample length by definition, and pij and qij (i=0, 1, 2, or 3; and j=0, 1, 2, …) represent the p-side and q-side samples on the ith line perpendicular to the block edge, j equal to 0 means the sample is adjacent to the block edge, and a larger j means the sample is at a larger distance from the block edge. The CB length refers to CB width and CB height for horizontal filtering across vertical block edge and vertical filtering across horizontal block edge, respectively. The adjacent block edges refer to the left and right connected block edges for horizontal edge, and the upper and lower connected block edges for vertical edge.

# Proposed DF

## Flowchart of filtering decisions

The filtering decisions to determine no/weak/strong filtering and the weak filtering are exactly the same as those in HEVC. To improve DF, the strong filtering is extended. Three long strong filter sets, named as R8W7-ultra, R8W7-strong, and R4W4-strong, are added for strong filtering. The corresponding details of long strong filter sets are described in Section 2.4. As illustrated in Figure 1 to derive filtering decisions for each block edge, if the answer to “Whether to apply strong filtering?” is yes, “Whether to apply long strong filtering?” is further checked. The corresponding details of long strong filtering decision are described in Section 2.2. If the answer to “Whether to apply long strong filtering” is yes, one of the three long strong filter sets is selected for p-side, and one of the three long strong filter sets is selected for q-side. The selected long strong filter set for p-side and the selected long strong filter set for q-side can be different. The corresponding details of long strong filter set selection are described in Section 2.3.

Figure 1.  The proposed flowchart of filtering decisions

## Long strong filtering decision

This subsection describes the answer to “Whether to apply long strong filtering?” in Figure 1. When all the following conditions are true, the answer is yes. Otherwise, the answer is no (i.e., apply HEVC strong filtering for p-side and q-side).

- The slice type is inter.

- The CB length at p-side and the CB length at q-side are either larger than or equal to eight.

- Both of the two inter-side difference values representing difference between p-side and q-side are larger than a threshold, inter_side_diff_th.

- Both of the two intra-side difference values representing variation within p-side plus variation within q-side are smaller than a threshold, intra_side_diff_th.

- Any one of adjacent block edges also satisfies all the above four conditions.

The two inter-side differences are described in Section 2.2.1, the two intra-side differences are described in Section 2.2.2.

### Inter-side difference

The two inter-side difference values are defined in Equation 1.

p0i-q1i+p1i-q0ia??1, where i = 0 or 3                                (1)

That is, for each block edge, only the first and fourth lines are used to obtain the inter-side difference. The threshold value, inter_side_diff_th, depends on quantization parameter (QP) and is shown in Figure 2.

Figure 2.  Derivation of inter_side_diff_th from QP

### Intra-side difference

The two intra-side difference values are defined in Equation 2. The threshold value, intra_side_diff_th, is set to theparameter divided by 2.

, where i = 0 or 3                            (2)

## Long strong filter set selection

When the long strong filtering decision output is yes (i.e., the answer to “Whether to apply long strong filtering?” in Figure 1 is yes), the long strong filter set selection (i.e., “Select long strong filter set for p-side and q-side separately” in Figure 1) is activated to select one of the three long strong filter sets for p-side and one of the three long strong filter sets for q-side. The selection based on the p-side and q-side length of the current block edge is described as follows, where lenp and lengq denote the CB length at p-side and the CB length at q-side, respectively.

- If “lenp ≥ 32 and lenq ≥ 32”, R8W7-ultra or R8W7-strong filtering is selected as follows:

  - If any one of adjacent block edges of the current block edge is also satisfied “lenp ≥ 32 and lenq ≥ 32”, R8W7-ultra filtering is applied to both p-side and q-side of the current block edge.

  - Otherwise (i.e., “lenp ≥ 32 and lenq ≥ 32” is only true for the current block edge), R8W7-strong filtering is applied to both p-side and q-side of the current block edge.

- If “32 > lenp ≥ 16”, R8W7-strong filtering is applied to p-side of the current block edge.

- If “16 > lenp ≥ 8”, R4W4-strong filtering is applied to p-side of the current block edge.

- If “32 > lenq ≥ 16”, R8W7-strong filtering is applied to q-side of the current block edge.

- If “16 > lenq ≥ 8”, R4W4-strong filtering is applied to q-side of the current block edge.

## Long strong filter sets

The filter set naming rule is explained here. “Applying RxWy-fs filtering (x & y are numbers, R stands for read, W stands for write, fs stands for filtering strength and can be ultra or strong) to z-side (z can be p or q)” means at most x samples at z-side are read and y samples at z-side are written (i.e., modified).

After applying the selected long strong filter set to one side of block edge, a clipping function is applied to filtered sample values, which is the same as that in HEVC and bounds the changes of sample values within -2tc to 2tc.

Three long strong filter sets are described in the rest of this subsection.

### R8W7-ultra filtering

If R8W7-ultra filtering is applied to p-side, filtered sample values from  to  are as follows:

If R8W7-ultra filtering is applied to q-side, filtered sample values from  to  are as follows:

### R8W7-strong filtering

If R8W7-strong filtering is applied to p-side, filtered sample values from  to  are as follows:

If R8W7-strong filtering is applied to q-side, filtered sample values from  to  are as follows:

### R4W4-strong filtering

If R4W4-strong filtering is applied to p-side, filtered sample values from  to  are as follows:

If R4W4-strong filtering is selected at q-side, filtered sample values from  to  are as follows:

## Deblocking bugfix

In VTM-1.0 and BMS-1.0, each CB larger than maximum TB is inferred to be further partitioned into multiple TBs by tiling with the maximum TBs, while each CB smaller than or equal to maximum TB has only one TB. However, DF is only performed at CB boundaries and always skipped at maximum TB boundaries that do not coincide with any CB boundaries. It is proposed to apply DF to maximum TB boundaries that do not coincide with any CB boundaries, as a bugfix.

# Experimental results

## Objective results

The proposed DF including the deblocking bugfix has been implemented on BMS-1.0 [2], and the objective results are summarized in the Table 1. The simulations were performed following the JVET common test conditions [3].

Table 1.  Objective results of the proposed DF including the deblocking bugfix

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 104% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 104% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 104% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 105% | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 104% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 104% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 103% | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 101% | 105% | 0.00% | 0.00% | 0.00% | 100% | 102% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.04% | -0.05% | 99% | 103% | -0.06% | -0.10% | 0.03% | 100% | 99% |
| Class A2 | -0.01% | -0.01% | 0.01% | 99% | 104% | 0.01% | 0.01% | 0.02% | 100% | 101% |
| Class B | 0.01% | 0.00% | 0.02% | 100% | 102% | -0.01% | -0.01% | -0.03% | 100% | 101% |
| Class C | 0.00% | -0.02% | -0.04% | 100% | 104% | 0.02% | -0.05% | -0.05% | 100% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.02% | -0.02% | -0.01% | 100% | 103% | -0.01% | -0.04% | -0.01% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.02% | 101% | 104% | 0.01% | -0.01% | -0.04% | 101% | 102% |
| Class F (optional) | 0.00% | 0.00% | 0.01% | 100% | 103% | 0.01% | 0.02% | -0.02% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.01% | 0.08% | 0.15% | 100% | 104% | -0.02% | -0.23% | 0.02% | 101% | 102% |
| Class C | 0.00% | 0.09% | 0.09% | 100% | 103% | -0.03% | 0.18% | 0.15% | 101% | 101% |
| Class E | 0.04% | -0.01% | -0.17% | 100% | 102% | -0.14% | -0.60% | -0.09% | 100% | 101% |
| Overall | 0.01% | 0.06% | 0.05% | 100% | 103% | -0.05% | -0.19% | 0.04% | 101% | 101% |
| Class D | -0.03% | -0.25% | 0.03% | 100% | 104% | -0.01% | 0.11% | -0.14% | 101% | 101% |
| Class F (optional) | -0.06% | -0.11% | -0.04% | 100% | 103% | -0.09% | -0.11% | 0.42% | 100% | 101% |

## Subjective results

An example of subjective comparisons are shown in Figure 3. By comparing Figure 3 (b) against Figure 3 (a), it is observed that blocky artifacts at maximum TB boundaries that do not coincide with any CB boundaries are reduced. This means the deblocking bugfix is subjectively helpful. By comparing Figure 3 (c) against Figure 3 (b), it is observed that the overall subjective quality is further improved. This means the long strong filtering is subjectively beneficial.

(a)

(b)

(c)

Figure 3.  The 3rd decoded picture of FoodMarket4, RA, QP=37; (a) BMS-1.0 (b) BMS-1.0 with the proposed deblocking bugfix (c) BMS-1.0 with the proposed DF including the deblocking bugfix

# Conclusions

This contribution proposes to add long DF in addition to HEVC DF. A deblocking bugfix is also proposed. The proposed DF including the deblocking bugfix leads to very minor changes in BD-rate and encoding time and slightly increases decoding time. Blocky artifacts are effectively reduced by the long DF, and thus the subjective quality of the proposed DF is better than the HEVC DF.

# References

- C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, H. Huang, S.-T. Hsiang, C.-C. Chen, M.-S. Chiang, C.-Y. Lai, C.-M. Tsai, Y.-C. Su, Z.-Y. Lin, Y.-L. Hsiao, J. Klopp, I.-H. Wang, Y.-W. Huang, S.-M. Lei, “Description of SDR video coding technology proposal by MediaTek,” Document of Joint Video Experts Team, JVET-J0018.

- BMS-1.0: https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-J1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).