| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0233-v1 |
| --- | --- |

| Title: | CE2.3.1: SAO with EO sign constraints removal and more EO patterns |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Chen-Yen Lai, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd, Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chingyeh.chen, jenny.lai, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution presents the test results of Core Experiment 2.3.1 (CE2.3.1), including edge offset (EO) sign constraints removal and more EO patterns. Removing EO sign constraints means signaling EO signs instead of using pre-defined signs as regulated in HEVC, and it achieves 0.00%, -0.04%, and -0.17% luma BD-rates for AI, RA, and LB on VTM-1.0. On BMS-1.0, 0.00%, 0.02%, and -0.07% luma BD-rates are achieved for AI, RA, and LB, respectively. Compared to VTM-1.0 with sample adaptive offset (SAO) off, SAO with EO sign constraints removal achieves -0.44%, -1.34%, and -1.29% luma BD-rates with 6%, 5%, and 2% decoding time increases, for VTM-AI, VTM-RA, and VTM-LB, respectively. Compared to BMS-1.0 with SAO off, it achieves -0.13%, -0.57%, and -1.16% luma BD-rates with 3%, 2%, and 2% decoding time increases, for BMS-AI, BMS-RA, and BMS-LB, respectively. SAO with removing EO sign constraints and adding three more EO patterns can achieve -0.48%, -1.42%, and -1.27% luma BD-rates with 6%, 4%, and 3% decoding time increases, for VTM-AI, VTM-RA, and VTM-LB, respectively, compared to VTM-1.0 with SAO off. Compared to BMS-1.0 with SAO off, it can achieve -0.16%, -0.63%, and -1.23% luma BD-rates, with 3%, 1%, and 2% decoding time increases, for BMS-AI, BMS-RA, and BMS-LB, respectively. There is no noticeable encoding time increase in VTM-1.0 and BTM-1.0.

Introduction

In HEVC, SAO supports 4 classes in EO, as shown in Figure 1 (a), (b), (c), and (d), and 32 different sets of selected bands in band offset (BO). EO sign constraints are further applied, and the signs of offsets in EO are pre-defined in order to allow smoothing and disallow sharpening. In the 10th JVET meeting, two SAO modifications were presented in JVET-J0018 [1] and planned in CE2.3.1, JVET-J1022 [2]. The first one is to remove the EO sign constraints. In this modification, the signs of offsets in EO can be explicitly signaled. The decision of EO signs can be made at the encoder. The other modification is to add additional three classes in EO, as shown in Figure 1 (e), (f), and (g), to increase coding efficiency.

Figure 1: Supported EO classes in SAO.

Experiment results

The proposed modifications have been implemented on top of BMS-1.0. The common test conditions [3] are used for evaluation. Note that the simulations in VTM test condition are performed by using VTM encoder configuration settings without MACRO changes in the software. Table 1 and Table 2 summarize the results. The anchors in Table 1 and Table 2 are VTM-1.0 without SAO and BMS-1.0 without SAO. Table 1 shows the results of SAO with EO sign constraints removal. Table 2 provides the results of SAO with EO sign constraints removal and three more EO patterns. Table 3 summarizes the results of SAO with EO sign constraints removal, and the anchor is SAO on. Table 4 summarizes the results of SAO with EO sign constraints removal and three more EO patterns on BMS-1.0, and the anchor is SAO on.

Table 1. SAO with EO sign constraints removal v.s. SAO off.

Table 2. SAO with EO sign constraints removal and three more EO patterns v.s. SAO off.

Table 3. SAO with EO sign constraints removal v.s. SAO on.

Table 4. SAO with EO sign constraints removal and three more EO patterns v.s. SAO on.

Conclusion

This contribution reports the simulation results of SAO with removing EO sign constraints and more EO patterns. Compared to VTM-1.0 without SAO, SAO with removing EO sign constraints achieves -0.44%, -1.34%, and -1.29% luma BD-rates, with 6%, 5%, and 2% decoding time increases, for VTM-AI, VTM-RA, and VTM-LB, respectively. It can achieve -0.13%, -0.57%, and -1.16% luma BD-rates, with 3%, 2%, and 2% decoding time increases, for BMS-AI, BMS-RA, and BMS-LB, respectively, compared to BMS-1.0 without SAO. Based on SAO with removing EO sign constraints, adding three more EO patterns can achieve -0.48%, -1.42%, and -1.27% luma BD-rates, with 6%, 4%, and 3% decoding time increases, for VTM-AI, VTM-RA, and VTM-LB, respectively, compared to VTM-1.0 without SAO. Compared to BMS-1.0 without SAO, it can achieve -0.16%, -0.63%, and -1.23% luma BD-rates, with 3%, 1%, and 2% decoding time increases, for BMS-AI, BMS-RA, and BMS-LB, respectively. There is no noticeable encoding time increase in VTM-1.0 and BMS-1.0. It is recommended to include SAO with removing EO sign constraints in the next VTM and BMS.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

References

[1] C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, H. Huang, S.-T. Hsiang, C.-C. Chen, M.-S. Chiang, C.-Y. Lai, C.-M. Tsai, Y.-C. Su, Z.-Y. Lin, Y.-L. Hsiao, J. Klopp, I.-H. Wang, Y.-W. Huang, S.-M. Lei (MediaTek), “Description of SDR video coding technology proposal by MediaTek,” Document of Joint Video Experts Team, JVET-J0018.

[2] L. Zhang, K. Andersson, C.-Y. Chen, “Description of Core Experiment 2 (CE2): In-loop filters,” Document of Joint Video Experts Team, JVET-J1022.

[3] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-J1010.