| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0235-v1 |
| --- | --- |

| Title: | CE2.4.2.2: CTB-based ALF with slice filter sets |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Yu-Chi Su, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd, Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chingyeh.chen, yu-chi.su, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

Coding tree block (CTB) based adaptive loop filter (ALF) with slice filter sets is proposed in JVET-J0018 and evaluated in this document as Core Experiment 2.4.2.2 (CE2.4.2.2). At slice level, one filter set of up to 16 filters can be signaled with a sample-level or block-level filter adaptation method. At CTB level, NewFilterMode, MergeFilterMode, or SliceFilterMode can be selected to achieve CTB-based ALF adaptation. The NewFilterMode signals a new filter and applies the new filter for the entire CTB. The MergeFilterMode copies a filter from a neighboring CTB and applies the copied filter for the entire CTB. The SliceFilterMode applies the filters of the signaled filter set at slice header for the entire CTB and performs the corresponding filter adaptation method within the current CTB. The proposed CTB-based ALF with slice filter sets can achieve -2.69%, -4.82%, and -4.52% luma BD-rates with 32%, 44%, and 35% decoding time increases, for VTM-AI, VTM-RA, and VTM-LB, respectively, compared to VTM-1.0. Compared to BMS-1.0 without ALF, it can achieve -2.17%, -4.57%, and -4.24% luma BD-rates with 23%, 27%, and 26% decoding time increases, for BMS-AI, BMS-RA, and BMS-LB, respectively. As for chroma components, the BD-rate reduction is more than 5%. The encoding time increase is less than 1% in VTM-1.0 and BMS-1.0.

Introduction

In the 10th JVET meeting, CTB-based ALF with slice filter sets was presented in JVET-J0018 [1] and planned in CE2.4.2.2, JVET-J1022 [2]. In the CTB-based ALF with slice filter sets, the reconstruction process is summarized as follows. First, a sample-level or block-level filter adaptation is performed for each to-be-processed sample or block to select a corresponding filter, if necessary. Next, filtering outputs are generated by applying corresponding filters to to-be-processed samples. ALF side information is signaled at slice level and CTB level. At slice header, one flag to indicate whether ALF is enabled or disabled, the selection of sample-level or block-level filter adaptation, coefficients of slice filters, and the supported CTB ALF modes, are signaled for each color component, separately. For each CTB, one on/off flag and the selected CTB ALF mode with related information are coded. In the following, more details will be introduced.

For each CTB, one on/off flag is signaled to indicate whether ALF is applied to the current CTB or not. If ALF is enabled for the current CTB, one of three CTB ALF modes, NewFilterMode, MergeFilterMode, and SliceFilterMode, can be selected and signaled. The number of supported CTB ALF modes is dependent on two flags signaled at slice header. One flag is used to indicate whether SliceFilterMode is allowed or not, and the other flag is used to indicate whether NewFilterMode and MergeFilterMode are allowed or not when SliceFilterMode is allowed. If the selected CTB ALF mode is NewFilterMode, then one new filter is signaled and used for all samples in the current CTB. If the selected CTB ALF mode is MergeFilterMode, then one filter is inherited from a neighboring CTB and used for all samples in the current CTB. If the selected CTB ALF mode is SliceFilterMode, then the slice filters and the corresponding filter adaptation method at slice header are used for all samples in the current CTB.

In the MergeFilterMode, if the selected neighboring CTB selects SliceFilterMode with multiple filters, the first filter in the neighboring CTB is used for the current CTB.

There is only one filter shape, and there are two coefficient symmetry characteristics, as shown in Figure 1. The filter shape is 9x9Cross+3x3Square with half symmetry or full symmetry. For one filter, 10 or 4 coefficients are required to be signaled according to the filter symmetry. The center coefficient is implicitly derived.

Figure 1. The filter shape in ALF with two coefficient symmetry characteristics. (a) 9x9Cross+3x3Square with half symmetry, 11 coefficients. (b) 9x9Cross+3x3Square with full symmetry, 5 coefficients.

At the slice header, besides the flag used to indicate whether ALF is enabled or disabled and the two flags used to select the supported CTB ALF modes, additional information related to slice filters is also signaled for each color component as follows. First, the selection of slice filters is signaled. An encoder can signal one new set of slice filters or signal one index to indicate which decoded set of slice filters is used for the current slice. The new set of slice filters will be added into the decoded sets of slice filters after applying ALF for the current slice. The number of decoded sets of slice filters is up to 6, and referencing these decoded sets of slice filters is disallowed when crossing random access points.

In one set of slice filters, multiple filters with one selected filter adaptation method are supported for luma component. For chroma components, only single filter without filter adaptation is allowed. Three filter adaptation methods, IntensitySA, SimilaritySA, and GeometricBA, are supported.

- Intensity sample adaptation (IntensitySA) is the same as the band offset (BO) in sample adaptive offset (SAO). The range of sample intensity is divided into 32 groups, and the group index is decided based on the intensity of to-be-processed sample.

- In similarity sample adaptation (SimilaritySA), neighboring samples in one 5x5 diamond shape are used to compare with the to-be-processed sample, as shown in Figure 2. The group index of the to-be-processed sample is first reset to zero. If the difference between a neighboring sample and the to-be-processed sample is larger than one pre-defined threshold, then the group index is increased by 1. If the difference between the neighboring sample and the to-be-processed sample is larger than two times of the pre-defined threshold, then additional 1 is added to the group index. Totally, there are 25 groups in SimilarySA.

- Geometric block adaptation (GeometricBA) is almost the same as the block classification with geometric transform in JEM ALF, except the size of activity window for one 2x2 block is reduced from 6x6 to 4x4. In this method, there are 25 groups at most.

Although the number of groups is up to 25 or 32 in different methods, the number of filters in one set of slice filters is limited to 16 at most. That is, the encoder merges successive groups to keep the number of merged groups smaller than or equal to 16.

Figure 2. The sample classification of SimilaritySA.

Experiment results

The proposed ALF has been implemented on top of BMS-1.0. The common test conditions [3] are used for evaluation. Note that the simulations in VTM test condition are performed by using VTM encoder configuration settings without MACRO changes in the software. In this CE test, several sub-tests are provided to evaluate different filter adaptation methods. Table 1 shows the results of CTB-based ALF with the three filter adaptation methods, IntensitySA, SimilaritySA, and GeometricBA. Table 2 shows the results of CTB-based ALF with IntensitySA and GeometricBA. Table 3 and Table 4 show the results of CTB-based ALF with GeometricBA only and CTB-based ALF with IntensitySA only, respectively. Table 5 shows the results of CTB-based ALF with single filter only at slice level, i.e. there is no filter adaptation in slice filter set.

Table 1. CTB-based ALF with IntensitySA, SimilaritySA, and GeometricBA.

Table 2. CTB-based ALF with IntensitySA and GeometricBA.

Table 3. CTB-based ALF with GeometricBA.

Table 4. CTB-based ALF with IntensitySA.

Table 5. CTB-based ALF with single filter in slice filter set.

Figures 3, 4, 5 and 6 show the comparison of CTB-based ALF with different filter adaptation methods in slice filter set and JEM ALF with/without pre-defined filters under different test conditions.

Figure 3. Comparison of CTB-based ALF with different filter adaptation methods in slice filter set and JEM ALF in VTM-RA.

Figure 4. Comparison of CTB-based ALF with different slice filter set and JEM ALF in VTM-LB.

Figure 5. Comparison of CTB-based ALF with different filter adaptation methods in slice filter set and JEM ALF in BMS-RA.

Figure 6. Comparison of CTB-based ALF with different filter adaptation methods in slice filter set and JEM ALF in BMS-LB.

Conclusion

This contribution reports the simulation results of CTB-based ALF with slice filter sets in CE2.4.2.2. CTB-based ALF with slice filter sets can achieve -2.69%, -4.82%, and -4.52% luma BD-rates with 32%, 44%, and 35% decoding time increases, for VTM-AI, VTM-RA, and VTM-LB, respectively, compared to VTM-1.0. Compared to BMS-1.0 without ALF, it can achieve -2.17%, -4.57%, and -4.24% luma BD-rates with 23%, 27%, and 26% decoding time increases, for BMS-AI, BMS-RA, and BMS-LB, respectively. As for chroma components, the BD-rate reduction is more than 5%. The encoding time increase is less than 1% in VTM-1.0 and BTM-1.0. It is recommended to include CTB-based ALF with slice filter sets in the next VTM and BMS.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

References

[1] C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, H. Huang, S.-T. Hsiang, C.-C. Chen, M.-S. Chiang, C.-Y. Lai, C.-M. Tsai, Y.-C. Su, Z.-Y. Lin, Y.-L. Hsiao, J. Klopp, I.-H. Wang, Y.-W. Huang, S.-M. Lei (MediaTek), “Description of SDR video coding technology proposal by MediaTek,” Document of Joint Video Experts Team, JVET-J0018.

[2] L. Zhang, K. Andersson, C.-Y. Chen, “Description of Core Experiment 2 (CE2): In-loop filters,” Document of Joint Video Experts Team, JVET-J1022.

[3] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-J1010.