| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0236-v1 |
| --- | --- |

| Title: | CE2.5.2: Non-local mean in-loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {jenny.lai, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution reports the results of Core Experiment 2.5.2 (CE2.5.2) on non-local mean (NLM) in-loop filters. Compared with VTM-1.0, the proposed method reportedly achieves -0.63%, -1.24%, and -1.00% luma BD-rates for the AI, RA, and LB, respectively, with 135%, 111%, and 79% decoding time increases. Compared with BMS-1.0, the proposed method reportedly achieves -0.31%, -0.57%, and -0.76% luma BD-rates for AI, RA, and LB, respectively, with 67%, 39%, and 38% decoding time increases. No coding gain shift between luma and chroma is applied, and the chroma BD-rate savings are approximately 1.5%, 3.5%, and 4.0% for AI, RA, and LB, respectively. The encoding time increases are smaller than 1%.

Introduction

NLM is often applied in image denoising. In JVET-J0018 [1], NLM is utilized as one in-loop filter to reduce decoded sample distortions. This document reports NLM results as planned in CE2.5.2 [2].

Basic flow of proposed method

The proposed NLM is applied after deblocking filter (DF) and before sample adaptive offset (SAO). First, the current picture is divided into non-overlapping 8x8 patches, and each sample in the picture is given an accumulated sample weight (AccSampleWeight) reset to zero and an accumulated sample value (AccSampleValue) reset to zero. Based on the received on/off flags, NLM is performed on those enabled patches. There are two stages in NLM: block matching stage and denoising stage. In the block matching stage, for each enabled patch, either multi-subsampling pattern based search (MSPS) with search region equal to [-16, 16] x [-16, 16] samples or neighbor-based fast search (NBFS) which references neighboring patches’ disparity vectors (DVs) is used. Only luma samples are involved in the block matching stage. The matching criterion is sum of squared differences (SSD) between current patch samples and reference patch samples. After 15 best reference patches are found, the current patch and the 15 best reference patches are formed as one patch group. In the denoising stage, each patch in the patch group is given a weight according to the SSDs and other given parameters. Next, a SampleValue and a SampleWeight for each sample in the current patch and the 15 best reference patches are derived according to patch sample values and patch weights and then accumulated to corresponding AccSampleValue and AccSampleWeight, respectively. After the entire picture is processed as described above in this section, each sample is normalized by dividing AccSampleValue by AccSampleWeight to obtain the final NLM output sample value.

Syntax design of proposed method

The NLM side information of each color component includes two parts, parameter selection in noise model and multi-level on/off control flags. The former is used to decide the filtering strength of NLM and coded in slice header; the latter is used to indicate whether NLM is enabled or disabled for each current patch and signaled at slice and coding tree block (CTB) levels. In slice header, one syntax element is signaled to indicate the on/off mode selection among {SliceAllOff, SliceAllOn, CTBOnOff}. If SliceAllOff is selected, NLM is disabled for the current component in current slice. If SliceAllOn is selected, NLM is applied to all samples of the current component in the current slice. If CTBOnOff is selected, whether NLM is applied or not depends on the syntax elements signaled at CTB level. At CTB level, there are also three on/off modes {CTBAllOff, CTBAllOn, BlockOnOff}. If CTBAllOff is selected, NLM is disabled for the current component in the current CTB. If CTBAllOn is selected, NLM is applied to all samples of the current component in the current CTB. If BlockOnOff is selected, whether NLM is applied or not depends on the on/off flags signaled at 32x32 block level.

Decoding process of proposed method

In the block matching stage, 15 best reference patches are found by using multi-subsampling pattern search (MSPS) and neighbor-based fast search (NBFS). In a 32x32 block, there are 16 non-overlapping 8x8 patches. MSPS are applied to the six white patches, and NBFS are applied to the rest 10 patches, as shown in Figure 1. Patches with MSPS are processed before patches with NBFS.

Figure 1. MSPS for the six white 8x8 patches, NBFS for the 10 dark 8x8 patches.

In MSPS, the search region includes [-16, 16] x [-16, 16] samples centered at zero DV and is divided into two sub-regions with different subsampling ratios. Sub-region 1 is within [-4, 4] x [-4, 4] and searched by using a search position subsampling ratio equal to 2 in each direction (DV[x] % 2 == 0 && DV[y] % 2 == 0). Sub-region 2 ([-16, 16] x [-16, 16] excluding sub-region 1) is searched by using a search position subsampling ratio equal to 4 (DV[x] % 4 == 0 && DV[y] % 4 == 0). After finding 15 initial best candidates, a refinement process is performed. For initial best candidates located in sub-region 1, a refinement range, [-1, 1] x [-1, 1], centered at the initial best candidate, is applied. For initial best candidates located in sub-region 2, a refinement range, [-2, 2] x [-2, 2], centered at the initial best candidate, is applied. To reduce computation complexity, an early termination check before the DV refinement process is done. If the distortion of the initial best candidate is much larger than the worst distortion of the up-to-date 15 best candidates, the DV refinement process will be early terminated.

NBFS is a predictor-based fast search algorithm. After MSPS process, 15 best matched reference patches of each of the six processed 8x8 patches are available, and the each of the 10 NBFS 8x8 patches will find 15 initial best candidates from all MSPS 8x8 patches’ DVs (15x6=90 in total). A refinement process is performed for each initial best candidate. The refinement algorithm is the same as that in MSPS and the early termination check is also applied for complexity reduction.

In the above paragraphs of this section, only integer DVs are searched, and the search algorithm is only applied to luma component. For chroma components, luma DVs are reused after performing DV scaling according to the chroma subsampling format.

In the denoising stage, all samples in one patch group can be modified. The SampleValue and the SampleWeight of each sample in the current patch are generated by the following equations.

SampleValue[m, n]= i=015weighti[m, n]*RecSampleim, n

SampleWeightm, n=i=015weighti[m, n]

In the above equations, [m, n] is the position of a to-be-processed sample in a patch, RecSampleim, n is the sample value at position [m, n] in the ith patch before NLM (i=0 means current patch, i>0 means a reference patch), weighti[m, n] is the corresponding weight of to-be-processed samples. As for each sample in the 15 best reference patches, the SampleValue and SampleWeight are generated by the following equations,

SampleValue[m, n]= weighti[m, n]*RecSample0m, n

SampleWeightm, n=weighti[m, n]

The weight, weightim, n, is derived based on the estimated quantization noise and SSD between the current patch and the ith reference patch. In order to estimate the quantization noise, one compression noise model is introduced, and the quantization parameter (QP), slice type (I or B or P), and standard deviation of the current patch are used in this model. To reduce the computation complexity of denoising process, if the corresponding weight of to-be-processed samples is too small, the weight is set to zero, so the operations in the above equations can be reduced in practical.

To further improve denoising performance, multi-compression noise model is used to adaptively estimate the quantization noise. An index is signaled to select the most efficient compression noise model for current slice. Only luma component is applied with multi-compression noise model.

As described in Section 2, after each 8x8 patch and its corresponding patch group in the entire picture are processed (i.e., calculate SampleValue and SampleWeight and perform accumulation to corresponding AccSampleValue and AccSampleWeight for every sample in all patch groups), a normalization process is done for each sample in the current picture to obtain the final NLM output sample value.

Experimental results

The proposed method has been integrated into BMS-1.0. All the tests were conducted according to the common test conditions [3]. The proposed NLM achieves -0.31%, -0.57%, and -0.76% luma BD-rates for AI, RA, and LB, respectively, compared with BMS-1.0, with corresponding decoding time increases as 67%, 39%, and 38%. Compared with VTM-1.0, -0.63%, -1.24%, and -1.00% luma BD-rates are achieved for AI, RA, and LB, respectively, with corresponding decoding time increases as 135%, 111%, and 79%. No coding gain shift between luma and chroma is applied. Chroma BD-rate savings are much higher than luma BD-rate savings and are 1.5%-2.0% for AI, about 3.5% for RA, and about 4.0% for LB. The impact on encoding time is about 1% on VTM-1.0, and less than 1% on BMS-1.0. The detail results are shown in Table 1.

Table 1. Results of the proposed NLM

Conclusion

NLM is effective in improving coding efficiency, but its decoding complexity issue is not yet solved. Further research on NLM complexity reduction is encouraged.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

References

[1] C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, H. Huang, S.-T. Hsiang, C.-C. Chen, M.-S. Chiang, C.-Y. Lai, C.-M. Tsai, Y.-C. Su, Z.-Y. Lin, Y.-L. Hsiao, J. Klopp, I.-H. Wang, Y.-W. Huang, S.-M. Lei (MediaTek), “Description of SDR video coding technology proposal by MediaTek,” Document of Joint Video Experts Team, JVET-J0018.

[2] L. Zhang, K. Andersson, C.-Y. Chen, “Description of Core Experiment 2 (CE2): In-loop filters,” Document of Joint Video Experts Team, JVET-J1022.

[3] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-J1010.