| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0238-v2 |
| --- | --- |

| Title: | CE2-related: Improvements of sample adaptive offset |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {jenny.lai, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports two modifications on sample adaptive offset (SAO). One modification is a greedy SAO merge encoding algorithm that explores more SAO merge possibilities across multiple coding tree units (CTUs) to enhance coding efficiency. The other modification is to reduce redundancy in offset coding of SAO. Simulation results are shown as follows.

Modification 1: Applying a greedy SAO merge encoding algorithm (negligible runtime changes)
VTM-1.0-AI: {Y-BD-rate = -0.07%}
VTM-1.0-RA: {Y-BD-rate = -0.13%}
VTM-1.0-LB: {Y-BD-rate = -0.14%}
BMS-1.0-AI: {Y-BD-rate = -0.06%}
BMS -1.0-RA: {Y-BD-rate = -0.08%}
BMS -1.0-LB: {Y-BD-rate = -0.23%}

Modification 2: Reducing redundancy in offset coding of SAO (negligible runtime changes)
VTM-1.0-AI: {Y-BD-rate = 0.00%}
VTM-1.0-RA: {Y-BD-rate = -0.01%}
VTM-1.0-LB: {Y-BD-rate = 0.01%}
BMS-1.0-AI: {Y-BD-rate = 0.00%}
BMS -1.0-RA: {Y-BD-rate = 0.00%}
BMS -1.0-LB: {Y-BD-rate = -0.09%}

# Introduction

Two experiments are tested for improving SAO design in this contribution, including SAO encoding algorithm, and redundancy reduction in offset coding.

# Algorithm description

## Applying a greedy SAO merge encoding algorithm

In the reference software (HM, VTM-1.0, BMS-1.0), the SAO encoder processes one CTU by one CTU. To improve coding efficiency, a greedy SAO merge encoding algorithm is applied. A set of CTUs can be grouped together to jointly optimize SAO parameters across multiple CTUs. The grouping is constrained within each CTU row, i.e, no grouping is tested across CTU rows. In a CTU group of SAO, SAO parameters are signaled at the first CTU in the group, and the rest CTUs in the group are coded with SAO merge-left mode. Compared to the encoding algorithm in the reference software, the difference is that the SAO parameters are re-derived to consider multiple CTUs when merge-left mode is tested, instead of directly reusing the SAO parameters of the previous CTU.

## Reducing redundancy in offset coding of SAO

SAO supports 4 classes in edge offset (EO) and 32 different sets of selected bands in band offset (BO). If SAO merge is not selected, in a luma CTB four offsets of EO or BO are signaled, and in the two chroma CTBs eight offsets are signaled. The current offset syntax allows all offsets to be zero. When SAO is enabled, it makes more sense to have at least one nonzero offset. In order to reduce the redundancy of all zero offsets, for EO mode, the last luma offset magnitude minus one is coded if all previous three coded luma offsets are zero, and the last chroma offset magnitude minus one is coded if all previous seven coded chroma offsets are zero. For BO mode, the first offset in the starting band should be nonzero, and the first offset magnitude minus one for BO mode is coded.

# Experimental results

The proposed method has been integrated into BMS-1.0. All the tests were conducted according to the common test conditions [3]. Table 1 and Table 2 show the results of applying a greedy SAO merge encoding algorithm and reducing redundancy in offset coding of SAO, respectively. Run time data in Table 1 is accurate, but run time data in Table 2 is not accurate. It is claimed that the proposed methods do not cause significant impact on run time.

Table 1.  Results of applying a greedy SAO merge encoding algorithm

Table 2.  Results of reducing redundancy in offset coding of SAO (run time not accurate)

# Conclusions

Two SAO modifications are tested. Applying a greedy SAO merge encoding algorithm can achieve 0.1% to 0.2% BD-rate savings without any normative changes. Reducing redundancy in offset coding of SAO can achieve minor BD-rate savings (<0.1%).

# References

- BMS-1.0: https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-J1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).