| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0239-v1 |
| --- | --- |

| Title: | CE2-related: Filter size reduction in CTB-based ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chi Su, Ching-Yeh Chen Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd, Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yu-chi.su, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

Based on CE2.4.2.2, a 9x7cross+3x3square filter shape is proposed for CTB-based adaptive loop filter (ALF) for reducing two line buffers in comparison with 9x9cross+3x3square. Compared with VTM-1.0, the 9x7cross+3x3square CTB-based ALF can achieve -2.63%, -4.78%, and -4.45% luma BD-rates with 30%, 39%, and 30% decoding time increases, for AI, RA, and LB, respectively. Compared with BMS-1.0 with ALF disabled, the 9x7cross+3x3square CTB-based ALF can achieve -2.12%, -4.52%, and -4.16% luma BD-rates with 23%, 25%, and 23% decoding time increases for AI, RA, and LB, respectively. Compared with 9x9cross+3x3square CTB-based ALF under VTM-1.0 configuration, 9x7cross+3x3square CTB-based ALF achieves 0.06%, 0.04%, and 0.08% luma BD-rates with 2%, 4%, and 4% decoding time decreases for AI, RA, and LB, respectively. Compared with 9x9cross+3x3square CTB-based ALF under BMS-1.0 configuration, 9x7cross+3x3square CTB-based ALF achieves 0.05%, 0.05%, and 0.08% luma BD-rates with 0%, 1%, and 2% decoding time decreases for AI, RA, and LB, respectively.

# Introduction

In JVET-J0018 [1], CTB-based ALF with slice filter sets was proposed and planned in CE2.4.2.2 as described in JVET-J1022 [2]. In the CTB-based ALF, there is only one filter shape, and there are two coefficient symmetry characteristics, as shown in Figure 1. The filter shape is 9x9cross+3x3square with half symmetry or full symmetry. For one filter, 10 or four coefficients are required to be signaled according to the filter symmetry. The center coefficient is implicitly derived.

Figure 1. The filter shape in ALF with two coefficient symmetry characteristics. (a) 9x9cross+3x3square with half symmetry, 11 coefficients. (b) 9x9cross+3x3square with full symmetry, 5 coefficients.

In the CTB-based ALF, one filter set of up to 16 filters with a sample-level or block-level filter adaptation method can be signaled at slice level. Three filter adaptation methods, IntensitySA, SimilaritySA, and GeometricBA, are supported.

- Intensity sample adaptation (IntensitySA): The range of sample intensity is divided into 32 groups, and the corresponding slice-level filter for each group is derived. For each to-be-processed sample, the slice-level filter can be determined based on the intensity of the sample.

- Similarity sample adaptation (SimilaritySA): The to-be-processed sample can be applied by a slice-level filter based on the similarity between the to-be-processed sample and its neighboring samples. In this method, there are 25 groups at most.

- Geometric block adaptation (GeometricBA): The filter adaptation method is almost the same as the block classification with geometric transform in JEM ALF, except the size of activity window for one 2x2 block is reduced from 6x6 to 4x4 samples. In this method, there are 25 groups at most. The 9x9cross+3x3square ALF filter can be rotated in four directions as shown in Figure 2.

Figure 2. The 9x9cross+3x3square ALF rotated in four directions.

# Proposed method

In this contribution, a 9x7cross+3x3square ALF with half symmetry or full symmetry is proposed to reduce line buffers. Figure 3 shows the 9x7cross+3x3square ALF with half symmetry and full symmetry. For one filter, nine or four coefficients are required to be signaled according to the filter symmetry. The center coefficient is implicitly derived.

Figure 3. The filter shape in ALF with two coefficient symmetry characteristics. (a) 9x7cross+3x3square with half symmetry, 10 coefficients. (b) 9x7cross+3x3square with full symmetry, 5 coefficients.

The 9x7cross+3x3square ALF filter can be rotated in four directions as shown in Figure 4.

Figure 4. The 9x7cross+3x3square ALF rotated in four directions.

# Experimental results

The proposed ALF has been implemented on top of BMS-1.0 [3]. Table 1 shows the performance of 9x7cross+3x3square CTB-based ALF under VTM-1.0 and BMS-1.0 configurations. In Table 1, the test enables IntensitySA and GeometricBA and disables SimilaritySA in CTB-based ALF. Table 2 shows the results of the 9x7cross+3x3square CTB-based ALF compared against the 9x9cross+3x3square CTB-based ALF under VTM-1.0 and BMS-1.0 configurations. In Table 2, both the test and the anchor enable IntensitySA and GeometricBA and disable SimilaritySA in CTB-based ALF.

Table 1. 9x7cross+3x3square CTB-based ALF with IntensitySA and GeometricBA versus ALF off.

Table 2. 9x7cross+3x3square CTB-based ALF with IntensitySA and GeometricBA versus 9x9cross+3x3square CTB-based ALF with IntensitySA and GeometricBA.

# Conclusions

In this contribution, the 9x7cross+3x3square CTB-based ALF causes BD-rate increase of less than 0.1% in comparison with 9x9cross+3x3square CTB-based ALF, and two sample line buffers can be saved.

# Patent rights declaration

MediaTek, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, H. Huang, S.-T. Hsiang, C.-C. Chen, M.-S. Chiang, C.-Y. Lai, C.-M. Tsai, Y.-C. Su, Z.-Y. Lin, Y.-L. Hsiao, J. Klopp, I.-H. Wang, Y.-W. Huang, S.-M. Lei, “Description of SDR video coding technology proposal by MediaTek,” Document of Joint Video Experts Team, JVET-J0018.

[2] L. Zhang, K. Andersson, C.-Y. Chen, “Description of Core Experiment 2 (CE2): In-loop filters,” Document of Joint Video Experts Team, JVET-J1022.

[3] BMS-1.0 reference software, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/