| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0240-v1 |
| --- | --- |

| Title: | CE3.2.9.1: Intra boundary filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {zhi-yi.lin, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents the test results of Core Experiment 3.2.9.1 (CE3.2.9.1), including DC filter, gradient filter, and bi-boundary filter. DC filter is applied to intra coding units (CUs) coded by DC mode. Gradient filter is applied to intra CUs coded with Vertical (Ver), Ver-1, Ver+1, Horizontal (Hor), Hor-1, or Hor+1 mode. Bi-boundary filter is applied to intra CUs whose intra prediction direction is in the range [3, 10] or the range [57, 64], when intra prediction direction modes are defined in the context of 65 angular modes. These filters are applied to both luma and chroma components, and the number of filtered lines in DC filter and gradient filter depends on CU width or CU height, while the number of filtered lines for bi-boundary filter is fixed to 4. The simulation result shows that -0.75% and -0.38% luma BD-rates can be achieved for VTM-1.0-AI, and VTM-1.0-RA, respectively. There are 4% and -1% encoding time increases for VTM-1.0-AI and VTM-1.0-RA, respectively. In BMS-1.0 conditions, -0.87% and -0.41% luma BD-rates can be achieved for AI and RA, respectively. There are 1% and 4% encoding time increases for BMS-1.0-AI and BMS-1.0-RA, respectively. Little decoding time changes are reported in VTM-1.0-AI, VTM-1.0-RA, BMS-1.0-AI and BMS-1.0-RA.

# Introduction

In HEVC, to reduce the boundary artifacts of intra CU, when CU width and height are smaller than or equal to 16, DC filter is applied to intra CUs coded by DC mode, and gradient filter is applied to intra CUs coded by Hor and Ver directions. The number of filtered lines is fixed to 1, and only luma component is filtered. Several modifications of intra boundary filters were presented in JVET-J0018 [1] and planned in CE3.2.9.1 as described JVET-J1023 [2]. The modifications involve DC filter, gradient filter, and bi-boundary filter. These filters are applied to luma component and chroma components.

# Algorithm description

For intra CUs coded by DC mode, the number of filtered columns and the number of filtered rows depend on current CU width and height, respectively. For CUs with width equal to 4, 8, 16, 32, and 64, the numbers of filtered columns are 1, 2, 2, 4, and 4, respectively. Similarly, for CUs with height equal to 4, 8, 16, 32, and 64, the numbers of filtered rows are 1, 2, 2, 4, and 4 respectively. An example of intra 8x4 CU is shown in Figure 1.

The weight is derived according to the distance between to-be-filtered sample and the CU boundary, which can be summarized in Table 1. DC filter applies 2-tap filtering to samples near horizontal and vertical boundaries and 3-tap filtering to samples with i equal to j, where i and j are the horizontal and vertical displacements between current sample and current CU’s left-top sample respectively.

Figure 1.  DC filtering on an 8x4 CU; one row and two columns are filtered.

Table 1.  Weights for DC filter in different filtered line.

|  | i > j or i < j (Above/Left, Current) | i == j (Above, Left, Current) |
| --- | --- | --- |
| line0 | (1/4, 3/4) | (1/4, 1/4, 2/4) |
| line1 | (1/8, 7/8) | (1/8, 1/8, 6/8) |
| line2 | (1/16, 15/16) | (1/16, 1/16, 14/16) |
| line3 | (1/32, 31/32) | (1/32, 1/32, 30/32) |

For intra CUs coded with Ver, Ver-1, Ver+1, Hor, Hor-1, or Hor+1, gradient filter is applied. Similar to DC filter, for CUs with width equal to 4, 8, 16, 32, and 64, the numbers of filtered columns are 1, 2, 2, 4, and 4, respectively. Similarly, for CUs with height equal to 4, 8, 16, 32, and 64, the numbers of filtered rows are 1, 2, 2, 4, and 4, respectively. For the gradient filter with multiple filtered lines, the weight is derived according to not only intra prediction mode but also the distance between to-be-filtered sample and the boundary. Figure 2 is one example of one 8x4 CU coded with Ver, Ver-1, or Ver+1 mode. The filtered samples are the weighted sum of above reference sample and the gradient between above-left reference sample and left reference sample. The equations to derive the filtered samples for Ver, Ver-1, and Ver+1 are listed in Table 2. Another example for 8x4 CU coded with Hor, Hor-1, or Hor+1 mode is shown in Figure 3. The filtered samples are the weighted sum of left reference sample and the gradient between above-left reference sample and above reference sample. The equations to derive the filtered samples for Hor, Hor-1, and Hor+1 are listed in Table 3.

Figure 2.  Gradient filtering on an 8x4 intra CU with Ver, Ver-1, or Ver+1. Two columns are filtered.

Table 2.  Equations for gradient filter in different position for Ver, Ver-1 and Ver+1.

|  | Vertical | Vertical +1, Vertical -1 |
| --- | --- | --- |
| line0 |  |  |
| line1 |  |  |
| line2 |  |  |
| line3 |  |  |

Figure 3.  Gradient filtering on an 8x4 intra CU with Hor, Hor -1, or Hor +1. One row is filtered.

Table 3.  Equations for gradient filter in different position for Hor, Hor -1, and Hor +1.

|  | Horizontal | Horizontal +1, Horizontal -1 |
| --- | --- | --- |
| line0 | L0+Ai-AL≫1 | L0+Ai-AL≫2 |
| line1 | L1+Ai-AL≫2 | L1+Ai-AL≫3 |
| line2 | L2+Ai-AL≫3 | L2+Ai-AL≫4 |
| line3 | L3+Ai-AL≫4 | L3+Ai-AL≫5 |

Assume that all intra directions are defined in the context of 65 angular modes. For intra CUs with intra direction mode in the range of [3, 10] or [57, 64], bi-boundary filter will be applied. In this proposal, the number of filtered lines is fixed to 4 for all sizes of CUs. An example of one 8x4 CU is shown in Figure 4, the filtered samples are the weighted sum of main-reference sample and side-reference sample. The weight depends on the distance between to-be-filtered sample and the boundary, which are listed in Table 4.

Figure 4.  Bi-boundary filtering on an 8x4 intra CU. Four columns are filtered.

Table 4.  Weights for bi-boundary filter in different filtered lines.

|  | (main-reference , side-reference) |
| --- | --- |
| line0 | (1/2, 1/2) |
| line1 | (3/4, 1/4) |
| line2 | (7/8, 1/8) |
| line3 | (15/16, 1/16) |

# Experiment results

The proposed modifications have been implemented on top of BMS-1.0. The common test conditions [3] are used for evaluation. Note that the simulations in VTM-1.0 test condition are performed by using VTM encoder configuration settings without MACRO changes in the software. Table 5 summarizes the results.

Table 5.  Simulation results of intra boundary filters.

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.65% | -0.70% | -0.54% | 105% | 99% | -0.66% | -0.51% | -0.46% | 102% | 99% |
| Class A2 | -0.79% | -0.89% | -0.76% | 104% | 100% | -0.93% | -0.66% | -0.81% | 101% | 101% |
| Class B | -0.74% | -0.58% | -0.60% | 105% | 97% | -0.86% | -0.50% | -0.42% | 100% | 99% |
| Class C | -0.69% | -0.46% | -0.74% | 104% | 101% | -0.78% | -0.48% | -0.55% | 101% | 100% |
| Class E | -0.93% | -0.96% | -1.19% | 104% | 101% | -1.16% | -0.66% | -0.84% | 100% | 98% |
| Overall | -0.75% | -0.69% | -0.75% | 104% | 99% | -0.87% | -0.55% | -0.59% | 101% | 99% |
| Class D | -0.65% | -0.40% | -0.25% | 103% | 102% | -0.61% | -0.21% | -0.22% | 99% | 98% |
| Class F (optional) | 0.55% | 0.69% | 1.06% | 105% | 103% | 0.41% | 0.39% | 0.33% | 102% | 103% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.36% | -0.43% | -0.37% | 100% | 101% | -0.32% | -0.49% | -0.50% | 103% | 98% |
| Class A2 | -0.34% | -0.34% | -0.07% | 97% | 98% | -0.41% | -0.25% | -0.35% | 104% | 98% |
| Class B | -0.39% | -0.11% | -0.25% | 99% | 101% | -0.49% | -0.19% | -0.31% | 104% | 100% |
| Class C | -0.42% | -0.07% | -0.27% | 100% | 98% | -0.39% | -0.36% | -0.70% | 103% | 104% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.38% | -0.21% | -0.24% | 99% | 99% | -0.41% | -0.31% | -0.46% | 104% | 100% |
| Class D | -0.27% | 0.09% | 0.06% | 102% | 99% | -0.27% | -0.28% | -0.15% | 105% | 109% |
| Class F (optional) | 0.27% | 0.55% | 0.75% | 102% | 102% | 0.12% | 0.17% | 0.38% | 104% | 109% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.18% | -0.20% | -0.04% | 101% | 101% | -0.21% | -0.43% | -0.34% | 100% | 101% |
| Class C | -0.13% | -0.36% | 0.06% | 101% | 100% | -0.21% | -0.24% | -0.55% | 101% | 99% |
| Class E | -0.07% | 0.18% | 0.25% | 98% | 98% | -0.43% | -0.69% | -0.53% | 102% | 104% |
| Overall | -0.14% | -0.16% | 0.07% | 100% | 100% | -0.26% | -0.43% | -0.46% | 101% | 101% |
| Class D | -0.13% | 0.04% | -0.49% | 100% | 100% | -0.06% | -0.03% | -0.56% | 102% | 102% |
| Class F (optional) | 0.05% | 0.15% | 0.19% | 100% | 99% | -0.03% | -0.13% | 0.04% | 102% | 103% |

# Conclusions

In CE3.2.9.1, intra boundary filtering including DC filter, gradient filter, and bi-boundary filter, is studied. DC filter is applied to intra CUs coded with DC mode. Gradient filter is applied to intra CUs coded with Ver, Ver-1, Ver+1, Hor, Hor-1, or Hor+1 mode. Bi-boundary filter is applied to intra CUs whose intra prediction direction is in the range of [3, 10] or [57, 64], when intra prediction direction modes are defined in the context of 65 angular modes. These filters are applied to both luma and chroma components, and the number of filtered lines in DC filter and gradient filter depends on CU width or CU height, while the number of filtered lines for bi-boundary filter is fixed to 4. The results show that -0.75%, -0.38%, and -0.14% luma BD-rates can be achieved for VTM-1.0-AI, VTM-1.0-RA, and VTM-1.0-LB, respectively. There are 4% and -1% encoding time increases for VTM-1.0-AI and VTM-1.0-RA, respectively. In BMS-1.0 conditions, -0.87%, -0.41%, and -0.26% luma BD-rates can be achieved for AI, RA, and LB, respectively. There are 1% and 4% encoding time increases for BMS-1.0-AI and BMS-1.0-RA, respectively. Little decoding time changes are reported in VTM-1.0-AI, VTM-1.0-RA, BMS-1.0-AI and BMS-1.0-RA.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, H. Huang, S.-T. Hsiang, C.-C. Chen, M.-S. Chiang, C.-Y. Lai, C.-M. Tsai, Y.-C. Su, Z.-Y. Lin, Y.-L. Hsiao, J. Klopp, I.-H. Wang, Y.-W. Huang, S.-M. Lei, “Description of SDR video coding technology proposal by MediaTek,” Document of Joint Video Experts Team, JVET-J0018.

[2] G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra prediction and mode coding,” Document of Joint Video Experts Team, JVET-J1023.

[3] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-J1010.