| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0241-v1 |
| --- | --- |

| Title: | CE3.4.4: Additional LM-based modes for intra chroma prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Chih-Wei Hsu, Ching-Yeh Chen, Tzu-Der Chuang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, cw.hsu, chingyeh.chen, peter.chuang, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

In this contribution, four additional linear model (LM) based modes presented in JVET-J0018 are tested, including LM-left, LM-top, LM-CbCr, and LM-fusion modes. Different combinations of LM-based modes are tested, and the Y, U, V BD-rates, encoding time, and decoding time in All Intra (AI) configuration are reported as follows.

Compared with AI VTM-1.0; anchor has no LM modes

[CE3.4.4.1] LM+LM-left+LM-top: {-1.54% (Y), -10.75% (U), -11.64% (V), EncT=121%, DecT=102%}

[CE3.4.4.2] LM+LM-CbCr: {-1.50% (Y), -10.16% (U), -11.23% (V), EncT=119%, DecT=102%}

[CE3.4.4.3] LM+LM-fusion: {-1.18% (Y), -11.17% (U), -11.37% (V), EncT=121%, DecT=102%}

[CE3.4.4.4] LM+4 new LM modes: {-1.19% (Y), -12.09% (U), -12.62% (V), EncT=121%, DecT=102%}

Compared with AI BMS-1.0; anchor has LM, multi-model LM (MMLM), and multi-filter LM (MFLM)

[CE3.4.4.1] LM+LM-left+LM-top: {0.14% (Y), -0.01% (U), 0.20% (V), EncT=88%, DecT=98%}

[CE3.4.4.2] LM+LM-CbCr: {0.18% (Y), 0.69% (U), 0.73% (V), EncT=87%, DecT=98%}

[CE3.4.4.3] LM+LM-fusion: {0.52% (Y), -0.52% (U), 0.21% (V), EncT=87%, DecT=101%}

[CE3.4.4.4] LM+4 new LM modes: {0.53% (Y), -1.59% (U), -1.09% (V), EncT=98%, DecT=101%}

Introduction

A cross-component linear model (CCLM) mode is proposed in COM16–C806 [1] for reducing the cross-components redundancy. CCLM includes two items; one is the conventional LM as in HM-7.0, and the other is Cb-to-Cr residual prediction for non-LM intra chroma modes. In addition to the CCLM mode, four additional LM-based modes were proposed in JVET-J0018 [2] and tested in CE3. The four additional LM-based modes are named as LM-left, LM-top, LM-CbCr, and LM-fusion.

Proposed methods

LM-left and LM-top modes

As shown in Figure 1, LM-left mode uses four left luma columns and two left chroma columns of neighboring coding block (CB) reconstructed samples to determine the linear model parameters between down-sampled luma and chroma components. Similarly, LM-top mode uses four above luma rows and two above chroma rows of neighboring CB reconstructed samples to derive the linear model parameters. The down-sampling of neighboring reconstructed luma samples and linear model derivation process of LM-left and LM-top modes are the same as the LM in BMS-1.0.

In addition, when above neighboring reconstructed samples are not available for current CU, then LM and LM-top modes are implicitly forbidden. Similarly, when left neighboring reconstructed samples are not available for current CU, then LM and LM-left modes are implicitly forbidden.

| (a) | (b) |
| --- | --- |
| Figure 1. Additional LM-based chroma modes using left columns or above rows of samples. (a) LM-left mode; (c) LM-top mode. |  |

LM-CbCr mode

As shown in Figure 2, LM-CbCr mode derives two separate linear models. One is between the neighboring reconstructed luma and Cb components for predicting Cb samples from luma samples, and the other is between the neighboring reconstructed Cb and Cr components for predicting Cr samples from Cb samples. The model derivation process of LM-CbCr mode is the same as the LM mode in BMS-1.0.

Figire 2. LM-CbCr mode derives two separate linear models.

LM-fusion mode

LM-fusion mode is to combine the predictions from LM and non-LM modes as a new prediction,

| , | (1) |
| --- | --- |

where  P1and  are the predictions from LM and non-LM modes, respectively. One flag is used right after a non-LM mode is signaled to indicate whether to apply the LM-fusion mode or not.

Experimental results

The proposed additional LM-based intra chroma modes has been implemented on BMS-1.0 [3], and results of four CE tests (i.e., CE3.4.4.1, CE3.4.4.2, CE3.4.4.3, and CE3.4.4.4) in AI and RA configurations are summarized in Table 1, Table 2, Table 3 and Table 4, respectively. The simulations were performed following the JVET common test conditions (CTC) [4]. More detailed results can be found in the accompanying Excel files.

Table 1. Results of CE3.4.4.1: LM + LM-left + LM-top

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -4.18% | -23.73% | -21.67% | 120% | 101% | 0.38% | 0.01% | 0.82% | 87% | 97% |
| Class A2 | -1.63% | -9.84% | -5.21% | 122% | 103% | 0.09% | 0.06% | 0.83% | 88% | 99% |
| Class B | -0.64% | -8.02% | -11.58% | 121% | 102% | 0.13% | 0.30% | 0.07% | 88% | 98% |
| Class C | -1.44% | -9.99% | -12.77% | 121% | 103% | 0.05% | -0.12% | 0.02% | 90% | 98% |
| Class E | -0.42% | -4.26% | -6.65% | 121% | 102% | 0.07% | -0.46% | -0.59% | 88% | 99% |
| Overall | -1.54% | -10.75% | -11.64% | 121% | 102% | 0.14% | -0.01% | 0.20% | 88% | 98% |
| Class D | -0.97% | -8.05% | -9.35% | 120% | 102% | 0.06% | -0.17% | -0.25% | 90% | 98% |
| Class F (optional) | -2.67% | -9.12% | -10.01% | 121% | 104% | 0.42% | 3.38% | 3.86% | 90% | 98% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.56% | -20.48% | -22.00% | 104% | 101% | 0.53% | 0.55% | 2.06% | 96% | 98% |
| Class A2 | -0.60% | -9.97% | -4.16% | 102% | 105% | 0.05% | 0.09% | 0.61% | 97% | 101% |
| Class B | -0.31% | -10.75% | -13.74% | 103% | 100% | 0.09% | 0.34% | 0.05% | 97% | 101% |
| Class C | -0.52% | -9.72% | -11.47% | 103% | 101% | 0.10% | -0.30% | -0.37% | 97% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.87% | -12.26% | -12.87% | 103% | 101% | 0.17% | 0.16% | 0.45% | 97% | 100% |
| Class D | -0.32% | -8.36% | -9.41% | 103% | 101% | 0.09% | -0.71% | -0.16% | 97% | 99% |
| Class F (optional) | -1.48% | -8.21% | -8.02% | 104% | 101% | 0.26% | 2.72% | 2.90% | 96% | 99% |

Table 2. Results of CE3.4.4.2: LM + LM-CbCr

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -4.11% | -22.66% | -21.30% | 118% | 101% | 0.46% | 1.20% | 1.53% | 86% | 97% |
| Class A2 | -1.64% | -9.46% | -4.87% | 119% | 102% | 0.13% | 0.64% | 1.14% | 86% | 98% |
| Class B | -0.60% | -7.34% | -11.08% | 119% | 102% | 0.17% | 1.03% | 0.60% | 87% | 98% |
| Class C | -1.36% | -9.34% | -12.19% | 118% | 103% | 0.10% | 0.72% | 0.84% | 88% | 99% |
| Class E | -0.41% | -4.16% | -6.49% | 119% | 102% | 0.09% | -0.40% | -0.43% | 87% | 99% |
| Overall | -1.50% | -10.16% | -11.23% | 119% | 102% | 0.18% | 0.69% | 0.73% | 87% | 98% |
| Class D | -0.92% | -7.44% | -8.96% | 118% | 102% | 0.12% | 0.55% | 0.34% | 89% | 99% |
| Class F (optional) | -2.48% | -8.45% | -9.58% | 119% | 104% | 0.64% | 4.35% | 4.51% | 89% | 98% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.47% | -19.45% | -21.49% | 103% | 99% | 0.64% | 1.43% | 2.85% | 95% | 98% |
| Class A2 | -0.59% | -9.48% | -3.81% | 101% | 100% | 0.09% | 0.91% | 1.03% | 96% | 100% |
| Class B | -0.26% | -9.69% | -13.09% | 102% | 99% | 0.12% | 1.45% | 0.71% | 96% | 101% |
| Class C | -0.51% | -8.93% | -11.03% | 102% | 100% | 0.13% | 0.64% | 0.27% | 96% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.84% | -11.39% | -12.36% | 102% | 100% | 0.22% | 1.12% | 1.08% | 96% | 100% |
| Class D | -0.32% | -7.43% | -8.57% | 103% | 101% | 0.13% | 0.30% | 0.20% | 96% | 100% |
| Class F (optional) | -1.33% | -7.25% | -7.53% | 104% | 100% | 0.34% | 3.51% | 3.27% | 95% | 100% |

Table 3. Results of CE3.4.4.3: LM + LM-fusion

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.78% | -24.86% | -20.79% | 118% | 101% | 0.78% | -0.79% | 1.71% | 86% | 98% |
| Class A2 | -1.28% | -10.06% | -5.16% | 120% | 104% | 0.47% | -0.46% | 0.55% | 87% | 100% |
| Class B | -0.30% | -8.15% | -11.20% | 118% | 104% | 0.51% | -0.01% | 0.00% | 87% | 100% |
| Class C | -1.27% | -9.52% | -12.17% | 117% | 104% | 0.22% | 0.25% | 0.40% | 88% | 100% |
| Class E | 0.18% | -5.83% | -7.40% | 119% | 104% | 0.72% | -2.18% | -1.53% | 87% | 105% |
| Overall | -1.18% | -11.17% | -11.37% | 118% | 103% | 0.52% | -0.52% | 0.21% | 87% | 101% |
| Class D | -0.90% | -7.58% | -8.98% | 116% | 104% | 0.12% | 0.41% | 0.28% | 89% | 100% |
| Class F (optional) | -2.43% | -8.52% | -9.28% | 119% | 105% | 0.68% | 3.79% | 4.01% | 89% | 99% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.31% | -20.97% | -20.77% | 104% | 99% | 0.79% | -0.43% | 2.81% | 96% | 99% |
| Class A2 | -0.39% | -10.01% | -4.01% | 102% | 101% | 0.28% | -0.12% | 0.41% | 97% | 100% |
| Class B | -0.16% | -10.21% | -12.79% | 102% | 99% | 0.23% | 0.45% | 0.23% | 96% | 100% |
| Class C | -0.50% | -8.99% | -10.86% | 102% | 100% | 0.14% | 0.27% | 0.20% | 96% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.73% | -12.00% | -12.12% | 102% | 99% | 0.33% | 0.11% | 0.77% | 96% | 100% |
| Class D | -0.37% | -7.17% | -8.52% | 102% | 101% | 0.11% | 0.16% | 0.30% | 96% | 101% |
| Class F (optional) | -1.33% | -7.49% | -7.14% | 104% | 100% | 0.33% | 3.33% | 3.66% | 96% | 100% |

Table 4. Results of CE3.4.4.4: LM + LM-left + LM-top + LM-CbCr + LM-fusion

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.84% | -26.09% | -22.75% | 136% | 101% | 0.75% | -2.54% | -0.45% | 98% | 99% |
| Class A2 | -1.39% | -11.08% | -5.86% | 136% | 103% | 0.38% | -1.60% | -0.17% | 98% | 101% |
| Class B | -0.28% | -9.19% | -12.93% | 135% | 103% | 0.55% | -1.09% | -1.70% | 97% | 100% |
| Class C | -1.28% | -10.44% | -13.26% | 134% | 104% | 0.24% | -0.83% | -0.86% | 98% | 100% |
| Class E | 0.24% | -6.13% | -7.86% | 137% | 104% | 0.80% | -2.50% | -1.96% | 98% | 102% |
| Overall | -1.19% | -12.09% | -12.62% | 136% | 103% | 0.53% | -1.59% | -1.09% | 98% | 101% |
| Class D | -0.90% | -8.30% | -9.79% | 132% | 104% | 0.21% | -0.61% | -0.74% | 97% | 100% |
| Class F (optional) | -2.58% | -9.62% | -10.66% | 136% | 104% | 0.56% | 2.73% | 2.80% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.35% | -22.27% | -22.57% | 106% | 99% | 0.74% | -1.78% | 0.57% | 100% | 98% |
| Class A2 | -0.45% | -10.91% | -4.64% | 104% | 100% | 0.27% | -1.53% | -0.18% | 100% | 101% |
| Class B | -0.15% | -11.56% | -14.89% | 105% | 99% | 0.27% | -0.88% | -1.58% | 100% | 100% |
| Class C | -0.49% | -10.06% | -12.03% | 105% | 101% | 0.21% | -0.99% | -1.30% | 100% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.74% | -13.17% | -13.61% | 105% | 100% | 0.35% | -1.22% | -0.79% | 100% | 100% |
| Class D | -0.29% | -8.40% | -9.57% | 105% | 102% | 0.12% | -0.90% | -1.23% | 100% | 100% |
| Class F (optional) | -1.42% | -8.56% | -8.49% | 108% | 101% | 0.27% | 2.03% | 2.22% | 100% | 100% |

Conclusion

This contribution presented four additional LM-based modes proposed in JVET-J0018, including LM-left, LM-top, LM-CbCr, and LM-fusion modes. The test results of CE3.4.4.1, CE3.4.4.2, CE3.4.4.3, and CE3.4.4.4 are also reported.

References

- J. Chen, Y. Chen, M. Karczewicz, X. Li, H. Liu, L. Zhang, X. Zhao, “Coding tools investigation for next generation video coding,” ITU-T SG16 Doc. COM16–C806, Feb. 2015.

- C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, H. Huang, S.-T. Hsiang, C.-C. Chen, M.-S. Chiang, C.-Y. Lai, C.-M. Tsai, Y.-C. Su, Z.-Y. Lin, Y.-L. Hsiao, J. Klopp, I.-H. Wang, Y.-W. Huang, S.-M. Lei, “Description of SDR video coding technology proposal by MediaTek,” JVET-J0018, April 2018.

- BMS-1.0: https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, April 2018.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).